3GPP TSG-SA WG6 Meeting #65	S6-250457

Athens, Greece, 17th – 21st February 2025	(revision of S6-250225)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0022 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to the Response of ML Model Information Storage and Discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Errors exist in the response of ML Model Information Storage and Discovery. In detail, either ML model profile list or ML model list shall be provided in the response of ML Model Information Discovery. The ML model retrieval endpoint need be provided in the response of ML Model Information Storage, instead of ML model address. This paper proposes to correct these errors. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Updated the response of ML Model Information Storage. - Updated the response of ML Model Information Discovery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors exist in the specification for the response of ML Model Information Storage and Discovery. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.11.4.2, 8.11.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.11.4.2	ML model information storage response

Table 8.11.4.2-1 describes the information flow from the ML repository to the AIMLE server or from the AIMLE server to the AIMLE client/VAL server as a response for the ML model information storage request.

Table 8.11.4.2-1: ML model information storage response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request. |
| ML model profile identifier | O | The identifier of the ML model profile created as a result of a successful ML model storage request. |

Table 8.11.4.2-2: ML model profile

| Information element | Status | Description |
| --- | --- | --- |
| ML model profile identifier | M | The identifier of the ML model profile. |
| AIMLE server identifier | M | The identifier of the AIMLE server that stored the ML model. |
| ML repository identifier | M | The identifier of the ML repository where the ML model is stored. |
| ML model information | M | The information about the ML model, as described in Table 8.11.4.1-2. |
| ML model retrieval endpoint | O | Represents the ML model retrieval endpoint (e.g., URL, URI, IP address and Port) that can be used to download the ML model. |

* * * Next Change * * * *

#### 8.11.4.4	ML model information discovery response

Table 8.11.4.4-1 describes the information flow from the model repository to the AIMLE server as a response for the ML model information discovery request.

Table 8.11.4.4-1: ML model information discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE 1) | Indicates that the request was successful. |
| > ML model profile list | O (NOTE 2) | Represents the ML model profile(s) of the discovered list of ML models, as described in Table 8.11.4.2-2. |
| >> ML model profile ID | M | Represents the ML model profile ID of the ML model profile. |
| > ML model list | O (NOTE 2) | Represents the ML model(s) of the discovered list of ML models. |
| >> ML model ID | M | Represents the ML model ID of the ML model. |
| > Indication of continuous model training | O | Indicate whether the model needs to be continuously trained or not. |
| Failure response | O (NOTE 1) | Indicates that the request failed. |
| > Cause | O | Indicates the failure cause. |
| NOTE 1: Only one of these information elements shall be provided. NOTE 2: Only one of these information elements shall be provided. |  |  |

* * * END of Changes * * * *