3GPP TSG-SA WG6 Meeting #65	S6-250409

Athens, Greece 17th – 21st February 2025	(revision of S6-250155)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0005 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove EN on access control rules |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_App |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Need to remove following EN: Editor's note: Details of Access control rules along with new terminology is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Renamed “access control rule” to better IE, provided more details in the descriptions |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN will not be removed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2.1, 7.3.3.1, 8.3.1.2, 8.3.1.3.1, 8.3.2.3.1, 8.3.4.2, 9.3.1.2, 9.3.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 7.3.2.1	Spatial anchor profile

Table 7.3.2.1-1 describes information elements for the spatial anchor profile stored in the SAn server.

Table 7.3.2.1-1: Spatial anchor profile

| Information element | Status | Description |
| --- | --- | --- |
| Spatial anchor ID | M | Identifier of the discovered spatial anchor. |
| Position (NOTE) | O | The 3D geographic position of the spatial anchor in space (x, y and z coordinates). |
| Spatial map position (NOTE) | O | Position information based on spatial map. It includes the identifier of the Spatial map and the corresponding localization information. |
| Spatial anchor specific information | O | A container to contain application specific information. |
| VAL service information | M | Information about the application service associated with the spatial anchor. |
| > VAL Service ID | M | The identifier of the application service. |
| Service area | O | The service area where the spatial anchor is accessible. |
| Allowed entities list | O | Identity of entities (e.g. VAL client, VAL server or VAL service identity) which are permitted to discover the spatial anchor. |
| NOTE: One of the IEs shall be present. |  |  |

* * * Next Change * * * *

#### 7.3.3.1	Spatial map information

Table 7.3.3.1-1 describes information elements for the spatial map information stored in the SM server.

Table 7.3.3.1-1: Spatial map information

| Information element | Status | Description |
| --- | --- | --- |
| Spatial map ID | M | The identifier of the spatial map |
| Spatial map coverage | M | The three-dimensional space coordinates information of the spatial map |
| Allowed entities list | O | Identity of entities (e.g. VAL client, VAL server or VAL service identity) which are permitted to discover and access the spatial map |
| List of spatial map layers | M | The list of spatial map layer information. At least one or more spatial map layers should be included. Each element includes the information described below. |
| > Spatial map layer ID | M | The identifier of the corresponding spatial map layer. |
| > Spatial map layer information | O | A container to carry the corresponding spatial map layer specific information |
| >> area | O | The area covered by this layer. This area can be the same or a subset of the Spatial map coverage space. |
| >> associated spatial map | O | The details about a spatial map associated with this layer. For example, if the layer is for a particular floor of the building, the detailed spatial map of that particular floor can be associated with this layer. |
| > List of objects | O | List of object information. Each element includes the information below. |
| >> Object ID | M | The identifier of the corresponding object |
| >> Object type | M | The type of object |
| >> Object position | O | The 3D position of the object in the layer |
| >> Object specific information | O | Object specific attribute information |

NOTE:	For the current release, the object specific information is implementation specific and out of scope.

* * * Next Change * * * *

#### 8.3.1.2	Procedure

Figure 8.3.1.2-1 illustrates the spatial anchor creation procedure.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server;

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

3.	For SAn client to trigger this procedure, the SAn client has received the required information from the VAL client over SAn-C interface.

Figure 8.3.1.2-1: Spatial anchor creation

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor create request to the SAn server. The request includes a requestor identifier, security credentials, and list of spatial anchors to be created as described in clause 8.3.1.3.1. The request includes a position, allowed entities list defining which entities are permitted to discover and access the spatial anchor, and discoverability information (e.g., indicating whether the spatial anchor is discoverable by any user). If the request includes spatial anchor specific information, that information remains transparent to the SAn server and can be stored with the spatial anchor. The request can indicate to create multiple spatial anchors.

NOTE:	The spatial anchor specific information is out of scope of standardization. For example, the spatial anchor specific information may contain application specific service information like a name, a URL, a media data, a type, a manufacturing company, a date, an expiry date, a ratings, and customer premise information (e.g. a residence, office or shop), a service schedule defining times when the spatial anchor(s) are active and inactive, etc. Such information may be stored in key-value pair and is transparent to the SAn server.

2.	Upon receiving the request, the SAn server validates if the requestor is authorized for the request. If the requestor is authorized, the SAn server creates the spatial anchor(s), assigns spatial anchor identifier to the newly created spatial anchor(s) and stores the spatial anchor information.

3.	The SAn server sends a spatial anchor create response to the requestor. If the SAn server created spatial anchors, the response includes an indication of success and spatial anchor identifier(s) of newly created spatial anchor(s). Otherwise, the response includes an indication of failure and can include a reason for failure.

* * * Next Change * * * *

##### 8.3.1.3.1	Spatial anchor create request

Table 8.3.1.3.1-1 describes information elements for the spatial anchor create request from the SAn client or VAL server to the SAn server.

Table 8.3.1.3.1-1: Spatial anchor create request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| VAL service information | M | Information about the application service associated with the spatial anchor. |
| > Service ID | M | The identifier of the application service. |
| > Service endpoint | M | Endpoint information (e.g., URI, FQDN, IP address) used to communicate with the application service. |
| > Service connection information | O | The connection information (e.g., DNN, DNAI(s)) for establishing connectivity to the application service. |
| List of spatial anchors | M | List of spatial anchors to be created. |
| > Position information | M | Position information of the spatial anchor |
| >> Position (NOTE) | O | The 3D geographic position of the spatial anchor in space (x, y and z coordinates). |
| >> Spatial map position (NOTE) | O | Position information of the spatial anchor within the spatial map. It includes the identifier of the Spatial map, identifier of spatial map layer(s) and the corresponding localization information (The localization information is specific to spatial map format). |
| > Spatial anchor specific information | O | A container to contain application specific information. |
| > Service area | O | The service area where the spatial anchor is accessible. |
| > Allowed entities list | O | Identity of entities (e.g. VAL client, VAL server or VAL service identity) which are permitted to discover the spatial anchor. |
| NOTE: One of the IEs shall be present. |  |  |

* * * Next Change * * * *

##### 8.3.2.3.1	Spatial anchor update request

Table 8.3.2.3.1-1 describes information elements for the spatial anchor update request from the SAn client or VAL server to the SAn server.

Table 8.3.2.3.1-1: Spatial anchor update request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| > VAL service information | O | Information about the application service associated with the spatial anchor. |
| >> Service ID | O | The identifier of the application service. |
| >> Service endpoint | O | Endpoint information (e.g., URI, FQDN, IP address) used to communicate with the application service. |
| >> Service connection information | O | The connection information (e.g., DNN, DNAI(s)) for establishing connectivity to the application service. |
| List of spatial anchors | M | List of spatial anchors to be updated. |
| > Spatial anchor ID | M | Identifier of the spatial anchor to be updated. |
| > Position information | O | Position information of the spatial anchor |
| >> Position (NOTE) | O | The 3D geographic position of the spatial anchor in space (x, y and z coordinates). |
| >> Spatial map position (NOTE) | O | Position information of the spatial anchor within the spatial map. It includes the identifier of the Spatial map, identifier of spatial map layer(s) and the corresponding localization information (The localization information is specific to spatial map format). |
| > Spatial anchor specific information | O | A container to contain application specific information. |
| > Service area | O | The service area where the spatial anchor is accessible. |
| > Allowed entities list | O | Identity of entities (e.g. VAL client, VAL server or VAL service identity) which are permitted to discover the spatial anchor. |
| NOTE: If Position information is included, at least one of the IEs shall be present. |  |  |

* * * Next Change * * * *

#### 8.3.4.2	Procedure

Figure 8.3.4.2-1 illustrates the procedure for spatial anchor discovery following the request-response model.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server; and

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

3.	For SAn client to trigger this procedure, the SAn client has received the required information from the VAL client over SAn-C interface.

Figure 8.3.4.2-1: Spatial anchor discovery

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor discovery request message to the SAn server to obtain information about spatial anchors of interest. The request includes a requestor identifier, security credentials, and can include spatial anchor discovery filters as described in clause 8.3.4.3.1. If the discovery request was triggered by a VAL client on a UE, the spatial anchor discovery request can additionally include a VAL client identifier.

2.	Upon receiving the request, the SAn server validates if the requestor is authorized for the request. If the requestor is authorized, the SAn server identifies spatial anchor(s) that satisfies the spatial anchor discovery filters.

NOTE 1:	User preferences in the spatial anchor discovery filters can be compared with spatial anchor specific information.

For each matching spatial anchor, the SAn server checks whether the requestor is authorized to obtain the spatial anchor based on the identity and location of the requestor, and based on the allowed entities list. To determine the location of interest of the requestor, the SAn server may use location information provided in the request. If location information is not provided in the request, the SAn server may obtain location information by invoking the 3GPP Core Network Location Services as described in 3GPP TS 23.273 [6] and 3GPP TS 23.502 [5] directly or via the NEF, or by invoking the SEAL Location Management APIs as described in 3GPP TS 23.434 [4].

3.	The SAn server sends a spatial anchor discovery response to the requestor. If the SAn server has identified spatial anchor(s), the response includes an indication of success and the identified spatial anchors and may includes the recommended list of spatial anchors. Otherwise, the response includes an indication of failure and may include a reason for failure.

NOTE 2:	The logic to generate a recommended list of spatial anchors is out of scope and implementation specific.

4.	Upon receiving the spatial anchor discovery response, if the response indicates success, the requestor can store the spatial anchor information in a spatial anchor cache. The requestor evaluates the validity conditions associated with the discovered spatial anchors prior to using the spatial anchor. If the requestor is the SAn client, the SAn client can provide spatial anchor information to the VAL client(s). Upon receiving valid spatial anchor information, VAL client(s) can access the service(s) associated with each spatial anchor.

* * * Next Change * * * *

#### 9.3.1.2	Procedure

Figure 9.3.1.2-1 depicts the procedure for creating a spatial map. For the request from VAL server or SEAL SM client, as a spatial map consumer, SEAL SM server creates a spatial map. A spatial map can be structured in layers where each layer signifies a specific aspect of the spatial information (e.g. three dimensional space of area of interest, objects).

Figure 9.3.1.2-1: Create spatial map procedure

1)	The requestor (e.g., VAL server or SEAL SM client) sends a request message to the SEAL SM server to create a spatial map. The request includes requestor ID, security credentials, three-dimensional area of interest, information to be included in the spatial map such as allowed entities list defining which entities are permitted to discover and access the spatial map. The request may include spatial map layering information parameters and may also include augmented layer information that can be requested with the spatial map.

NOTE:	SEAL SM server can obtain augmented layer information and can provide the obtained augmented layer information to the requestor in the response message.

2)	The SEAL SM server authorizes the requestor, and validates the request. The SEAL SM server produces a requested spatial map using layering information and processed sensor data.

a)	If the augmented layer information indicates to include VAL UE information, the SEAL SM server fetches the list of VAL UEs in the area of interest from LM server using clause 9.3.10 of 3GPP TS 23.434 [4] and/or from NEF as specified in 3GPP TS 23.502 [5].

b)	If the augmented layer information indicates to include spatial anchors information within area of interest, the SEAL SM server fetches the list of spatial anchors in the area of interest as specified in clause 8.3.4.

NOTE:	For the current release, the processed sensor data is stored at VAL layer (e.g., in database at VAL layer) and the SEAL SM server gets access to such database to create a spatial map via application specific way which is out of scope.

3)	The SEAL SM server sends response message to the requestor with a indication of success, in-progress or fail. If the requested spatial map has been created sussesfully, the response message includes assigned spatial map ID and information which includes three-dimensional space defined by the spatial map and can include a list of spatial map layers with their corresponding layer ID, objects belong to the layer, etc.. If the response indicates "in-progress", an assigned spatial map ID is included in the message. Further, if the response indicates "in-progress" and the requestor has not provided a notification target address, then the requestor subscribes for spatial map event to receive notification when the spatial map is created. Otherwise, the response includes an indication of failure and may include a reason for the failure.

4)	If the response indicates "in-progress", the SM server detects when the SEAL SM server has sufficient information to map all the objects related to area of interest and the spatial map is ready to provide service (e.g. employed for localization). If the requestor has provided a notification target address in the request or has subscribed for spatial map events, the SEAL SM server notifies the requestor with the spatial map ready notification message. The notification includes parameters as specifed in Table 9.3.6.3.3-1.

* * * Next Change * * * *

##### 9.3.1.3.1	Create spatial map request

Table 9.3.1.3.1-1 descibes the information elements from VAL server (or SEAL SM client) to SM server for create spatial map request.

Table 9.3.1.3.1-1: Create spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Application service ID | M | ID of the requesting VAL application service |
| Area of interest | M | Three-dimensional area information to produce the spatial map |
| Notification Target Address | O | Notification target address (e.g. URL, URI, IP) where the notifications should be sent. |
| Information to include | O | Information to be included in the spatial map |
| > Allowed entities list | O | Identity of entities (e.g. VAL client, VAL server or VAL service identity) which are permitted to discover and access the spatial map |
| > List of spatial map layers | O | List of spatial map layer information. Each element includes the information described below. |
| >> Spatial map layer information | O | Information to specify the corresponding spatial map layer specific information |
| > Augmented layer information | O | List of required augmented layer information. This IE can contain multiple values. Possible values are: - VAL_USERS; - SPATIAL_ANCHORS. |

NOTE 1:	For the current release, the spatial map layer information is implementation specific and out of scope.

* * * End of Change * * * *