3GPP TSG SA Meeting #107								SP-250393

Incheon, KR, 12-14 March 2025

Agenda Item:		21.5

Source:			Apple Inc. (Rapporteur)

Title:				Summary for WI “Study on new HEVC profiles and operating points”

WI code(s):			FS_HEVC_Profiles

leading WG:		SA4

Release:			Rel-18

This document provides a summary of the WI “Study on new HEVC profiles and operating points” (FS_HEVC_Profiles) for TR 21.918.

| Unique ID | Name | Acronym | WG | WID | WI Rapporteur name |
| --- | --- | --- | --- | --- | --- |
| 1000017 | Study on new HEVC profiles and operating points | FS_HEVC_Profiles | S4 | SP-230540 | Waqar Zia, Apple Inc. |

# Summary

This Rel-18 SA WI "Study on new HEVC profiles and operating points" has produced 3GPP TR 26.966 [1] which gathers the opportunities for improving HEVC-based services. This TR includes documentation of motivating use cases and scenarios. Specifically, potential of improving on the following use cases are identified: the compression performance for stereoscopic 3D content, the network performance related to exploding adaptive streaming traffic, and the demands for very high-quality image applications. HEVC based solutions to address each opportunity are identified: HEVC Multiview profiles, HEVC Scalable profiles, and HEVC 4:4:4 (up to 10 bits) capable profiles. Methodologies to investigate and document the pros and cons of the proposed solutions for each use case are documented. A summary of conclusions drawn on the relevancy of solutions and on new normative specification work to be done is in the following.

MV-HEVC and HEVC frame packing are suitable solutions for addressing the scenarios of streaming of stereoscopic 3D content and for low delay applications of stereoscopic 3D video, where MV-HEVC represents a more versatile tool. With HEVC simulcast and HEVC frame packing already included in SA4 specifications, and given the coding benefits it provides compared to alternative solutions, it was recommended to add support for stereoscopic MV-HEVC to the related SA4 specifications.

To address the scenario of high-quality photography, it was concluded that native and derived 4:4:4 coding can achieve better visual quality than the baseline solution of HEVC 4:2:0 coding. Derived 4:4:4 coding however can achieve this improvement by reusing existing hardware support, without a need for a specialised hardware. However, a higher level may be needed.

On the scenario for optimising multi-bitrate delivery, scalable HEVC coding shows improvement potential for enhancing the adaptive streaming experience by allowing more switchable representations to be made available, while optimising storage overhead for this purpose. Scalable HEVC is also supported by MPEG specifications such as CMAF. The need to do normative work was left to industry interest in this direction.

On the scenario for pose correction optimisation, MV-HEVC with depth/alpha channels shows feasibility of the combination of MV-HEVC with depth and/or alpha channels, both in terms of additional data rate and complexity. The support and usage of depth and alpha channels in the industry is relevant for various application, potentially including closed caption insertion and pose-correction. Based on such usage cases and their potential benefits, it may be desirable to add support for carriage of these channels in 3GPP specifications.

# 2	References

- 3GPP TR 26.966, Evaluation of new HEVC coding tools