3GPP TSG SA Meeting #107								SP-250398

Incheon, KR, 12-14 March 2025

Agenda Item:		21.5

Source:			Qualcomm Incorporated, Dolby Laboratories, Tencent, BBC

Title:				Summary for WI "5G Media Streaming Architecture Phase 2 (5GMS_Ph2)"

WI code(s):			960047

leading WG:		SA4

Release:			Rel-18

This document provides a summary of the Work Item Summary for WI "5G Media Streaming Architecture Phase 2 (5GMS_Ph2)".

| Unique ID | Name | Acronym | WG | WID | WI Rapporteur name |
| --- | --- | --- | --- | --- | --- |
| 960047 | 5G Media Streaming Architecture Phase 2 | 5GMS_Ph2 | S4 | SP-230164 | Iraj Sodagar, Tencent |

# Summary

TS 26.501 [1] defines the 5GMS architecture, call flows, and procedures, originally specified in Rel-16. Based on two feasibility studies "Study on 5G media streaming extensions (FS_5GMS_EXT)" and " Feasibility Study on Multicast Architecture Enhancements for 5G Media Streaming (FS_5GMS_Multicast)", several potential improvement areas were identified to initiate normative work in the context of 5GMS. The work item addressed the stage-2 aspects 5G Media Streaming Architecture Phase 2 based on the conclusions in TR 26.802 (for MBS) and in TR 26.804 (for 5GMS):

- 	Uplink streaming:  While introduced in principle in Rel-16, uplink streaming was severely underdefined and lacked the essential features needed for mainstream deployments. Features initially only available to downlink streaming are generalized to also be usable for uplink stream and additional features are added to support different collaboration scenarios. Details of uplink streaming are defined in clause 5 of TS 26.501.

-	5GMS content delivery via 5G System and MBS: The MBS delivery feature is applicable to downlink media streaming only. It enables the 5GMS System to provision the delivery of downlink media streaming content via MBS User Services sessions. In addition, the clause defined details to support for the case that a 5GMS service is available on MBS and at the same time on unicast. The service on unicast may be richer and extended and may provide additional user experiences. This feature is defined in clause 4.9 and high-level procedures are defined in clause 5.12 of TS 26.501.

-	Hybrid DASH/HLS operation: This feature considers the case that a 5GMSd Application Provider publishes the same service in different media formats, all to be ingested by the same logical 5GMSd AS. The different media formats of the service are described by different presentation manifests (e.g. Dynamic Adaptive Streaming over HTTP and HTTP Live Streaming) but are described by a common Content Hosting Configuration under a single Provisioning Session. The different media formats may or may not share the media resources described by their respective presentation manifests.

-	Extensions to Dynamic Policies to support service operation points: A conceptual Service Operation Point is an abstract set of requirements that support a media streaming service (e.g., SD, HD, UHD, low latency). It is identified by an External reference that is used to tag Policy Template resources provisioned in the 5GMS System and Service Descriptions included in Media Entry Point documents. If there is a Policy Template available for the current media streaming session with the indicated External reference, the 5GMS Client instantiates this Policy Template by interacting with a network-side component of the 5GMS System in order to realise the Service Operation Point described by the Policy Template and the Service Description. The effect of this is that the corresponding network Quality of Service is applied to the media streaming session.

-	5GMS AS configuration: Procedures on interface M3 are specified. This allows the 5GMSd AF to configure the related 5GMSd AS instance(s) via reference point M3d to prepare for media ingest for that particular Content Hosting Configuration. As well as configuring any necessary Server Certificates and/or Content Preparation Templates, this also involves instructing the 5GMSd AS instance(s) to set appropriate caching rules, to perform URL signature validation and to limit access through geofencing.

-	Improvements to Data collection: The data collection, reporting and exposure feature is applicable to both downlink media streaming and uplink media streaming. It enables the 5GMS System to log data relating to media streaming sessions and to expose this to subscribers in the form of Events. In TS 26.501, this feature is defined in clause 4.7 and high-level procedures are defined in clause 5.11 (for downlink media streaming) and clause 6.8 (for uplink media streaming).

-	3GPP Service URL: Service URL handling is applicable to downlink and uplink media streaming. This feature is defined in clause 4.10 and high-level procedures are defined in clause 9 of TS 26.501. The intent of 3GPP Service URL handling is to launch UE functions based on the execution of a URL. This enables 5G Media Streaming services to be announced within a third-party application, a general web page, a messaging service or shared via social messages using a 3GPP Service URL for 5GMS. When a service is launched using a 3GPP Service URL for 5GMS, a 5GMS Client function for media session handling is expected to be launched implicitly alongside, for example, a primary media stream handling function

-	Background Data Traffic: A new feature is added such that the Policy Template may include a reference to an existing Background Data Transfer policy. Parameters are specified that include desired time windows when Background Data Transfer may be advertised to 5GMS Clients, a quota representing the maximum number of 5GMS Clients that are permitted to take advantage of Background Data Transfers in each such time window and a quota representing a ceiling for the aggregate volume of data that all 5GMS Clients are permitted to transfer in each Background Data Transfer window. The procedures for downlink background data transfer using dynamic policy invocation are specified.

The work item and the respective CRs address the updates recommended above.

# 2	References

- 3GPP TS 26.501, 5G Media Streaming (5GMS); General description and architecture