# 3GPP TSG-SA Meeting #107	SP-250412

Incheon, South Korea, Mar 12 – 14, 2025	Revision of SP-250357

# Source:	Nokia, vivo

# Title:	pCR TR 22.850 Data Collection and management for AI/ML analysis across 3GPP

# Document for:	Approval

# Agenda Item:	7

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[8]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[9]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management".

[15]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[33]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service".

# 3	Rationale

This pCR aims to capture data collection for AI/ML aspects from Release 18 across 3GPP. It seeks to identify inconsistencies and misalignments among them.

# 4	Detailed proposal

Start of modification

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 21.918: "Summary of Rel-18 Work Items".

[3]	3GPP TR 38.843: "Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)".

[4]	3GPP TR 23.700-84: "Study on Core Network Enhanced Support for Artificial Intelligence (AI) / Machine Learning (ML)".

[5]	3GPP TR 22.874: "5G System (5GS); Study on traffic characteristics and performance requirements for AI/ML model transfer".

[6]	3GPP TS 22.261: "Service requirements for the 5G system".

[7]	3GPP TR 23.700-82: "Study on application layer support for AI/ML services".

[8]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[9]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management".

[10]	3GPP TR 38.843: "Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface".

[11]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[12]	3GPP TR 26.927: "Study on Artificial Intelligence and Machine learning in 5G media services".

[13]	3GPP TS 38.401: "NG-RAN; Architecture description".

[14]	3GPP TS 38.420: "NG-RAN; Xn general aspects and principles".

[15]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[16]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[17]	3GPP TR 37.817: "Study on enhancement for data collection for NR and ENDC".

[18]	3GPP TR 28.908: "Study on Artificial Intelligence/Machine Learning (AI/ ML) management".

[19]	3GPP TR 28.858: "Study on Artificial Intelligence / Machine Learning (AI/ML) management; Phase 2".

[20]	3GPP TR 29.889: "Study on Protocol for AI Data Collection from UPF".

[21]	3GPP TR 22.876: "Study on AI/ML Model Transfer; Phase 2".

[22]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[23]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[24]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[25]	3GPP TR 33.784: "Study on security aspects of core network enhanced support for Artificial Intelligence Machine Learning (AIML)".

[26]	3GPP TR 26.847: "Evaluation of Artificial Intelligence and Machine learning in 5G media services".

[27]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[28]	3GPP TS 32.425: "Telecommunication management; Performance Management (PM); Performance measurements Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[29]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[30]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[31]	3GPP TS 32.254: "Telecommunication management; Charging management; Exposure function Northbound Application Program Interfaces (APIs) charging".

[32]	3GPP TS 32.291: "Telecommunication management; Charging management; 5G system, charging service; Stage 3".

[33]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service".

[34]	3GPP TS 23.482: "Functional architecture and information flows for AIML Enablement Service".

[35]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[36]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[37]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[38]	3GPP TS 29.513: "5G System; Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3".

[39]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[40]	3GPP TS 29.517: "5G System; Application Function Event Exposure Service; Stage 3".

[41]	3GPP TS 29.591: "5G System; Network Exposure Function Southbound Services; Stage 3".

[42]	3GPP TS 29.519: "5G System; Usage of the Unified Data Repository Service for Policy Data, Application Data and Structured Data for Exposure; Stage 3".

[43]	3GPP TS 29.520: "5G System; Network Data Analytics Services; Stage 3".

[44]	3GPP TS 29.521: "5G System; Binding Support Management Service; Stage 3".

[45]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[46]	3GPP TS 29.523: "5G System; Policy Control Event Exposure Service; Stage 3".

[47]	3GPP TS 29.525: "5G System; UE Policy Control Service; Stage 3".

[48]	3GPP TS 29.508: "5G System; Session Management Event Exposure Service; Stage 3".

[49]	3GPP TS 29.551: "5G System; Packet Flow Description Management Service; Stage 3".

[50]	3GPP TS 29.552: "5G System; Network Data Analytics signalling flows; Stage 3".

[51]	3GPP TS 29.574: "5G System; Data Collection Coordination Services; Stage 3".

[52]	3GPP TS 29.575: "5G System; Analytics Data Repository Services; Stage 3".

[53]	3GPP TS 29.576: "5G System; Messaging Framework Adaptor Services; Stage 3".

[54]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[55]	3GPP TS 29.504: "5G System; Unified Data Repository Services; Stage 3".

[56]	3GPP TS 29.505: "5G System; Usage of the Unified Data Repository services for Subscription Data; Stage 3".

[57]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3".

[58]	3GPP TS 29.564: "5G System; User Plane Function Services; Stage 3".

[59]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[60]	3GPP TS 29.536: "5G System; Network Slice Admission Control Services; Stage 3".

[61]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[62]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[63]	3GPP TS 38.215: "NR; Physical layer measurements".

[64]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[65]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[66]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[67]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[68]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[69]	3GPP TR 38.743: "Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN".

[70]	3GPP TR 38.744: "Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR".

[y]	3GPP TS 28.622: “Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)”

Start of next change

# 6	Analysis on AI/ML across 3GPP

## 6.3	AI/ML related features

### 6.3.X	Analysis on data collection and management for AI/ML

The analysis focuses on the specifications from SA WG2, SA WG6 and RAN WG3, considering these are the working groups defining services and operations related to data collection for AI/ML in 3GPP Release 18. SA WG1, SA WG3, SA WG4, SA WG5, RAN WG1 and RAN WG2 have not defined any services or operations related to data collection for AI/ML in Release 18. SA WG5 specifies data collection and performance measurement services that can be leveraged for AI/ML purposes (see 3GPP TS 28.622 [y]).

Table 6.3.x-1 provides a detailed overview of the specific services defined by each working group.

The key findings from the analysis are as follows:

- SAWG2: Defines multiple network functions capable of producing data collection services and defines a function for data storage related services. Leverages event exposure framework and defines event exposure services for network functions that can be consumed by NWDAF (see 3GPP TS 23.288 [8] clause 6.2.2.1). Defines data collection for AI/ML services through a clear consumer-producer relationship.

Editor’s note: 	Description for DCCF and data collection coordination is FFS.

- SA WG6: Defines network functions similar to those in SA WG2. Data collection services defined for A-DCCF are generic while data collection and data storage services defined for A-ADRF are a mix of generic and use case specific.

Editors’ note:		More analysis is needed for Rel-19 A-DCCF part.

- RAN WG3: Defines data collection messages exchanged between two gNBs over the Xn interface, in a P2P manner. It is to be noted that procedures used for AI/ML support in the NG-RAN shall be “data type agnostic”, which means that the intended use of the data (e.g., input, output, feedback) shall not be indicated.

Editors’ note:		This analysis is based on Release 18 and does not consider Release 19. Further analysis needs to be conducted as Release 19 matures and normative work progresses.

While SA WG2 and SA WG6 both define data collection services, their approaches to data storage and retrieval are different. SA WG2 defines generic data storage and retrieval services that can be supported by an entity (ADRF) and requested by another entity but SA WG6 defines both generic and individual services (related to each analytics type) for storing and retrieving data. RAN WG3 operates independently and is unrelated to services defined in SA WGs and therefore can coexist.

Table 6.3.x-1: Data Collection for AI/ML related services and operations as specified across 3GPP WGs

| Data Collection for AI/ML |  |  |  |
| --- | --- | --- | --- |
| TSG (TS/TR) | Service/API/Message Type | Service/API/IOC/Message Name | Description [Consumer, Producer] |
| SA WG2 TS 23.288 [8] | Event Exposure services | Namf_EventExposure_Subscribe | The NWDAF uses this service operation to subscribe to or modify event reporting for one UE, a group of UE(s) or any UE. Producer: AMF |
|  |  | Namf_EventExposure_Unsubscribe | The NWDAF uses this service operation to unsubscribe for a specific event for one UE, group of UE(s), any UE. Producer: AMF |
|  |  | Namf_EventExposure_Notify | Provides the previously subscribed event information to the NWDAF which has subscribed to that event before. Producer: AMF |
|  |  | Nsmf_EventExposure_Subscribe | This service operation is used by an NWDAF to subscribe or modify a subscription for event notifications on a specified PDU Session or for all PDU Sessions of one UE, group of UE(s) or any UE. Producer: SMF |
|  |  | Nsmf_EventExposure_UnSubscribe | This service operation is used by an NWDAF to unsubscribe event notifications. Producer: SMF |
|  |  | Nsmf_EventExposure_Notify | Report UE PDU Session related event(s) to the NWDAF which has subscribed to the event report service. Producer: SMF |
|  |  | Npcf_EventExposure_Subscribe | The NWDAF uses this service operation to subscribe to or modify event reporting for a group of UE(s) or any UE accessing a combination of (DNN, S-NSSAI). Producer: PCF |
|  |  | Npcf_EventExposure_Unsubscribe | The NWDAF uses this service operation to unsubscribe for a specific event for a group of UE(s) or any UE accessing a combination of (DNN, S-NSSAI). Producer: PCF |
|  |  | Npcf_EventExposure_Notify | This service operation reports the event to the NWDAF that has previously subscribed either using Npcf_EventExposure_Subscribe service operation or provided as part of the Data Set Application Data and Data Subset Service Parameters stored in UDR. Producer: PCF |
|  |  | Nudm_EventExposure_Subscribe | The NWDAF subscribes to receive an event. Producer: UDM |
|  |  | Nudm_EventExposure_Unsubscribe | The NWDAF deletes the subscription of an event if already defined in UDM. Producer: UDM |
|  |  | Nudm_EventExposure_Notify | UDM reports the event to the NWDAF that has previously subscribed. Producer: UDM |
|  |  | Nudm_EventExposure_ModifySubscription | The NWDAF requests to modify an existing subscription to event notifications. Producer: UDM |
|  |  | Nnef_EventExposure_Subscribe | The NWDAF subscribes to receive an event, or if the event is already defined in NEF, then the subscription is updated. Producer: NEF |
|  |  | Nnef_EventExposure_Unsubscribe | The NWDAF deletes an event if already defined in NEF. Producer: NEF |
|  |  | Nnef_EventExposure_Notify | NEF reports the event to the NWDAF that has previously subscribed. Producer: NEF |
|  |  | Naf_EventExposure_Subscribe | The NWDAF subscribes the event to collect AF data for UE(s), group of UEs, or any UE, or updates the subscription which is already defined in AF. Producer: AF |
|  |  | Naf_EventExposure_Unsubscribe | The NWDAF unsubscribes for a specific event. Producer: AF |
|  |  | Naf_EventExposure_Notify | The AF provides the previously subscribed event information to the NWDAF which has subscribed to that event before. Producer: AF |
|  |  | Nnsacf_SliceEventExposure_Subscribe | This service operation is used by the NWDAF to subscribe or modify a subscription with the NSACF for event based notifications of the current number of UEs registered for a network slice or the current number of PDU Sessions established on a network slice. Producer: NSACF |
|  |  | Nnsacf_SliceEventExposure_Unsubscribe | This service operation is used by the NWDAF to unsubscribe from the event notification. Producer: NSACF |
|  |  | Nnsacf_SliceEventExposure_Notify | This service operation is used by the NSACF to report the current number of UEs registered with a network slice or the current number of PDU Sessions established on a network slice in numbers or in percentage from the maximum allowed numbers, based on threshold or at expiry of periodic timer. Producer: NSACF |
|  |  | Nupf_EventExposure_Subscribe | This service operation reports the event and information to the NWDAF that has subscribed implicitly. Producer: UPF |
|  |  | Nupf_EventExposure_Unsubscribe | This service operation is used by an NWDAF to subscribe or modify a subscription to UPF event exposure notifications e.g. for the purpose of UPF data collection on a specified PDU Session or for all PDU Sessions of one UE or any UE. Producer: UPF |
|  |  | Nupf_EventExposure_Notify | The NF consumer uses this service operation to unsubscribe for a specific event. Consumer: Any NF Producer: UPF |
|  |  | Nscp_EventExposure_Notify | The NWDAF uses this service operation to unsubscribe for a specific event. Producer: SCP |
|  |  | Nscp_EventExposure_Subscribe | This service operation is used by an NWDAF to subscribe or modify a subscription to SCP event exposure notifications. Producer: SCP |
|  |  | Nscp_EventExposure_Unsubscribe | The NWDAF uses this service operation to unsubscribe from an existing subscription. Producer: SCP |
|  | NWDAF Data Management services | Nnwdaf_DataManagement_Subscribe | The consumer subscribes to data exposed by an NWDAF. It can be historical data or runtime data. The subscription includes service operation specific parameters that identify the data to be provided and may include formatting and processing instructions that specify how the data is to be delivered to the consumer. Consumer: NWDAF, DCCF Producer: NWDAF |
|  |  | Nnwdaf_DataManagement_Unsubscribe | The consumer unsubscribes to the data exposed by an NWDAF. Consumer: NWDAF, DCCF Producer: NWDAF |
|  |  | Nnwdaf_DataManagement_Notify | The NWDAF notifies the consumer of the requested data or notifies of the availability of previously subscribed data when delivery is via an NWDAF. The NWDAF may also notify the consumer when Data or Analytics is to be deleted. Consumer: NWDAF, DCCF, MFAF, ADRF Producer: NWDAF |
|  |  | Nnwdaf_DataManagement_Fetch | The consumer retrieves from the NWDAF subscribed data, as indicated by Fetch Instructions from Nnwdaf_DataManagement_Notify. Consumer: NWDAF, DCCF, MFAF, ADRF Producer: NWDAF |
|  | NWDAF Roaming Data services | Nnwdaf_RoamingData_Subscribe | The consumer subscribes for input data related to roaming UE(s) for NWDAF analytics. The subscription includes service operation specific parameters that identify the data to be provided and may include formatting and processing instructions that specify how the data is to be delivered to the consumer. Consumer: H-RE-NWDAF, V-RE-NWDAF Producer: H-RE-NWDAF, V-RE-NWDAF |
|  |  | Nnwdaf_RoamingData_Unsubscribe | The consumer unsubscribes to input data related to roaming UE(s). Consumer: H-RE-NWDAF, V-RE-NWDAF Producer: H-RE-NWDAF, V-RE-NWDAF |
|  |  | Nnwdaf_RoamingData_Notify | NWDAF notifies the consumer about input data related to roaming UE(s) that the consumer has subscribed to. Consumer: H-RE-NWDAF, V-RE-NWDAF Producer: H-RE-NWDAF, V-RE-NWDAF |
|  | DCCF Data Management Services | Ndccf_DataManagement_Subscribe | The consumer subscribes to receive data or analytics from the DCCF. The subscription includes service operation specific parameters that identify the data or analytics to be provided and may include formatting and processing instructions that specify how the data is to be delivered to the consumer. The consumer may also request that data be stored in an ADRF or an NWDAF hosting ADRF functionality. Consumer: NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF Producer: DCCF |
|  |  | Ndccf_DataManagement_Unsubscribe | The consumer unsubscribes to DCCF for data or analytics. Consumer: NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF Producer: DCCF |
|  |  | Ndccf_DataManagement_Notify | DCCF notifies the consumer instance of the requested data or analytics according to the request or notifies of the availability of previously subscribed Data or Analytics when data delivery is via the DCCF. The DCCF may also notify the consumer instance when Data or Analytics is to be deleted. Consumer: NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF Producer: DCCF |
|  |  | Ndccf_DataManagement_Fetch | The consumer retrieves from the DCCF, data or analytics as indicated by Ndccf_DataManagement_Notify Fetch Instructions. Consumer: NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF Producer: DCCF |
|  |  | Ndccf_DataManagement_Transfer | The Source DCCF transfers UE data subscription context to the target DCCF. Consumer: DCCF Producer: DCCF |
|  | MFAF Data Management Services | Nmfaf_3daDataManagement_Configure | The consumer configures or reconfigures the MFAF to map data or analytics received by the MFAF to out-bound notification endpoints and to format and process the out-bound data or analytics. Consumer: DCCF, NWDAF Producer: MFAF |
|  |  | Nmfaf_3daDataManagement_Deconfigure | The consumer configures the MFAF to stop mapping data or analytics received by the MFAF to one or more out-bound notification endpoints. Consumer: DCCF, NWDAF Producer: MFAF |
|  |  | Nmfaf_3caDataManagement_Notify | MFAF provides data or analytics or notification of availability of data or analytics to notification endpoints. Consumer: NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF Producer: MFAF |
|  |  | Nmfaf_3caDataManagement_Fetch | The consumer retrieves from the MFAF, data or analytics as indicated by Nmfaf_3caDataManagement_Notify Fetch Instructions. Consumer: NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF Producer: MFAF |
|  | ADRF Data Management Services | Nadrf_DataManagement_StorageRequest | The consumer NF uses this service operation to request the ADRF to store data or analytics. Data or analytics are provided to the ADRF in the request message. Consumer: DCCF, NWDAF, MFAF Producer: ADRF |
|  |  | Nadrf_DataManagement_StorageSubscriptionRequest | The consumer (NWDAF or DCCF) uses this service operation to request the ADRF to initiate a subscription for data or analytics. Data or analytics provided in notifications as a result of the subsequent subscription by the ADRF are stored in the ADRF. Consumer: NWDAF, DCCF Producer: ADRF |
|  |  | Nadrf_DataManagement_StorageSubscriptionRemoval | The consumer NF uses this service operation to request that the ADRF no longer subscribes to data or analytics it is collecting and storing. Consumer: NWDAF, DCCF Producer: ADRF |
|  |  | Nadrf_DataManagement_RetrievalRequest | The consumer NF uses this service operation to retrieve stored data or analytics from the ADRF. The Nadrf_DataManagement_RetrievalRequest response either contains the data or analytics or provides instructions for fetching the data or analytics. Consumer: NWDAF, DCCF Producer: ADRF |
|  |  | Nadrf_DataManagement_RetrievalSubscribe | The consumer NF uses this service operation to retrieve stored data or analytics from the ADRF and to receive future notifications containing the corresponding data or analytics received by ADRF. Consumer: NWDAF, DCCF Producer: ADRF |
|  |  | Nadrf_DataManagement_RetrievalUnsubscribe | The consumer NF uses this service operation to request that the ADRF no longer sends data or analytics to a notification endpoint. Consumer: NWDAF, DCCF Producer: ADRF |
|  |  | Nadrf_DataManagement_RetrievalNotify | This service operation provides consumers with either data or analytics from an ADRF, or instructions to fetch the data or analytics from an ADRF. The notifications are provided to consumers that have subscribed using the Nadrf_DataManagement_RetrievalSubscribe service operation. Consumer: NWDAF, DCCF Producer: ADRF |
|  |  | Nadrf_DataManagement_Delete | This service operation instructs the ADRF to delete stored data. Consumer: NWDAF, DCCF Producer: ADRF |
| SA WG6 TS 23.436 [36] | A-ADRF Data Collection APIs | SS_AADRF_Data_Collection Subscribe | The consumer subscribes for offline data from A-ADRF. Consumer: ADAES Producer: A-ADRF |
|  |  | SS_AADRF_Data_Collection Notify | The consumer is receiving the offline data from A-ADRF as notification, based on subscription. Consumer: ADAES Producer: A-ADRF |
|  |  | SS_ AADRF_Historical_ServiceAPI_Logs Get | The consumer requests API logs from A-ADRF. Consumer: ADAES Producer: A-ADRF |
|  |  | SS_AADRF_NetworkSlice_Data Get | The consumer requests network slice data from A-ADRF. Consumer: ADAES Producer: A-ADRF |
|  |  | SS_AADRF_Location_Accuracy_Data Get | The consumer is receiving offline location analytics/data from A-ADRF. Consumer: ADAES Producer: A-ADRF |
|  |  | SS_AADRF_EdgeData_Collection Subscribe | The consumer subscribes for offline edge data from A-ADRF. Consumer: ADAES Producer: A-ADRF |
|  |  | SS_AADRF_EdgeData_Collection Notify | The consumer is receiving the offline edge data from A-ADRF as notification, based on subscription. Consumer: ADAES Producer: A-ADRF |
|  |  | SS_AADRF_Edge_Preparation_Data Get | The consumer is receiving offline edge computing preparation data from the A-ADRF. Consumer: ADAES Producer: A-ADRF |
|  |  | SS_AADRF_Data_Storage Request Subscription | The consumer requests A-ADRF to subscribe for data or analytics from ADAE server or A-DCCF for store. This service operation provides parameters needed by the A-ADRF to initiate the subscription (to an ADAE server or A-DCCF). Consumer: ADAE server, A-DCCF Producer: A-ADRF |
|  |  | SS_AADRF_Data_Storage Store Data | The consumer requests A-ADRF to store data or analytics from ADAE server or A-DCCF. Data or analytics are provided to the A-ADRF in the request message. Consumer: ADAE server Producer: A-ADRF |
|  |  | SS_ADRF_ ServerToServer_Analytics Get | The consumer is receiving offline server-to-server analytics/data from A-ADRF. Consumer: ADAES Producer: A-ADRF |
|  | A-DCCF Data Collection APIs | SS_ADCCF_Data_Collection Subscribe | The consumer subscribes to receive data or analytics from A-DCCF. The subscription includes service operation specific parameters that identify the data or analytics to be provided. Consumer: ADAE server Producer: A-DCCF |
|  |  | SS_ADCCF_Data_Collection Notify | The A-DCCF notifies the consumer of the requested data or analytics according to the request or notifies of the availability of previously subscribed data or analytics when data delivery is via the A-DCCF. The A-DCCF may also notify the consumer when data or analytics is to be deleted. Consumer: ADAE server Producer: A-DCCF |
|  |  | SS_ADCCF_Data_Collection Get | The consumer retrieves data or analytics from the A-DCCF. Consumer: ADAE server Producer: A-DCCF |
| RAN WG3 TS 38.423 [15] | Data Collection procedures | DATA COLLECTION REQUEST | NG-RAN node 1 initiates the procedure by sending the DATA COLLECTION REQUEST message to NG-RAN node 2 to start information reporting or to stop information reporting. Upon receipt, NG-RAN node 2: - shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to "start"; or - shall stop all measurements and predictions and terminate the reporting in case the Registration Request for Data Collection IE is set to "stop". Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message indicates the type of objects NG-RAN node 2 performs measurements or predictions on. |
|  |  | DATA COLLECTION RESPONSE | If NG-RAN node 2 is capable of providing all of the requested information, it shall initiate the information reporting as requested by NG-RAN node 1 and respond with the DATA COLLECTION RESPONSE message. If NG-RAN node 2 is capable of providing some but not all of the requested information, it shall initiate the information reporting for the admitted requested information and include the Node Measurement Initiation Result List IE or the Cell Measurement Initiation Result List IE or both in the DATA COLLECTION RESPONSE message. |
|  |  | DATA COLLECTION FAILURE | If none of the requested information can be initiated, NG-RAN node 2 shall send the DATA COLLECTION FAILURE message with an appropriate cause value. |
|  |  | DATA COLLECTION UPDATE | NG-RAN node 2 shall include in the DATA COLLECTION UPDATE message one or more of the following IEs based on the request: SSB Area Radio Resource Status List IE, Predicted Radio Resource Status, Predicted Number of Active UEs, Predicted RRC Connections, Average UE Throughput DL, Average UE Throughput UL, Average Packet Delay, Average Packet Loss, Energy Cost and Measured UE Trajectory. These IEs are specified in Rel. 18 to support three AI/ML for NG-RAN use cases, i.e., Energy Saving, Load Balancing and Mobility Optimization. |

Editors’ note:		Some of the SA6 defined APIs in table above are defined in Rel-19 but are not complete yet.

End of modifications