3GPP TSG-SA WG4 Meeting #132	S4-250887

# Japan, Fukuoka, 19 – 23 May 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.522 | CR | 0017 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5G_RTP_Ph2] Correction of URN and SDP signalling for RTP HE for dynamically changing traffic characteristics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 2025-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The URN currently includes the WG and WI code ("sa4:5grtp"), which is not consistent with the URN schemes used for other RTP header extensions defined in 3GPP specifications (TS 26.522 and TS 26.114). Including the WG and WI code is unnecessary for unique and unambiguous identification, and introduces inconsistency in URN naming conventions across specifications. Furthermore, the SDP signalling ABNF syntax does not include extensionattributes as per RFC 8285 to define the format extension attribute. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the "sa4:5grtp" part in the URN for the RTP header extension for dynamically changing traffic characteristics and add extensionattributes to SDP syntax. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Inconsistent naming scheme for RTP header extensions in 3GPP specifications. - Incomplete ABNF syntax for SDP signalling |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.5, D.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 4.5.5	SDP Signaling

An RTP sender capable of sending RTP HE for dynamically changing traffic characteristics shall use the SDP extmap attribute for RTP HE for dynamically changing traffic characteristics in the media description of the RTP stream(s) carrying the RTP HE for dynamically changing traffic characteristics. An RTP receiver that does not support RTP HE for dynamically changing traffic characteristics can ignore that RTP HE when included. The signaling of the RTP HE for dynamically changing traffic characteristics shall follow the SDP signaling design and the syntax and semantics of the "extmap" attribute as outlined in RFC 8285. The URN for the dynamically changing traffic characteristics marking shall be set to "urn:3gpp:dynamic-traffic-characteristics:rel-19".

The ABNF syntax for the extmap attribute for the signaling of RTP HE for dynamically changing traffic characteristics is defined as follows, extending the ABNF in RFC 8285:

extensionname = " urn:3gpp:dynamic-traffic-characteristics:rel-19"

extensionattributes = [format]

format = "short" / "long"

The extension attributes have the following semantics:

-	format: indicates if the RTP HE for dynamically changing traffic characteristics uses the 1-byte (short) or the 2-byte (long) format. This extension attribute shall not be included more than once.

NOTE:	Regardless of whether this extension attribute is present or not, the use of long or short format is determined as described by section 4.1.2 of RFC 8285, i.e., based on what format other RTP HEs use in the same RTP session, unless both endpoints announced support for handling mixed format with "a=extmap-allow-mixed" as described by section 6 of RFC 8285 [7].

Below is an example:

a=extmap:7 dynamic-traffic-characteristics:rel-19 long

# * * * * Second change * * * *

# D.5	urn:3gpp:dynamic-traffic-characteristics:rel-19

The desired extension naming URI:

urn:3gpp:dynamic-traffic-characteristics:rel-19

A formal reference to the publicly available specification:

TS 26.522

A short phrase describing the function of the extension:

Marking of dynamically changing traffic characteristics such as burst size and time to next burst

Contact information for the organization or person making the registration:

3GPP Specifications Manager

3gppContact@etsi.org

+33 (0)492944200