3GPP TSG-SA #108	SP-250784

Prague, Czech Republic, 10-13 June 2025	(was SP-250xxx)

Source:	Qualcomm Incorporated

Title:	Way Forward on AIoT device credentials storage

Document for:	Discussion/Approval

Agenda Item:	3.3

Work Item / Release:	Ambient_IoT-SEC/ Rel-19

Abstract of the contribution: Proposes a way forward for AIoT device credentials storage.

# 1.	Proposal

It is proposed that SA#108 endorses the following guidance for AIoT device credentials storage:

1) 	when the AIoT system is defined as private network e.g. SNPN, the AIoT device credentials storage follows 3GPP defined requirements, the exact mechanism is out of scope of 3GPP (similar to Annex I.2.2 of TS 33.501 ;

2) 	when the AIoT system is defined as public network i.e. PLMN, the AIoT device credentials storage shall use UICC.

NOTE 0: The above  requirements are based on the assumption of no interconnection between AIoT systems in rel.19. Revisiting the above requirement can happen in future releases.

NOTE 1: In case UICC is used, the exact form factor and whether it is removable, non-removable or integrated is out of scope of 3GPP.

NOTE 2: It is up to the operator to define the AIoT system as private network e.g. SNPN or public network i.e. PLMN.

The above guidance, if endorsed, can be sent to SA3 and other involved WGs with LS from SA#108, or documented in the meeting minutes of SA#108.