3GPP SA3LI#97	s3i250187

# 29 April – 02 May 2025, Washington DC (US)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0280 | rev | 1 | Current version: | 17.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Identifier deassociation xIRI to support alignment with stage 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Ericsson, EVE Compliancy Solutions) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI17 |  |  |  |  |  | Date: |  |  | 2025-04-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 33.128 a (x)IRI to report Identifier Deassociation for a LI target was missing. SA3-LI agreed to use a separate message rather than enhancing the already existing Identifier Association (x)IRI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Identifier Deassociation xIRI is introduced while the existing stage 2 requiring an indication of the association change type in the Identifier Association xIRI is deleted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between stage 2 and stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.4; 6.2.2.6; 6.3.2.3.1; 6.3.2.3.2; 6.3.2.5.1; 6.3.2.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 33.128 CR 0726 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | s3i250127 |  |  |  |  |  |  |  |  |

*** FIRST CHANGE ***

#### 6.2.2.4	IRI events

The IRI-POI present in the AMF shall generate xIRI, when it detects the following specific events or information:

-	Registration.

-	Deregistration.

-	Location update.

-	Identifier association.

-	Identifier deassociation.

-	Start of interception with already registered UE.

-	Unsuccessful communication related attempt.

-	Positioning info transfer.

NOTE:	AMF reporting of UE state changes other than registration or deregistration is not supported in the present document.

The registration xIRI is generated when the IRI-POI present in an AMF detects that a target UE has successfully registered to the 5GS via 3GPP NG-RAN or non-3GPP access. The registration xIRI describes the type of registration performed (e.g. initial registration, periodic registration, registration mobility update) and the access type (e.g. 3GPP, non-3GPP). Unsuccessful registration shall be reported only if the target UE has been successfully authenticated.

The deregistration xIRI is generated when the IRI-POI present in an AMF detects that a target UE has deregistered from the 5GS. The deregistration xIRI shall indicate whether it was a UE-initiated or a network-initiated deregistration.

The location update xIRI is generated each time the IRI-POI present in an AMF detects that the target's UE location is updated due to target UE mobility (e.g. in case of Xn based inter NG-RAN handover) or when the AMF observes target UE location information during some service operation (e.g., LCS, Location Reporting, or emergency services). The generation of such xIRI may be omitted if the updated UE location information is already included in other xIRIs (e.g. mobility registration) provided by the IRI-POI present in the same AMF. If the information in the AMF received over N2 (TS 38.413 [14]) includes one or more cell IDs, then all cell IDs shall be reported to the LEMF whenever location reporting is triggered at the AMF.

The identifier association xIRI is generated each time the IRI-POI in the AMF detects a SUCI or 5G-GUTI allocation for a SUPI associated with the target UE.

The identifier deassociation xIRI is generated each time the IRI-POI in the AMF detects a 5G-GUTI deallocation for a SUPI associated with the target UE, without any new allocation replacing the existing one.

The start of interception with already registered UE xIRI is generated when the IRI-POI present in an AMF detects that interception is activated on the target UE that has already been registered in the 5GS.

When additional warrants are activated on a target UE, MDF2 shall be able to generate and deliver the start of interception with already registered UE related IRI messages to the LEMF associated with the warrants without receiving the corresponding start of interception with already registered UE xIRI.

The unsuccessful communication related attempt xIRI is generated when the IRI-POI present in an AMF detects that a target UE initiated communication procedure (e.g. session establishment, SMS) is rejected or not accepted by the AMF before the proper NF handling the communication attempt itself is involved. The unsuccessful communications related attempt xIRI is also generated when the IRI-POI present in the AMF detects that a PDU session modification request to convert a single access PDU session to a Multi-Access PDU (MA PDU) session is not accepted by the AMF and therefore not forwarded to the SMF.

The IRI-POI in the AMF shall support per target selective activation or deactivation of reporting of identifier association xIRI independently of activation of LI for all other events. When identifier association xIRI only reporting is activated, the IRI-POI in the AMF shall also generate location update xIRI.

The positioning info transfer xIRI is generated when the IRI-POI present in the AMF detects one the the following events:

-	network-based or network-assisted positioning requests, responses or reports related to a target UE are being exchanged between LMF and NG-RAN via the AMF.

-	UE-based or UE-assisted positioning requests, responses or reports related to a target UE are being exchanged between LMF and the target UE via the AMF.

NOTE:	The activation and invocation of the positioning info transfer capability exclusively for LALS is not supported in the current version of the specification. Instead, the capability is invoked whenever any LCS operation (including LALS) is performed on the target.

*** NEXT CHANGE ***

#### 6.2.2.6	Specific IRI parameters

The list of parameters in each xIRI are defined in TS 33.128 [15]. The following give a summary.

The registration xIRI shall include the following:

-	Registration type information.

-	Access type information.

-	Requested slice information.

The deregistration xIRI shall include the following:

-	UE initiated de-registration.

-	Access type information.

-	Network initiated de-registration.

The location update xIRI shall include the following:

-	Location of the target UE (see clause 7.3).

The identifier association xIRI shall include the following:

-	Subscription permanent identifier.

-	Temporary identifier association (i.e. SUCI or 5G-GUTI).

The identifier deassociation xIRI shall include the following:

-	Subscription permanent identifier.

-	Temporary identifier association (i.e. 5G-GUTI).

The start of interception with already registered UE xIRI shall include the following:

-	Access type information.

-	Requested slice information.

The unsuccessful communication attempt xIRI shall include the following:

-	Rejected type of communication attempt.

-	Access type information.

-	Failure reason.

When the access type is non-3GPP, the IP address used by the UE to reach the N3A Entity shall be reported. The port shall also be reported if available.

*** NEXT CHANGE ***

##### 6.3.2.3.1	Option A

The IRI-POI present in the MME shall generate xIRI, when it detects the following specific events or information:

-	Attach.

-	Detach.

-	Tracking Area/EPS Location Update.

-	Start of interception with EPS attached UE.

-	Unsuccessful communication related attempt.

-	Identifier association.

-	Identifier deassociation.

-	Positioning info transfer.

The attach xIRI is generated when the IRI-POI present in an MME detects that a target UE has performed an E-UTRAN attach procedure including via a HeNB. The attach xIRI describes the type of attach performed. Unsuccessful registration shall be reported only if the target UE has been successfully authenticated.

The detach xIRI is generated when the IRI-POI present in an MME detects that a target UE has detached from the EPS including via a HeNB. The detach xIRI shall indicate whether it was a UE-initiated or a network-initiated detach.

The tracking area/EPS location update xIRI is generated each time the IRI-POI present in an MME detects that the target UE location is updated due to target UE mobility (e.g. in case of X2 based handover, S1 based handover) or when the MME observes target UE location information during some service operation (e.g., periodic Tracking Area Update, UE triggered Service Request). If the information in the MME received over S1 (TS 36.413 [14]) includes one or more cell IDs, then all cell IDs shall be reported to the LEMF whenever location reporting is triggered at the MME.

The start of interception with EPS attached UE xIRI is generated when the IRI-POI present in an MME detects that interception is activated on a target UE that is already attached to the EPS. If there are multiple PDN connections active for the target, then a start of interception with EPS attached UE xIRI is generated for each of them.

When additional warrants are activated on a target UE, MDF2 shall be able to generate and deliver the start of interception with E-UTRAN attached UE related IRI messages to the LEMF associated with the warrants without receiving the corresponding start of interception with already registered UE xIRI.

The unsuccessful communication related attempt xIRI is generated when the IRI-POI present in an MME detects that a target UE initiated communication procedure (e.g. service request, SMS) is rejected or not accepted by the MME before the proper NF handling the communication attempt itself is involved.

The identifier association xIRI is generated each time the IRI-POI in the MME detects a GUTI allocation for an IMSI associated with the target UE.

The identifier deassociation xIRI is generated each time the IRI-POI in the MME detects a GUTI deallocation for an IMSI associated with the target UE, without any new allocation replacing the existing one.

The IRI-POI in the MME shall support per target selective activation or deactivation of reporting of only identifier association/deassociation xIRI independently of activation of LI for all other events. When identifier association/deassociation xIRI only reporting is activated, the IRI-POI in the MME shall also generate Tracking Area/EPS Location Update xIRI.

The positioning info transfer xIRI is generated when the IRI-POI present in the MME detects one the following events:

-	network-based or network-assisted positioning requests, responses or reports related to a target UE are being exchanged between E-SMSC and eNB via the MME.

-	UE-based or UE-assisted positioning requests, responses or reports related to a target UE are being exchanged between E-SMLC and the target UE via the MME.

NOTE:	The activation and invocation of the positioning info transfer capability exclusively for LALS is not supported in the current version of the specification. Instead, the capability is invoked whenever any LCS operation (including LALS) is performed on the target.

*** NEXT CHANGE ***

##### 6.3.2.3.2	Option B

The IRI-POI present in the MME shall generate xIRI, when it detects the applicable events specified in TS 33.107 [11].

In addition to the events specified in TS 33.107 [11] the MME shall generate xIRI, when it detects the following additional event:

-	Identifier association.

Identifier deassociation.

The identifier association xIRI is generated each time the IRI-POI in the MME detects a GUTI allocation for an IMSI associated with the target UE.

The identifier deassociation xIRI is generated each time the IRI-POI in the MME detects a GUTI deallocation for an IMSI associated with the target UE, without any new allocation replacing the existing one.

The IRI-POI in the MME shall support per target selective activation or deactivation of reporting of only identifier association/deassociation xIRI independently of activation of LI for all other events. When identifier association/deassociation xIRI only reporting is activated, the IRI-POI in the MME shall also generate Tracking Area/EPS Location Update xIRI.

*** NEXT CHANGE ***

##### 6.3.2.5.1	Option A

The list of parameters in each xIRI are defined in TS 33.128 [15]. The following give a summary.

The attach xIRI shall include the following:

-	Attach type information.

-	Access type information.

-	HeNB information.

The detach xIRI shall include the following:

-	Detach Direction.

-	Detach type information.

-	HeNB information.

The tracking area/EPS location update xIRI shall include the following:

-	Location of the target (see clause 7.3).

-	HeNB information.

The start of interception with EPS attached UE xIRI shall include the following:

-	Attach type information.

-	Access type information.

-	PDN connection information.

The unsuccessful communication attempt xIRI shall include the following:

-	Rejected type of communication attempt.

-	Access type information.

-	Failure reason.

The identifier association xIRI shall include the following:

-	IMSI.

-	IMEI.

-	Temporary identifier association (i.e. GUTI).

The identifier deassociation xIRI shall include the following:

-	IMSI.

-	IMEI.

-	Temporary identifier association (i.e. GUTI).

*** NEXT CHANGE ***

##### 6.3.2.5.2	Option B

The list of parameters in each xIRI are defined in TS 33.128 [15], for events which are imported from TS 33.107 [11] clause 12.2.1.2.

The identifier association xIRI shall include the following:

-	IMSI.

-	IMEI.

-	Temporary identifier association (i.e. GUTI).

The identifier deassociation xIRI shall include the following:

-	IMSI.

-	IMEI.

-	Temporary identifier association (i.e. GUTI).

*** END OF CHANGES ***