3GPP TSG-WG SA2 Meeting #169                                       	S2-2506080

# Fukuoka, Japan, 19-23 May 2025		   											    			    Was S2-2505905

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1556 | rev | 2 | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | BSF subscription validity time |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Oracle, Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI18 |  |  |  |  |  | Date: |  |  | 2025-5-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A consumer such as AF, NEF or PCF for a UE may subscribe at the BSF to receive notification when a PCF for PDU session or a PCF for a UE respectively registers itself in the BSF. In a large network there might be hundreds of thousands of such subscriptions. Many of them may get stale, occupying BSF resources. Consequently, BSF needs to be able to detect Validity of a subscription for either session binding or UE binding and perform cleanup operation if the session is not active. Alternatively, it can alert the consumer, and have it cancel a stale subscription itself. Similar subscription validity time functionality already exists for bulk subscription to PCF EE notifications (23.503 6.1.3.18) and for subscription to UDR data modification notifications (29.519 6.2, 6.4.2.10). In addition, the ability of BSF to allow a consumer to subscribe to the BSF to be notified when a new PCF is registered for a UE or for a PDU session, has never been properly documented in the main BSF clause in 23.503. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to add the mechanism of validity time to BSF subscription. In addition, a description of the BSF subscribe/notify functionality is added in the main clause of the BSF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No subscription validity time. Subscription validity time is crucial as it facilitates efficient resource allocation and prevents stale subscriptions which can lead to service disruptions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.2.2, 6.1.1.2a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

##### 6.1.1.2.2	The Binding Support Function (BSF)

The BSF has the following characteristics:

-	The BSF stores internally information about the corresponding selected PCF:

-	For a certain PDU Session, the BSF stores internally information about the user identity, the DNN, the UE (IP or MAC) address(es), the S-NSSAI, the selected PCF address and if available the associated PCF instance ID, PCF set ID and the level of binding (see clause 6.3.1.0 of TS 23.501 [2]).

-	For a certain UE, the BSF stores internally information about the user identity, the selected PCF address and if available the associated PCF instance ID, PCF set ID and the level of binding (see clause 6.3.1.0 of TS 23.501 [2]).

NOTE 1:	Only NF instance or NF set Level of Binding indication are supported at the BSF.

-	The PCF registers, updates and removes the stored information in the BSF using the Nbsf management service operations defined in TS 23.502 [3]:

-	For a PDU Session, the PCF ensures that it is updated each time an IP address is allocated or de-allocated to the PDU Session or, for Ethernet PDU Sessions supporting binding of AF request based on MAC address, each time it has been detected that a MAC address is used or no more used by the UE in the PDU Session.

-	For a UE, the PCF ensures that it is updated each time the AMF selects a new PCF.

-	Based on operator's policies and configuration, the PCF determines whether the same PCF shall be selected for the SM Policy Associations to the same UE ID, S-NSSAI and DNN combination in the non-roaming or home-routed scenario.

NOTE 2:	This applies to usage monitoring.

-	Based on operator's policies and configuration, the PCF determines whether the same PCF shall be selected for the SM Policy Associations to the same UE ID and S-NSSAI combination in the non-roaming or home-routed scenario.

NOTE 3:	This applies to network slice related policy control.

-	The selected PCF (if needed) downloads the user profile from the UDR as described in clause 4.16.4 step 2 of TS 23.502 [3]. If usage monitoring is enabled for the user, and based on operator's policies, the PCF checks if the BSF has already existing PCF serving the combination of SUPI, S-NSSAI and DNN:

-	If no such PCF is found the PCF shall register itself to the BSF as described above in this clause.

-	Else if an existing PCF is found for the above combination, the PCF shall return to the SMF the available information about the existing PCF and a redirection indication.

NOTE 4:	The assumption is that for DNN, S-NSSAI combinations where usage monitoring be applied, the same BSF instance or the same BSF SET is selected for all UE PDU Sessions to the same DNN, S-NNSAI.

-	The BSF verifies whether to provide the address of a PCF for the PDU Session or a PCF for the UE according to the information provided by the consumer NF (e.g. the AF, the NEF, or the PCF for the UE). If the consumer NF provides the user identity and neither a UE address nor a (DNN, S-NSSAI) tuple, the BSF shall provide the address of the PCF for the UE. If the consumer NF (e.g. the PCF for the UE) provides the user identity (e.g. SUPI or GPSI) and the tuple (DNN, S-NSSAI), the BSF shall provide the address of a PCF for the PDU Session for this UE. If the consumer NF (e.g. the AF or the NEF) provides the UE address, the BSF shall provide the address of a PCF for the PDU Session.

NOTE 5:	It is up to stage3 to ensure an unambiguous error proof way for the BSF to differentiate between PCF for the PDU Session and PCF for the UE. This might or might not require providing the BSF additional parameter(s) when a PCF registers itself with the BSF and/or when a consumer attempts to discover a PCF via discovery or subscription.

-	For retrieval binding information, any NF, such as NEF or AF, that needs to discover the selected PCF address(es), and if available, the associated PCF instance ID, PCF set ID and level of binding (see clause 6.3.1.0 of TS 23.501 [2]) for the tuple (UE address, DNN, S-NSSAI, SUPI, GPSI) (or for a subset of this Tuple) uses the Nbsf management service discovery service operation defined in TS 23.502 [3].

-	The NF may discover the BSF via NRF or based on local configuration. When registering the NF profile in NRF, the Range(s) of UE IPv4 addresses, Range(s) of UE IPv6 prefixes supported by the BSF and optionally, the DNN list, S-NSSAI(s) or IP domain list as described in TS 29.510 [32], may be provided to NRF.

-	If the NF received a PCF set ID or a PCF instance ID with an indication of level of binding as result of the Nbsf management service discovery service operation, it should use that information as NF set level or NF instance level Binding Indication to route requests to the PCF as defined in clause 6.3.1.0 of TS 23.501 [2] and according to the following provisions:

-	For the NF set level of binding, the NF will receive a PCF set ID but no PCF instance ID. If an NF is not able to reach the received PCF address(es) and applies direct discovery, it should query the NRF for PCF instances within the PCF set and select another instance.

-	For the NF instance level of binding, the NF will receive a PCF set ID and a PCF instance ID. If an NF is not able to reach the received PCF address(es) and applies direct discovery, it should query the NRF for PCF service instances within the PCF and select another instance.

-	The NF should provide a Routing Binding Indication based on the received PCF set ID, level of binding and possible PCF instance ID in requests it sends to the PCF.

-	For an ongoing NF service session, the PCF may provide Binding indication to the NF (see clause 6.3.1.0 of TS 23.501 [2]). This Binding indication shall then be used instead of any PCF information received from the BSF.

-	If a new PCF instance is selected, the new PCF should invoke Nbsf_Management_Update service operation to update the binding information in BSF.

The BSF may be deployed standalone or may be collocated with other network functions, such as PCF, UDR, NRF and SMF.

NOTE 6:	Collocation allows combined implementation.

A consumer may subscribe to the BSF to be notified when a PCF for the PDU Session for a certain SUPI is registered in the BSF. The subscription may include an expiration time. When the subscription expires, if the NF Service Consumer wants to keep receiving notifications, it shall create a new subscription in the BSF.

# *** 2nd Change ***

#### 6.1.1.2a	Binding an NF request targeting a UE to the relevant PCF for the UE

When multiple and separately addressable PCFs have been deployed, a network functionality is required in order to ensure that a consumer NF (e.g. AF, 5G DDNMF) reaches the PCF serving the UE. This network functionality is provided by the BSF (described in clause 6.1.1.2.2) and has the following characteristics:

-	It has information about the user identity, and the selected PCF address for a certain UE.

-	The functionality determines the PCF address and if available the associated PCF instance ID and PCF set ID, selected by the PCF discovery and selection function described in TS 23.501 [2].

NOTE:	The above is required e.g. for dynamic control of access and mobility related policy control functionality or for reporting events for a UE to a consumer such as 5G DDNMF.

A consumer may subscribe to the BSF to be notified when a PCF for the UE is registered in the BSF for a certain SUPI or GPSI. The subscription may include an expiration time. When the subscription expires, if the NF Service Consumer wants to keep receiving notifications, it shall create a new subscription in the BSF.

# *** End of Change ***