3GPP TSG SA WG2#168	            S2-2504399

Goteborg, Sweden, 07 – 11 April 2025	(revision of S2-2503853)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6266 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on NR Femto node deployments |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_Femto |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | NR Femto RAN architecture is defined in RAN specs. Hence, there is no need to have below note. NOTE: For NR Femto deployments, it is considered that NR Femto nodes are connected to 5GC, not EPC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removes the Note in clause 5.50.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.50.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.50.1	Overview

This clause provides an overview of 5GS functionalities and architecture to support NR Femto deployments.

For the support of NR Femto in 5GS, the architecture is described in clause 5.50.2.

For the UE access control to the NR Femto cell, the existing CAG concept defined in the clause 5.30.3 for PNI-NPN is used. The Femto cell may broadcast one or multiple CAG Identifiers as specified in the clause 5.30.3 if access control is needed.

The CAG owner or an authorized administrator may control/provision subscribers to allow accessing their NR Femto/CAG cells as described in clause 5.50.3.

An NR Femto Hosting Party plays the role of a CAG owner.

End of CHANGES