3GPP TSG-SA2 Meeting #168	S2-2504216

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1425 | rev | 2 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Parameters for vertical federated learning inference procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Vivo, Huawei, Oppo, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some parameters as defined for the the analytics request (as defined in Clause 6.1.3 of TS 23.228) that cannot be handled by the VFL server alone need to be forwarded to the VFL clients also to the VFL interence service. See considerations in S2-2503233. In addition, the data time window is already documented in the call flow but missing in the service operations. Some ENs for the services can then be removed: No more parameters are expected: Also a separate service has been defined to request inference from a AF VFL server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add parameters as defined for the the analytics request (as defined in Clause 6.1.3 of TS 23.228) that cannot be handled by the VFL server alone but need to be forwarded to the VFL clients also to the VFL interence service. Also add the data time window as parameter to the service operations. Removed resolved ENs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not all analyitics requests can be fully supported via VFL. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.4.1, new 6.2H.2.4.x, 7.13.2, 7.13.4, 7.13.5, 11.3.1, 11.3.2, 11.3.4, 11.3.5, 12.3.1, 12.3.2, 12.3.4, 12.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

##### 6.2H.2.4.1	Inference procedure for vertical federated learning when NWDAF or Trusted AF is acting as VFL server

Figure 6.2H.2.4.2-1: Inference procedure for vertical federated learning when NWDAF or Trusted AF is acting as VFL server

The inference procedure when trusted AF is acting as VFL server may be triggered by a request or subscription from a 5GC consumer NF or internal service logic of the AF acting as VFL server. If triggered by internal service logic of the AF acting as VFL server, the steps 0, 1,7 and 8 are skipped.

0.	The analytics consumer NF sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting= e.g. UE IDs and optionally both Analytics Reporting Information=Analytics target period and Analytics Filter, Analytics Accuracy Request if the analytics consumer NF requests Analytics Accuracy Monitoring) to NWDAF containing AnLF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe., providing parameters as defined in clause 6.1.3.

1.	If the NWDAF containing AnLF can be the VFL server to generate the VFL inference results for the requested analytics ID, then step 1 is skipped.

If the NWDAF containing AnLF can not generate the analytics output, the NWDAF containing AnLF determines the VFL Server AF for the requested analytics.

If the VFL server is trusted AF, the NWDAF containing AnLF sends a request to trusted AF VFL server using Naf_Inference_subscribe/request including Analytics ID, Target of Analytics Reporting = e.g. UE IDs and optionally Analytics Reporting Information=Analytics target period Analytics Filter and Analytics Accuracy Request.

2.	Based on the information received in the step 0 or 1, VFL server decides to initiate the VFL inference procedure with the VFL clients. Before the VFL server initiate the VFL inference procedure, the VFL server may initiate the VFL training procedure if no VFL model is already trained as described in the clause 6.2H.2.3 based on the information received in the step 0 or 1 or VFL server local configuration. VFL Server selects clients(s) using information stored in the VFL training process. The VFL server may select some or no clients, e.g. depending on the accuracy of the VFL model, the contribution to the training result and the current status of the VFL clients.

When no VFL Clients are selected, the VFL server may generates the VFL inference results based only on its local trained ML model associated with the determined VFL correlation ID, skipping the steps 2 - 6 (and 7 if step 1 was also skipped).

NOTE 1:	If the server does not select VFL clients for all features, it can internally store information about the features considered when deriving the analytics results together with a timestamp to identify the analytics results, to enable to subsequently trace the origin of analytics results and to interpret possible feedback on analytics accuracy.

NOTE 2:	If a client is not available for inference, e.g. depending on the contribution to the training result, the server can decide to select another available VFL client with a well-trained model, or the VFL server can start a new training procedure for a new VFL client.

VFL server NWDAF or trusted AF determines and sends a VFL Inference request/subscription to each VFL client including the Target of VFL inference = e.g. UE IDs, VFL correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used and optionally VFL inference filter, Data time window,Time when intermediate local result is needed, Dataset Statistical Properties,  Analytics Metadata Request, as defined in clause 6.2H.2.4.x.

2a.	For each NWDAF VFL client, the VFL Server NWDAF or trusted AF sends an Nnwdaf_VFLInference_Subscribe or Nnwdaf_VFLInference_Request to the VFL client.

2b.	For each trusted AF VFL client, the VFL Server NWDAF sends an Naf_VFLInference_Subscribe or Naf_VFLInference_Request to the VFL client.

2c.	For each untrusted AF VFL client, the VFL Server NWDAF sends an Nnef_VFLInference_Subscribe or Nnef_VFLInference_Request to the NEF serving the AF.

2d.	For each untrusted AF VFL client, the NEF converts any internal identifiers to external identifiers and sends an Naf_VFLInference_Subscribe or Naf_VFLInference_Request to the untrusted AF VFL client.

NOTE 3:	For the trusted AF is acting as VFL server, the VFL client can only be the NWDAF.

3.	Each VFL Client collects its local data by using mechanism as specified in Clause 6.2 and considering a possibly received data time window to ensure that different VFL clients use the inference data collected in the same time period if the VFL Client does not have local data available already.

4.	Based on the VFL correlation ID, each VFL Client determines the VFL local model to generate the intermediate local inference results.

NOTE 4:	In this Release, it is assumed that local intermediate inference is shared between VFL server and VFL client.

5.	VFL Client sends the client intermediate local results to the VFL server. If requested in step 2, it includes Analytics Metadata, as defined in clause 6.2H.2.4.x. Alternatively, the VFL client may report their status and other cause value for rejecting the VFL inference process (e.g. overload, target UE moved out of NWDAF serving area) in the VFL inference response service.

The intermediate local results, which are sent from the VFL Client to the VFL Server during the VFL inference process, are the information for the VFL Server to combine and generate the VFL inference results.

If the VFL server used an inference subscription in step 2, step 5 may be repeated.

5a.	Each NWDAF VFL client sends an Nnwdaf_VFLInference_Notify or Nnwdaf_VFLInference_Request response to the VFL Server NWDAF or trusted AF.

5b.	Each trusted AF VFL client sends a Naf_VFLInference_Notify or Naf_VFLInference_Request response to the VFL Server NWDAF.

5c.	Each untrusted AF VFL client sends a Naf_VFLInference_Notify or Naf_VFLInference_Request response to the NEF.

5d.	For each untrusted AF VFL client, the NEF converts any external to internal identifiers and sends an Nnef_VFLInference_Notify or Nnef_VFLInference_Request response to the NWDAF VFL server.

6.	The VFL server may collect its local data and generate the intermediate local inference results. When the VFL Server selected VFL clients to participate in the VFL Inference process, it combines all the intermediate local results to generate the combined inference results based on the VFL correlation ID. The VFL server takes into account the participation of each VFL client during the ML training process and the importance of the intermediate local results when it generates the combined inference results.

The VFL server may compute the VFL accuracy based on all the intermediate local results received from VFL clients and the label, if it receives Analytics accuracy request in step 0. The server also aggregates possibly received analytics metadata.

7.	Depending on request, the NWDAF or trusted AF as VFL server sends Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify or Naf_Inference_Response/Notify to the consumer (i.e. NWDAF containing AnLF) including the VFL inference results, optionally, VFL accuracy.

8.	The NWDAF containing AnLF provides the analytics output to the analytics consumer NF based on the VFL inference results by means of either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 0.

The NWDAF containing AnLF may provide VFL accuracy if the consumer NF provides Analytics Accuracy request in step 0.

NOTE 3:	There may be some time delay between the time when the AnLF provides the analytics output to consumer NF and the time when the AnLF provides VFL accuracy, as the labels are collected after making predictions. The analytics result and VFL accuracy may be send in different response messages.

2nd change

### 7.13.2	Nnwdaf_VFLInference_Subscribe service operation

Service operation name: Nnwdaf_VFLInference_Subscribe

Description: Subscribe to VFL inference.

Inputs, Required:

For new subscription:

-	Notification Target Address (+ Notification Correlation ID).

-	Analytics ID.

-	VFL Correlation ID.

-	Target of VFL inference.

When updating a subscription:

-	Subscription Correlation ID.

Inputs, Optional:

-	VFL inference filter.

-	Data time window.

-	Time when intermediate local result is needed.

-	Dataset Statistical Properties.

-	Analytics metadata request.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: Client intermediate results.

3rd change

### 7.13.4	Nnwdaf_VFLInference_Notify service operation

Service operation name: Nnwdaf_VFLInference_Notify

Description: Notify VFL inference result.

Inputs, Required:

-	Notification Correlation Information.

Inputs, Optional:

-	client intermediate results.

-	Analytics Metadata Information.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

4th change

### 7.13.5	Nnwdaf_VFLInference_Request service operation

Service operation name: Nnwdaf_VFLInference_Request

Description: The consumer requests the NWDAF to perform a one-time VFL inference.

Inputs, Required:

-	Target of VFL inference.

-	VFL Correlation ID.

-	Analytics ID.

Inputs, Optional:

-	VFL inference filter.

-	Data time window.

-	Time when intermediate local result is needed.

-	Dataset Statistical Properties.

-	Analytics metadata request.

Outputs, Required: If the request is accepted, then client intermediate results. When the request is not accepted, an error response.

Outputs, Optional:

-	Analytics Metadata Information.

.

5th change: 11.3	Naf_VFLInference Service

### 11.3.1	General

Service Description: This service is provided by AF acting as VFL client and enables an VFL server as consumer to request or subscribe/unsubscribe for a VFL inference.

When the subscription is accepted by the AF, the consumer receives from the NWDAF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription.

6th change

### 11.3.2	Naf_VFLInference_Subscribe service operation

Service operation name: Naf_VFLInference_Subscribe

Description: Subscribe to VFL inference.

Inputs, Required:

For new subscription:

-	Notification Target Address (+ Notification Correlation ID).

-	VFL Correlation ID.

-	Target of VFL inference.

-	Analytics ID.

When updating a subscription:

-	Subscription Correlation ID.

Inputs, Optional:

-	VFL inference filter.

-	Data time window.

-	Time when intermediate local result is needed.

-	Dataset Statistical Properties.

-	Analytics metadata request.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: Client intermediate results.

7th change

### 11.3.4	Naf_VFLInference_Notify service operation

Service operation name: Naf_VFLInference_Notify

Description: Notify VFL inference result.

Inputs, Required:

-	Notification Correlation Information.

Inputs, Optional:

-	Client intermediate results.

-	Analytics Metadata Information.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

8th change

### 11.3.5	Naf_VFLInference_Request service operation

Service operation name: Naf_VFLInference_Request

Description: The consumer requests the AF to perform a one-time VFL inference.

Inputs, Required:

-	Target of VFL inference.

-	VFL Correlation ID.

-	Analytics ID.

Inputs, Optional:

-	VFL inference filter.

-	Data time window.

-	Time when intermediate local result is needed.

-	Dataset Statistical Properties.

-	Analytics metadata request.

Outputs, Required: If the request is accepted, then client intermediate results. When the request is not accepted, an error response.

Outputs, Optional:.

-	Analytics Metadata Information.

9th change: Nnef_VFLInference Service

### 12.3.1	General

Service Description: This service is provided by by an NEF on behalf of an AF acting as VFL client and enables an VFL server as consumer to request or subscribe/unsubscribe for a VFL inference.

When the subscription is accepted by the AF, the consumer receives from the NWDAF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription.

10th change

### 12.3.2	Nnef_VFLInference_Subscribe service operation

Service operation name: Nnef_VFLInference_Subscribe

Description: Subscribe to VFL inference.

Inputs, Required:

For new subscription:

-	Notification Target Address (+ Notification Correlation ID).

-	VFL Correlation ID.

-	Target of VFL inference.

When updating a subscription:

-	Subscription Correlation ID.

-	For NWDAF as VFL client, external NWDAF ID.

Inputs, Optional:

-	VFL inference filter.

-	Data time window.

-	Time when intermediate local result is needed.

-	Dataset Statistical Properties.

-	Analytics metadata request.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: Client intermediate results.

11th change

### 12.3.4	Nnef_VFLInference_Notify service operation

Service operation name: Nnef_VFLInference_Notify

Inputs, Required:

-	Notification Correlation Information.

Inputs, Optional:

-	Client intermediate results.

-	Analytics Metadata Information.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

12th change

### 12.3.5	Nnef_VFLInference_Request service operation

Service operation name: Nnef_VFLInference_Request

Description: The consumer requests the NWDAF to perform a one-time VFL inference.

Inputs, Required:

-	Target of VFL inference.

-	VFL Correlation ID.

-	Analytics ID.

Inputs, Optional:

-	VFL inference filter.

-	Data time window.

-	Time when intermediate local result is needed.

-	Dataset Statistical Properties.

-	Analytics metadata request.

Outputs, Required: If the request is accepted, then client intermediate results. When the request is not accepted, an error response.

Outputs, Optional:

-	Analytics Metadata Information.

13th change

##### 6.2H.2.4.x 	Contents of VFL Inference service

The consumers of the VFL inference services may provide the input parameters in Nnwdaf_VFLInference service or Naf_VFLInference service or in Nnef_VFLInference service as listed below:

-	Analytics ID: identifies the analytics for which the ML Model is requested to be used for inference.

-	(Only for Inference subscribe service) A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from the VFL client with the subscription.

-	(for new Inference subscription or inference request) A VFL Correlation ID: indicate the VFL client which previously well-trained VFL local model associated with this ID will be used.

-	(for new Inference subscription or inference request) Target of VFL inference: indicates the object(s) for which data for VFL inference is requested, i.e. a list of SUPIs, group of UEs identified by a list of Internal-Group-Ids or External-Group-Ids.

-	(Only for new Inference subscribtion) Subscription Correlation ID.

-	(only for Nnef_VFLInference service) external NWDAF ID

-	[Optional] VFL inference filter: indicates the conditions to be fulfilled for generating intermediate local inference results, e.g. S-NSSAI, Area of Interest. Parameter types in the VFL inference filter are the same as parameter types in the Analytics Filter Information which are defined in procedures. S-NSSAI is only applicable for the Nnwdaf_VFLInference service.

-	[Optional] Time when intermediate local inference result is needed: indicates to the VFL client the latest time to receive intermediate local inference result。

-	[Optional] Dataset Statistical Properties: information in order to influence the data selection mechanisms to be used for the generation of an intermediate local inference results. See Clause 6.1.3 for details.

-	[Optional] Data time window: if specified, only events that have been created in the specified time interval are considered for the inference result generation.

-	[OPTIONAL] Analytics metadata request: indicates a request from VFL server to VFL client to provide the "analytics metadata information" related to the produced intermediate local inference results.

The VFL client provides to the consumer of the VFL inference service operations the output information in as listed below:

- (Only for Inference notify) Notification Correlation Information.

- Intermediate local inference results: the output of the client VFL local model, which is used for the VFL server to generate the VFL inference results.VFL correlation ID.

- Analytics ID.

- [Optional] Analytics metadata information: additional information required to aggregate the output analytics for the requested Analytics ID(s).

-	Number of data samples used for the generation of the output analytics.

-	Data time window of the data samples.

-	Dataset Statistical Properties of the analytics output used for the generation of the analytics.

-	[OPTIONAL] Data source(s) of the data used for the generation of the output analytics.

End of changes