3GPP TSG-SA2 Meeting #168	S2-2504083

# 7 – 11 April 2025, Gothenburg, Sweden

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5401 | rev | 2 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications to Direct subscription to UPF Events in case of session release or UPF relocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2025-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clarification is needed in clause 4.15.4.5.6 that Remaining data reporting indication is per Event and not for the subscription. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 4.15.4.5.6, clarified that Remaining Data reporting indication has to be per event |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.4.5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# ************************************ FIRST CHANGE *****************************

##### 4.15.4.5.6	Information flow for subscription to UPF event exposure service via SMF during UPF relocation and PDU Session release

Figure 4.15.4.5.6-1: Subscription to UPF event exposure service via SMF during UPF relocation and PDU Session release

This information flow is used in the scenario of UPF relocation, e.g. the source UPF is I-UPF or L-PSA which is relocated in SSC mode 1, or the source UPF is PSA UPF which is relocated in SSC mode 3. The information flow also applies to PDU Session release.

When the UPF event consumer targets a specific UE, the procedure is as follows:

1.	The UPF event consumer (e.g. NWDAF) subscribes to the UPF event(s) via SMF by invoking Nsmf_EventExposure_Subscribe service operation, and the Target of Event Reporting indicates a specific UE. The consumer may include a Remaining data reporting indication for an Event ID(s), which indicates how to handle the data collected for that specific event(s) by the source UPF when the N4 session is released (e.g. due to UPF relocation or PDU session release) i.e. either to discard the collected data or send the collected data to the consumer.

2.	The SMF selects the source UPF and subscribes to the source UPF by invoking Nupf_EventExposure_Subscribe service operation or via N4 Session Modification, depending on the event. The SMF forwards the Remaining data reporting indication to the source UPF if received in step 1.

3.	When the notification event is triggered, the UPF sends the Nupf_EventExposure_Notify service operation to the UPF event consumer, including the locally collected UPF data.

4.	The SMF determines that the UPF needs to be relocated or PDU Session needs to be released.

5.	In the case of UPF relocation, the SMF initiates a PDU Session update to relocate UPF (for example procedure in clause 4.3.5.3) including N4 session release in source UPF. In the case of PDU Session release, the SMF initiates a PDU Session release procedure, including N4 session release.

6.	When (source) UPF terminates the subscription upon release of the UE related N4 Session (step 5), the (source) UPF may send any collected data not yet sent to the UPF event consumer taking into account local configuration and/or the Remaining data reporting indication (if received) for an event(s) from SMF. For example, for periodic reporting type of the UPF event exposure service when the UPF changes, the (source) UPF may report to the consumer any data collected between the time instance of the last notification to the consumer and the time instance of the UPF change or for event triggered reporting with Reporting suggestion information, when the UPF changes, the source UPF may report to the consumer any data that has been collected but not sent to the consumer yet.

NOTE:	Step 6 takes place as part of step 5. The UPF can send any collected data not yet sent to the UPF event consumer before N4 session is released.

7.	In the case of UPF relocation, to continue the subscription of the UPF event consumer, if the UPF related information is matched with the subscription request in step 1, the SMF may select a target UPF and may re-subscribe to the target UPF based on the subscription from UPF event consumer in step 1. Depending on the event, SMF may have sent the subscription to target UPF in step 5 instead.

8.	When the notification event is triggered, the UPF sends the Nupf_EventExposure_Notify service operation to the UPF event consumer, including locally collected UPF data.

In the case of PDU Session release, steps 7-8 do not apply.

When the UPF event consumer targets Any UE, the same procedure applies with the following changes:

-	In step 1 target is Any UE.

-	Step 2 and step 3 are performed for each of the UEs selected by the SMF to serve the request received in step 1.

-	Step 4 to step 8 may apply to any of the PDU sessions selected by the SMF to serve the request received in step 1.

# ************************************ END OF CHANGES ****************************************