3GPP TSG-SA5 Meeting #161	S5-252895

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025                          revision of S5-252320

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0381 | rev | 3 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.532 Add new notifications for reliability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson India Private Limited, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBMA_Ph3 |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As decribed in TR 28.871 V19.0.0 (2024-12) clause 5.13 in some cases the producer is not able to provide reliable information to the consumer and may not be able to reliably prepare and send all the notifications that the consumer requested in NTFSubscriptionControl. Reasons for this may include, overload or internal errors of the producer or limitations in generating data change notifications in case of big configuration updates. If the producer detects such problems it should notify the consumer about the onset of the problems, but even more importantly when normal, reliable operations are resumed. The latter is important as this is the point where the consumer may take corrective actions to retrieve the data lost in the notifications. 2 notifications are defined to serve this purpose. Based on TR 28.871 V19.0.0 (2024-12) clauses 5.13. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new notifications notifyDataNodeTreeUnreliable and notifyDataNodeTreeSynchRecommended |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No way to indicate unreliable notification transport caused by producer issues |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.1.x, 11.1.1.x.1, 11.1.1.x.2, 11.1.1.y, 11.1.1.y.1, 11.1.1.y.2, 12.1.1.2.1. 12.1.1.2.x, 12.1.1.2.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1769 at commit 6606fd3446cfa8e2f8ff1ebcead02a97319fa1ed |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 11.1.1.x	Notification notifyPotentialFaultyDataNodeTree

##### 11.1.1.x.1	Definition

This notification is sent to subscribed MnS consumer(s) when the information in the data node tree is potentially faulty, and that subsequently data node tree change notifications may be faulty as well. The reported problem may be caused for example by internal errors of the MnS producer or a loss of connection between the MnS producer and underlying data sources or managed entities.

The notification allows to specify the parts of the data node tree that are potentially faulty, referred to as (faulty) scope, using the parameters "objectInstance" and "scope".

When only the "objectInstance" parameter in the notification header is present, then the complete subtree, whose root data node is identified by the "objectInstance", is potentially faulty.

The "scope" parameter allows to specify more complex faulty scopes. It is expressed using the "scopeType" and "scopeLevel" parameters or the "dataNodeSelector" parameter. The "objectInstance" parameter in the notification header specifies the base object for the scope expressions. The notification shall use the method that is used in the subscription for this notification.

The reported scope should not be broader than the scope in the subscription for this notification. In other words, the reported scope shall be equal to the common subset of the data nodes included in the scope of the subscription and those included in the potentially faulty scope. Note that, when using the "scopeType" and "scopeLevel" parameters to define the scope, this may require to send multiple notifications with different scopes. For example, when the subscription scope includes a complete subnetwork with multiple base stations, and two base stations are not reachable, then the scope in one notification identifies the data node subtree representing one base station, and the scope in the second notification identifies the data node subtree representing the other base station. The expressions for selecting data nodes used with the "dataNodeSelector" parameter are so powerful that usually only one notification is required.

##### 11.1.1.x.2	Input parameters

| Parameter Name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| objectClass | M | See clause 11.0.2 | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A |
| objectInstance | M | If the "scope" parameter is absent, this parameter identifies the root object of the subtree that is potentially faulty. If the "scope" parameter is present, this parameter identifies the base object of the scope. | Type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A |
| notificationId | M | See clause 11.0.2 | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A |
| notificationType | M | "notifyPotentialFaultyDataNodeTree" | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A |
| eventTime | M | See clause 11.0.2 | Type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A |
| systemDN | M | See clause 11.0.2 | Type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A |
| sequenceNo | O | See clause 11.0.2 | Type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A |
| subscriptionId | O | See clause 11.0.2 | Type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A |
| scope | O | This parameter specifies the set of data nodes that is potentially faulty. The base object of the scope is specified by the "objectInstance" parameter. The parameter has the same structure and semantics as the "Scope" data type in TS 28.622 [11], clause 4.3.23 CHOICE_1.1 scopeType CHOICE_1.2 scopeLevel CHOICE_2.1 dataNodeSelector If the parameter is absent, the scope is the full subtree under "objectInstance". | Type: Scope multiplicity: 0..1 isOrdered: N/A isUnique: N/A |

Next change

#### 11.1.1.y	Notification notifyDataNodeTreeSynchRecommended

##### 11.1.1.y.1	Definition

This notification is sent to subscribed MnS consumer(s) if the MnS producer recommends to the MnS consumer to synchronize his copy of the data node tree on the MnS producer with the current data node tree on the MnS producer.

The notification should be sent in numerous scenarios and for various reasons:

- If the data node tree on the MnS producer becomes reliable again after it was unreliable, and a "notifyPotentialFaultyDataNodeTree" notification was sent.

- If the MnS producer realizes that the data node tree was unreliable only when it is reliable again, and no previous "notifyPotentialFaultyDataNodeTree" notification was sent.

- If, for whatever reason, not all data node tree change notifications, that should have been sent based on the changes in the data node tree, are sent, and the fault causing this erroneous behaviour is gone.

- If the connection from the MnS producer to a MnS consumer was down and the MnS producer determines it is up again.

The notification allows to specify the parts of the data node tree, that should be synchronized, using the parameters "objectInstance" and "scope". The usage of these parameters is defined in clause 11.1.1.x.1.

If the "notifyPotentialFaultyDataNodeTree" notification is supported, the "notifyDataNodeTreeSynchRecommended" notification shall be supported as well. Furthermore, if a "notifyPotentialFaultyDataNodeTree" is sent to a subscribed consumer, a subsequent "notifyDataNodeTreeSynchRecommended" notifications shall be sent to the consumer, when the data node tree is reliable again.

##### 11.1.1.y.2	Input parameters

| Parameter Name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| objectClass | M | See clause 11.0.2 | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A |
| objectInstance | M | If the "scope" parameter is absent, this parameter identifies the root object of the subtree that should be synchronized. If the "scope" parameter is present, this parameter identifies the base object of the scope. | Type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A |
| notificationId | M | See clause 11.0.2 | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A |
| notificationType | M | "notifyPotentialFaultyDataNodeTree" | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A |
| eventTime | M | See clause 11.0.2 | Type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A |
| systemDN | M | See clause 11.0.2 | Type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A |
| sequenceNo | O | See clause 11.0.2 | Type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A |
| subscriptionId | O | See clause 11.0.2 | Type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A |
| scope | O | This parameter specifies the set of data nodes that should be synchronized. The base object of the scope is specified by the "objectInstance" parameter. The parameter has the same structure and semantics as the "Scope" data type in TS 28.622 [11], clause 4.3.23 CHOICE_1.1 scopeType CHOICE_1.2 scopeLevel CHOICE_2.1 dataNodeSelector If the parameter is absent, the scope is the full subtree under "objectInstance". | Type: Scope multiplicity: 0..1 isOrdered: N/A isUnique: N/A |

Next change

##### 12.1.1.2.1	Introduction

The IS notifications are mapped to SS equivalents according to table 12.1.1.2.1-1.

Table 12.1.1.2.1-1: Mapping of IS notifications to SS equivalents

| IS notification | HTTP Method | Resource URI | S |
| --- | --- | --- | --- |
| notifyMOICreation | POST | {notificationTarget} | M |
| notifyMOIDeletion | POST | {notificationTarget} | M |
| notifyMOIAttributeValueChanges | POST | {notificationTarget} | M |
| notifyMOIChanges | POST | {notificationTarget} | M |
| notifyEvent | POST | {notificationTarget} | M |
| notifyPotentialFaultyDataNodeTree | POST | {notificationTarget} | M |
| notifyDataNodeTreeSynchRecommended | POST | {notificationTarget} | M |

Next change

##### 12.1.1.2.x	Notification notifyPotentialFaultyDataNodeTree

Principles:

- 	Only information not documented in the OpenAPI files is included in this clause.

- 	The following items are documented in the OpenAPI files: HTTP-Method, parameter name and type.

- 	The name of each parameter is the same in the stage 2 information model (clause 11.1) and in the stage 3 OpenAPI definition. Exceptions, if any, are listed below.

Table 12.1.1.2.x-1: Mapping of IS notification input parameters to SS equivalents (HTTP POST)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type |
| --- | --- | --- | --- |
| objectClass | request body | href | Uri |
| objectInstance |  |  |  |

##### 12.1.1.2.y	Notification notifyDataNodeTreeSynchRecommended

Principles:

- 	Only information not documented in the OpenAPI files is included in this clause.

- 	The following items are documented in the OpenAPI files: HTTP-Method, parameter name and type.

- 	The name of each parameter is the same in the stage 2 information model (clause 11.1) and in the stage 3 OpenAPI definition. Exceptions, if any, are listed below.

Table 12.1.1.2.y-1: Mapping of IS notification input parameters to SS equivalents (HTTP POST)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type |
| --- | --- | --- | --- |
| objectClass | request body | href | Uri |
| objectInstance |  |  |  |

Next change

*** START OF CHANGE 1 ***

*** OpenAPI/TS28532_ProvMnS.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: Provisioning MnS

version: 19.0.0

description: >-

OAS 3.0.1 definition of the Provisioning MnS

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.532; Generic management services

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.532/

servers:

- url: '{MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}'

variables:

MnSRoot:

description: See clause 4.4.2 of TS 32.158

default: http://example.com/3GPPManagement

MnSVersion:

description: Version number of the OpenAPI definition

default: XXX

URI-LDN-first-part:

description: See clause 4.4.2 of TS 32.158

default: ''

paths:

'/{className}={id}':

parameters:

- name: className

in: path

required: true

schema:

type: string

- name: id

in: path

required: true

schema:

type: string

put:

summary: Replaces a complete single resource or creates it if it does not exist

description: >-

With HTTP PUT a complete resource is replaced or created if it does not

exist. The target resource is identified by the target URI.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/Resource'

responses:

'200':

description: >-

Success case ("200 OK").

This status code shall be returned when the resource is replaced, and

when the replaced resource representation is not identical to the resource

representation in the request.

This status code may be returned when the resource is updated and when the

updated resource representation is identical to the resource representation

in the request.

The representation of the updated resource is returned in the response

message body.

content:

application/json:

schema:

$ref: '#/components/schemas/Resource'

'201':

description: >-

Success case ("201 Created").

This status code shall be returned when the resource is created.

The representation of the created resource is returned in the response

message body.

content:

application/json:

schema:

$ref: '#/components/schemas/Resource'

'204':

description: >-

Success case ("204 No Content").

This status code may be returned only when the replaced resource

representation is identical to the representation in the request.

The response has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponseDefault'

callbacks:

notifyMOICreation:

'{request.body#/notificationRecipientAddress}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyMoiCreation'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response

has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyMOIDeletion:

'{request.body#/notificationRecipientAddress}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyMoiDeletion'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response

has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyMOIAttributeValueChanges:

'{request.body#/notificationRecipientAddress}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyMoiAttributeValueChanges'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response

has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyEvent:

'{request.body#/notificationRecipientAddress}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyEvent'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response

has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyMOIChanges:

'{request.body#/notificationRecipientAddress}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyMoiChanges'

application/yang-data+json:

schema:

$ref: '#/components/schemas/NotifyMoiChanges'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response

has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyPotentialFaultyDataNodeTree:

'{request.body#/notificationRecipientAddress}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyPotentialFaultyDataNodeTree'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response

has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyDataNodeTreeSyncRecommended:

'{request.body#/notificationRecipientAddress}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyDataNodeTreeSyncRecommended'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response

has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

get:

summary: Reads one or multiple resources

description: >-

With HTTP GET resources are read. The resources to be retrieved are

identified with the target URI. The attributes and fields parameter

of the query components allow to select the resource properties to be returned.

parameters:

- name: scope

in: query

description: >-

This parameter extends the set of targeted resources beyond the base

resource identified with the path component of the URI.

required: false

content:

application/json:

schema:

$ref: '#/components/schemas/Scope'

- name: filter

in: query

description: >-

This parameter reduces the targeted set of resources by applying a

filter to the scoped set of resource representations. Only resource

representations for which the filter construct evaluates to "true"

are targeted.

required: false

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/JexConditions'

- name: attributes

in: query

description: >-

This parameter specifies the attributes of the scoped resources that

are returned.

required: false

schema:

type: array

items:

type: string

style: form

explode: false

- name: fields

in: query

description: >-

This parameter specifies the attribute field of the scoped resources

that are returned.

required: false

schema:

type: array

items:

type: string

style: form

explode: false

- name: dataNodeSelector

in: query

description: >-

This parameter contains an expression allowing to conditionally

select data nodes.

required: false

schema:

oneOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/JexNodeSelectionBasic'

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/JexNodeSelectionAdvanced'

responses:

'200':

description: >-

Success case ("200 OK").

The resources identified in the request for retrieval are returned

in the response message body. In case the attributes or fields query

parameters are used, only the selected attributes or sub-attributes are

returned. The response message body is constructed according to the

hierarchical response construction method (TS 32.158 [15]).

content:

application/json:

schema:

$ref: '#/components/schemas/Resource'

application/vnd.3gpp.object-tree-hierarchical+json:

schema:

$ref: '#/components/schemas/Resource'

application/vnd.3gpp.object-tree-flat+json:

schema:

type: array

items:

$ref: '#/components/schemas/Resource'

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponseGet'

patch:

summary: Patches one or multiple resources

description: >-

With HTTP PATCH resources are created, updated or deleted. The resources

to be modified are identified with the target URI (base resource) and

the patch document included in the request message body.

requestBody:

description: >-

The request body describes changes to be made to the target resources.

The following patch media types are available

- "application/merge-patch+json" (RFC 7396)

- "application/3gpp-merge-patch+json" (TS 32.158)

- "application/json-patch+json" (RFC 6902)

- "application/3gpp-json-patch+json" (TS 32.158)

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/Resource'

application/3gpp-merge-patch+json:

schema:

$ref: '#/components/schemas/Resource'

application/json-patch+json:

schema:

type: array

items:

$ref: '#/components/schemas/PatchItem'

application/3gpp-json-patch+json:

schema:

type: array

items:

$ref: '#/components/schemas/PatchItem'

responses:

'200':

description: >-

Success case ("200 OK").

This status code is returned when the updated the resource representations

shall be returned for some reason.

The resource representations are returned in the response message body. The

response message body is constructed according to the hierarchical response

construction method (TS 32.158 [15])

content:

application/json:

schema:

$ref: '#/components/schemas/Resource'

'204':

description: >-

Success case ("204 No Content").

This status code is returned when there is no need to return the updated

resource representations.

The response message body is empty.

default:

description: Error case.

content:

application/json:

schema:

oneOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponseDefault'

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponsePatch'

delete:

summary: Deletes one resource

description: >-

With HTTP DELETE one resource is deleted. The resources to be deleted is

identified with the target URI.

responses:

'200':

description: >-

Success case ("200 OK").

This status code is returned, when the resource has been successfully deleted.

The response body is empty.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponseDefault'

components:

schemas:

CmNotificationTypes:

type: string

enum:

- notifyMOICreation

- notifyMOIDeletion

- notifyMOIAttributeValueChanges

- notifyEvent

- notifyMOIChanges

- notifyPotentialFaultyDataNodeTree

- notifyDataNodeTreeSyncRecommended

SourceIndicator:

type: string

enum:

- RESOURCE_OPERATION

- MANAGEMENT_OPERATION

- SON_OPERATION

- UNKNOWN

ScopeType:

type: string

enum:

- BASE_ONLY

- BASE_NTH_LEVEL

- BASE_SUBTREE

- BASE_ALL

Operation:

type: string

enum:

- add

- remove

- replace

Insert:

type: string

enum:

- before

- after

PatchOperation:

type: string

enum:

- add

- replace

- remove

- copy

- move

- test

Resource:

oneOf:

- type: object

properties:

id:

type: string

objectClass:

type: string

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

attributes:

type: object

additionalProperties:

type: array

items:

type: object

required:

- id

- $ref: 'TS28623_FeatureNrm.yaml#/components/schemas/resources-feature'

Scope:

type: object

properties:

scopeType:

$ref: '#/components/schemas/ScopeType'

scopeLevel:

type: integer

CorrelatedNotification:

type: object

properties:

source:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

notificationIds:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationId'

required:

- source

- notificationIds

MoiChange:

type: object

properties:

notificationId:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationId'

correlatedNotifications:

type: array

items:

$ref: '#/components/schemas/CorrelatedNotification'

additionalText:

type: string

sourceIndicator:

$ref: '#/components/schemas/SourceIndicator'

op:

$ref: '#/components/schemas/Operation'

path:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

insert:

$ref: '#/components/schemas/Insert'

value: {}

oldValue: {}

required:

- notificationId

- op

- path

NotifyMoiCreation:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

correlatedNotifications:

type: array

items:

$ref: '#/components/schemas/CorrelatedNotification'

additionalText:

type: string

sourceIndicator:

$ref: '#/components/schemas/SourceIndicator'

attributeList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

NotifyMoiDeletion:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

correlatedNotifications:

type: array

items:

$ref: '#/components/schemas/CorrelatedNotification'

additionalText:

type: string

sourceIndicator:

$ref: '#/components/schemas/SourceIndicator'

attributeList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

NotifyMoiAttributeValueChanges:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

correlatedNotifications:

type: array

items:

$ref: '#/components/schemas/CorrelatedNotification'

additionalText:

type: string

sourceIndicator:

$ref: '#/components/schemas/SourceIndicator'

attributeListValueChanges:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeValueChangeSet'

required:

- attributeListValueChanges

NotifyEvent:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- specificProblem

properties:

specificProblem:

$ref: 'TS28111_FaultNrm.yaml#/components/schemas/SpecificProblem'

additionalText:

type: string

additionalInformation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

NotifyMoiChanges:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

moiChanges:

type: array

items:

$ref: '#/components/schemas/MoiChange'

required:

- moiChanges

NotifyPotentialFaultyDataNodeTree:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

scope:

$ref: 'TS28623_SubscriptionControlNrm.yaml#/components/schemas/Scope'

NotifyDataNodeTreeSyncRecommended:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

scope:

$ref: 'TS28623_SubscriptionControlNrm.yaml#/components/schemas/Scope'

PatchItem:

type: object

properties:

op:

$ref: '#/components/schemas/PatchOperation'

from:

type: string

path:

type: string

value:

nullable: true

required:

- op

- path

<CODE ENDS>

*** END OF CHANGE 1 ***