3GPP TSG-SA WG6 Meeting #66	S6-251047

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-250049)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0361 | rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of response elements in the 5GSAT procedure in TS23.434 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC, NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_APP |  |  |  |  |  | Date: |  |  | 2025-3-28 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This contribution adds details about the response elements in the 5GSAT procedure. While some other sections (such as clause 14.3.2) include Cause in the response, NTN-related sections do not. So, this information was added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When there is a failure in the response, the cause is being appended. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The response elements are inadequate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 21.2.3.2, 21.3.3.2, 21.3.3.5, 21.3.3.7, 21.3.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The present CR was agreed during SA6#65 but was by (MCC) mistake excluded from SA#107 CR pack submission. The CR is hence formally re-presented for SA6#66 as a revision (with no actual change compared with CR0361 in S6-250049). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 21.2.3.2	Satellite coverage availability information configuration response

Table 21.2.3.2-1 describes the information flow from the configuration management client to the configuration management server for the response of the satellite coverage availability information configuration request.

Table 21.2.3.2-1: Satellite coverage availability information configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL user or VAL UE. |
| Result | M | Indicates the request result. |
| Cause | O | The cause for the request failure. |

* * * Next Change * * * *

#### 21.3.3.2	Satellite S&F events subscription response

Table 21.3.3.2-1 describes the information flow from NRM server to the VAL server or from NRM client to NRM server for response of satellite S&F events subscription request.

Table 21.3.3.2-1: Satellite S&F events subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL users or VAL UEs or NRM server. |
| Subscription status | M | It indicates the subscription result |
| Cause | O | The cause for the request failure. |

* * * Next Change * * * *

#### 21.3.3.5	Satellite S&F events unsubscribe response

Table 21.3.3.5-1 describes the information flow from NRM server to the VAL server or from NRM client to NRM server for response of satellite S&F events unsubscribe request.

Table 21.3.3.5-1: Satellite S&F events unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Request status | M | It indicates the unsubscribe request result. |
| Cause | O | The cause for the request failure. |

* * * Next Change * * * *

#### 21.3.3.7	Report UE satellite information response

Table 21.3.3.7-1 describes the information flow the NRM server to the NRM client for the response of reporting the UE satellite related information request.

Table 21.3.3.7-1: Report UE satellite information response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | It indicated the request result. |
| Cause | O | The cause for the request failure. |

* * * Next Change * * * *

#### 21.3.3.9	Satellite S&F configuration response

Table 21.3.3.9-1 describes the information flow from the NRM client to the NRM server for the response of satellite S&F configuration request.

Table 21.3.3.9-1: Satellite S&F events subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL users or VAL UEs or NRM server. |
| Result | M | It indicates the request result |
| Cause | O | The cause for the request failure. |

* * * End of Changes * * * *