3GPP TSG-SA WG6 Meeting #66	S6-251470

Goteborg, Sweden, 7th – 11th April 2025	(revision of S6-251451)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0382 | rev | 3 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NRM enabled S&F transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2025-04-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As specified in TS 22.865, Annex A, the Store and Forward satellite operation mode “is intended to provide some level of communication service for UEs under satellite coverage with intermittent/temporary satellite connectivity (e.g. when the satellite is not connected via a feeder link or via ISL to the ground network) for delay-tolerant communication service. (…) (…)under “S&F Satellite operation” mode, the end-to-end exchange of signalling/data traffic is now handled as a combination of two steps not concurrent in time (Step A and B in Figure A-1). In Step A, signalling/data exchange between the UE and the satellite takes place, without the satellite being simultaneously connected to the ground network (i.e. the satellite is able to operate the service link without an active feeder link connection). In Step B, connectivity between the satellite and the ground network is established so that communication between the satellite and the ground network can take place. So, the satellite moves from being connected to the UE in step A to being connected to the ground network in step B”. However, in the currently specified solution it is not considered that the feeder and the service link are not concurrent in time, and that, as specified in TS 23.401, some network functionality is needed onboard the satellite to enable the forwarding of signalling between the NRM client and the NRM server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce a new S&F function in the satellite. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VAL cannot use NRM to control data delivery when the UEs are in S&F Satellite operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 21.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************************* First Change *****************************/

### 21.3.1	General

This following sub-clauses shows how the application enablers provisioning the satellite S&F configuration to the VAL UE and expose the satellite S&F events information to the VAL server to help minimize the service interruption (e.g., adjust the DL frequency, etc.) when the VAL UE starts to perform the satellite S&F operation.

NOTE 1:	In S&F Satellite operation, the end-to-end exchange of traffic is handled in a sequence of steps reflecting the intermittent availability of the service link (when the satellite can exchange data with the NRM client) and the intermittent availability of the feeder (ground) link (when the satellite can exchange data with the ground network (NRM server)). To support S&F satellite operation, network functionality needs to be deployed on the satellite, as specified in 3GPP TS 23.401 [9].

NOTE 2:	The network functionality deployed on the satellite enables that the NRM server can also send/receive request/responses when the feeder link is available and that the NRM client can also send/receive request/responses when the service link is available.

# /******************************* End Change *****************************/