3GPP TSG-SA WG6 Meeting #67	S6-252533

Fukuoka, Japan, 19th – 23rd May 2025	(revision of S6-252465)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0394 | rev | 2 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct the reference of Satellite coverage availability information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT,Ericsson, Nokia, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the feedback from CT3 and CT1, the definition of satellite coverage availability information is not clear in the current stage-2 specifications. In order to make it clear how to use it in SA6, it’s needed to define the contents of Satellite coverage availability information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rename and define the Satellite coverage availability information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The functions related to SCAI will not be performed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 21.2.3.1, 21.2.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

21.2.3.1	Application satellite coverage availability information (ASCAI) configuration request

Table 21.2.3.1-1 describes the information flow from the configuration management server to the configuration management client to configure the ASCAI.

Table 21.2.3.1-1: Application satellite coverage availability information configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL user or VAL UE. |
| VAL service ID | O | Identity of the VAL service that is requested. |
| ASCAI | M | Defines the application satellite coverage information including a list of geographical areas, corresponding available time periods and satellite RAT types. |
| > Geographical area | M | It includes a list of geographical area information (e.g. coordinates, as defined in 3GPP TS 29.572 [51]) where the satellite coverage is available. |
| > Availability time period | M | It provides the time periods when the satellite coverage is available over the indicated geographical area. |
| > Availability satellite RAT types | O | It provides the satellite RAT types information corresponding to the satellite availability in the indicated geographical area. |

* * * Next Change * * * *

21.2.3.4	Get application satellite coverage availability information (ASCAI) response

Table 21.2.3.4-1 describes the information flow from the configuration management server to the configuration management client or VAL server to notify the ASCAI.

Table 21.2.3.4-1: Get application satellite coverage availability information response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL user or VAL UE. |
| VAL service ID | O | Identity of the VAL service that is requested. |
| ASCAI | M | Defines the application satellite coverage information including a list of geographical areas, corresponding available time periods and satellite RAT types. |
| > Geographical area | M | It includes a list of geographical area information (e.g. coordinates, as defined in 3GPP TS 29.572 [51]) where the satellite coverage is available. |
| > Availability time period | M | It provides the time periods when the satellite coverage is available over the indicated geographical area. |
| > Availability satellite RAT types | O | It provides the satellite RAT types information corresponding to the satellite availability in the indicated geographical area. |

* * * Next Change * * * *

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GS	5G System

5GVN	5G Virtual Network

CAPIF	Common API Framework for northbound APIs

CNC	Centralized Network Configuration

CRUDN	Create, Retrieve, Update, Delete and Notify

EPC	Evolved Packet Core

GPSI	Generic Public Subscription Identifier

LWP	Light-weight Protocol

NEF	Network Exposure Function

NR	New Radio

PCC	Policy and Charging Control

SCEF	Service Capability Exposure Function

SEAL	Service Enabler Architecture Layer for Verticals

SEAL-UU	Service Enabler Architecture Layer for Verticals, Universal UE-network interface

TSC	Time Sensitive Communication

TSN	Time Sensitive Networking

VAL	Vertical Application Layer

NOP	Network Operator

NSaaS	Network Slice as a Service

ASCAI	Application Satellite Coverage Availability Information

* * * End of Changes * * * *