3GPP TSG-SA WG6 Meeting #66	S6-251452

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-251196, S6-251411)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0738 | rev | 2 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Edge Enabler Using Satellite Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2025-3-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some description is needed to clarify the EES profile, Service provisioning request for EDN deployment with gNB on satellite. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description to clarify the EES profile, Service provisioning request for EDN deployment with gNB on satellite. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear about the EES profile, Service provisioning request for EDN deployment with gNB on satellite |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 11	Edge Enabler Using Satellite Access

## 11.1	General

This clause provides clarifications on the procedures and information flows necessary for enabling edge application using satellite access.

### 11.1.1	EES determination based on UE serving satellite ID Procedure

This procedure is applicable for EDN deployment with gNB on satellite.

A satellite EDN may be deployed as described in Annex A.2.5. The service provisioning procedure for satellite EDN is the same as clause 8.3.3.2.2 and clause 8.3.3.2.3 with the following clarifications.

When UE connects to the satellite, the service provisioning request message contains the UE serving Satellite ID if available (i.e., serving gNB corresponding Satellite ID, if received). If the service provisioning request message contains the UE serving Satellite ID, then the ECS determines the EES where the Satellite ID in the EES profile matches the UE serving Satellite ID.

When UE connects to the satellite, and the service provisioning request message does not contain the UE serving Satellite ID, then service provisioning response message may contain the list of EES along with the EES Satellite ID, and then the EEC determine EES where the Satellite ID in the EES profile matches the UE serving Satellite ID, if available in the UE.

### 11.1.2	EES determination based on UE location and route Procedure

This procedure and the corsponding EES profile and Service provisioning request are applicable for EDN deployment with gNB on satellite.

A satellite EDN may be deployed as described in Annex A.2.5. The service provisioning procedure for satellite EDN is the same as clause 8.3.3.2.2 and clause 8.3.3.2.3 with the following clarifications.When EEC sends the service request message to the ECS and the request message contains the UE location, predicted/expected UE location, Prediction expiration time, then the ECS determines a list of EES where the Satellite coverage availability information in EES profile matches the UE location, predicted/expected UE location, Prediction expiration time, and EES Satellite coverage availability information. Alternatively, the ECS may query external server with the UE current location and UE predicted/expected location to obtain a list of Satellite IDs and Trajectory IDs corresponding the UE location and the UE trajectory, then the ECS determines list of EES based on the Satellite IDs and Trajectory IDs in EES profiles by matching with the Satellite IDs and Trajectory IDs obtained from external server for the UE. The “Expected AC Geographical Service Area” in the AC profile, as specified in clause 8.2.2, is used by the ECS to request the external server (managed by the satellite service provider or operator) for such satellite IDs and trajectory IDs. When ECS sends the service provisioning response which contains a list of EES information which EES is deployed on MEO or LEO satellites, the EEC determines the EES where the Satellite ID in the EES profile matches the UE serving Satellite ID.

In addition, when a serving satellite is providing EES and EAS, and due to discontinuous coverage the same satellite comes back for serving, then the EEC connects to same EES and EAS (i.e. to which the EEC had previously connected to), without needing to discover EES and EAS endpoints again.

Editor's Note: Whether and how the ACR procedure need to be enhanced is FFS.