3GPP TSG-SA3 Meeting #122	S3-252348

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025										was S3-251688

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.180 | CR | 0220 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [33.180] Alignment of Logging Recording and Audit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Airbus |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCXSec4 |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Alignment of SA3 and SA6 specifications regarding logging, recording and audit. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Architectural and terminology change |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be a mismatch between SA3 and SA6 specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.1, 10.1.2, 10.2.1, 10.2.2, 10.2.3, 10.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*********************** START of 1st change *****************************

# 10	Logging, Recording, Audit and Discreet Monitoring

## 10.1	Logging and audit of service metadata

### 10.1.1	Overview

The MC system generates service metadata. The full range of user events that may be logged by the MC system is out-of-scope of the current specification

This clause defines the security and communications-related data associated to user events that are required to enable the audit of MC user actions within the MC system. Logging of user events is required to support discreet monitoring and audit.

To ensure the privacy of MC users’ data, where this information is collected it shall be protected as defined in Clause 10.1.2.4.

### 10.1.2	User events

#### 10.1.2.1	Void

#### 10.1.2.2	Void

#### 10.1.2.3	Security content within user event logs

When a user event occurs in a log, the following security-related information is required:

-	IP addresses (source and destination)

-	Signalling layer identifiers

-	SIP URI (source and destination).

-	HTTP target URL

-	The initiating user or server

-	The receiving user, group, server or [set of multicast users]

-	Security parameters (if present in signalling):

-	MIKEY message

-	Access or Security Token

-	Identifiers for related media bearers (if applicable).

NOTE:	These logs are required to support discreet monitoring or audit of user media.

#### 10.1.2.4	Protection of user event logs

User event logs need to be protected as they contain information that impacts the user's privacy. User event logs shall be encrypted and integrity protected while stored. Access to user event logs shall only be granted to authorised users and such access shall be logged.

## 10.2	Audit and Discreet Monitoring of Service Media

### 10.2.1	Overview

Discreet Monitoring is access to service media at a network element within the MC Domain. Where Discreet Monitoring is used to access to user voice communications, it is known as Discreet Listening. Discreet Monitoring includes access to voice, video and data communications. For the purposes of this document, discreet monitoring and audit are equivalent processes. For discreet monitoring the access to media is in real-time. For audit, the access to media is at some point after the recording.

Disceet Monitoring and Audit are required functions of a public-safety network. For non-public safety services, these functions shall not be implemented in the network without explicit consent from all users of the MC system

### 10.2.2	Collection of service media

The storage solution is out-of-scope of this document.

Service media is collected from the media paths within the MC Domain. It is expected that the encrypted media required to be collected (MCPTT, MCData, MCVideo or MC Location) shall be routed from the appropriate MC Service Server to the MC Recording server.

Where SDS messages are routed within a signalling path, media will need to be extracted from within MCData signalling paths by the MCData Server. It is expected that the encrypted media routed over the signalling path shall be routed from the MCData Server to the MC Recording server.

The storage solution is out-of-scope of this document.

### 10.2.3	Storing of service media

Service media in the MC System is end-to-end encrypted by default. Consequently, media can be recorded without modification and without additional protection by the MC Recording server.

NOTE a:	It’s up to the implementation of the MC Recording server to decrypt and re-encrypt before storing it to mass data storage.

### 10.2.4	Decryption of service media

To decrypt a specific target user's service media for audit or discreet listeningat a specific time 'T', the following process should be used for decryption of user media.  The controlling entity shall be either the KMS or a secured logging device.

1.	The auditor obtains the target user's key material and KMS certificate that was active at time 'T'. This could be performed, for example, using an Audit Client. An Audit Client is a Key Management Client with the special access privilege to request previously-issued user key material for existing clients. The provision of user key material by the KMS grants the auditor access to the user’s communication content.

a.	Any request made by the auditor shall be controlled and logged (to allow the audit action to be audited).

b.	It is recommended that each release of key material be authorised by a secondary auditor (e.g. a double-lock mechanism).

c.	It is recommended that the number of requests from each auditor within a time-period be limited. It is also recommended that the total number of requests from all auditors within a time-period be limited.

NOTE:	The release of key material for a user at time 'T' only allows the auditor access to content for the defined 'key period' associated to time ‘T’. By limiting the total number of requests (e.g. to 0.1% of users), this limits the auditor’s access to communications. These controls help to ensure that the granted access to user content is time-limited and proportionate.

2.	The auditor extracts the user events associated with the user at time 'T'.

3.	The auditor extracts the MIKEY messages from the signalling events and use the audited user's KMS-supplied key material to decrypt the media encryption keys held within the MIKEY messages.

4.	The auditor is now able to associate media with user events and use the media encryption keys extracted from MIKEY message to decrypt the media.

*********************** END of 1st change *****************************