3GPP TSG-SA3 Meeting #122																			S3-252312

Fukuoka, Japan, 19 – 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.700-22 | CR | 0002 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous updates in introductionary part |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_CAPIF_Ph3-sec |  |  |  |  |  | Date: |  |  | 2025-05-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Update of scope needed, removal of 3GPP in references, typos |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated scope and corrections in introductionalry part and key issue part |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete scope, no alignment with conventions |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 3.1, 3.3, 4.2, 5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3-252239 |  |  |  |  |  |  |  |  |

********* START OF CHANGES

# 1	Scope

The present document investigates the security and privacy impacts of the CAPIF procedures and covers the following:

-	Resource owner authorization management

-	CAPIF interconnection security

-	Authorizing API invoker on one UE accessing resources related to another UE

-	Nested API invocation

-	Authentication and authorization of multiple API invokers

-	Onboarding aspects

Note: 	void.

********* NEXT CHANGE

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Resource owner authorization: The permission by the resource owner to allow the API invoker to access the resource owner's resource via the northbound API.

********* NEXT CHANGE

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

********* NEXT CHANGE

## 4.2	High-level architecture for CAPIF interconnection

Figure 4.2-1 in the present document (see also TS 23.222 [2]) shows the architectural model for the CAPIF interconnection which allows API invokers of a CAPIF provider to utilize the service APIs from the 3rd party CAPIF provider.

The API invoker within the trust domain of CAPIF provider A onboards in CCF of CAPIF provider A.

The API invoker within the trust domain of CAPIF provider A interacts with the CAPIF core function of the CAPIF provider A via CAPIF-1, discovers the service APIs of both CAPIF providers, and invokes the service APIs in the trust domain of CAPIF provider A via CAPIF-2 and invokes the service APIs in the trust domain of CAPIF provider B via CAPIF-2e.

Figure 4.2-1: High level functional architecture for CAPIF interconnection with multiple CAPIF provider domains

Figure 4.2-2 in the present document (see also TS 23.222 [2]) shows the architectural model for the CAPIF interconnection within the same CAPIF provider domain, which allows API invokers of CAPIF core function 1 to utilize the service APIs from CAPIF core function 2, where both CAPIF core function 1 and CAPIF core function 2 are hosted within the trust domain of the CAPIF provider A.

The API invokers of CAPIF core function 1 indicates that API invoker onboards in CAPIF core function 1.

Figure 4.2-2: High level functional architecture for CAPIF interconnection within a CAPIF provider domain

************ NEXT CHANGE

### 5.3.1	Key issue details

This key issue addresses the security aspects of TR 23.700-22 [3] KI #6.

It studies the security aspects for the case that API invoker(s) are deployed on one UE and requests to access resources (hosted in the network) related to another UE (e.g., application client on UE is fetching location of another UE or setting QoS for PDU sessions of another UE).

As specified in TS 23.222 [2], the API invoker may be deployed in any of the following ways:

a.	API invoker may be deployed as AF on the UE (i.e. 3rd party application).

b.	API invoker may be deployed as AF on the UE supporting several other 3rd party applications deployed on the UE.

c.	API invoker may be deployed on the network as AF.

So far, only a UE accessing its own resources is considered if the API invoker is on a UE. Resource owner-aware northbound API access (RNAA) defined in TS 33.122 [4] only supports authorizing API invoker on one UE to request resources related to the same UE.

Therefore, it is proposed to study how to authorize an API invoker on one UE to access resources related to another UE.

The use case in Annex A.2 of TR 23.700-95 [13] is UE-originated API invocation (Location tracking) where the application running on a UE initiates the procedure and the resource owner can be not using the application at the API invocation time. In CAPIF RNAA, only client credential and authorization code flow (w/o PKCE) are supported. In the client credential flow, it is assumed that the resource owner authorization information is available. In the authorization code flow, it is assumed that the client (API invoker) can access to the user agent of the resource owner and direct the resource owner to the authorization server.

********* END OF CHANGES