3GPP TSG-SA3 Meeting #122																			S3-252244

Fukuoka, Japan, 19 – 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.700-22 | CR | 0007 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solution 19 figure update and formatting issues |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_CAPIF_Ph3-sec |  |  |  |  |  | Date: |  |  | 2025-05-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Figure update needed. formatting needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implementing as provided in reason for changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Readibility issue in text in figure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.19.2, 6.19.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********* START OF CHANGES

### 6.19.2	Solution details

Pre-conditions:

API invoker's onboarding is completed in CCF-A

The CCF-B is connected with the AEF

API invoker's onboarding is completed in CCF-A

The CCF-B is connected with the AEF

Solution and information flow:

The authorization related request in this solution can be the enhancement of OAuth 2.0 based access token request defined in clause 6.5.2.3 of TS 33.122[4].

The authorization related response in this solution can be the enhancement of access token response defined in clause 6.5.2.3 of TS 33.122[4].

This solution reuses the service API discovery procedure defined in clause 8.25.3.3 of TS 23.222 [2].

Figure 6.19.2-1: API invoker authorization mechanism in CAPIF interconnection scenarios

0.	API invoker and CCF-A have established the TLS tunnel and completed the mutual authentication.

1.	API invoker sends the service API discovery related request to the CCF-A.

2.	CCF-A determines to send the service API discovery request to CCF-B based on local policy (e.g., A specific category of API needs to send the request to CCF-B).

3.	CCF-B sends the discovered service API information to the CCF-A.

4.	CCF-A sends the discovered service API information to the API invoker ID.

5.	API invoker sends the authorization related request to the CCF-A. The request also includes the expected resource owner ID, expected service/service operation/service API, date type, and the corresponding data processing purpose.

6.	To check if the authorization related request should be sent to the CCF-B, the following rules apply.

-	If the CCF-A finds that the expected service/service operation/service API cannot be provided by AEF in its domain and finds that the expected service/service operation matches the discovered service API from CCA-B received in step 3, the CCF-A sends the authorization related request to the CCF-B.

-	If the CCF-A finds that the expected service/service operation/service API is previously published by CCF-B, the CCF-A sends the authorization related request to the CCF-B.

-	If the CCF-A finds that the expected service/service operation/service API cannot be provided by AEF in its domain and finds that the expected service/service operation matches the discovered service API from CCA-B received in step 3, the CCF-A sends the authorization related request to the CCF-B.

-	If the CCF-A finds that the expected service/service operation/service API is previously published by CCF-B, the CCF-A sends the authorization related request to the CCF-B.

The CCF-A may use the date type and the corresponding data processing purpose in addition to the expected service/service operation to identify the matched service API. Then the CCF-A can determine how to handle the authorization related request.

The request sent to CCF-B also includes the API invoker ID, expected resource owner ID, expected service/service API/service operation, date type, and the corresponding data processing purpose. Based on the trusted business relationship between CCF-A and CCF-B, the CCF-B trusts that the API invoker indicated by the API invoker ID is already authenticated and onboarded in the CCF-A.

7.	If API invoker is authorized, CCF-B sends the token to the CCF-A. The token includes the API invoker ID, expected resource owner ID, expected service/service operation, date type, and the corresponding data processing purpose.

8.	The CCF-A sends the token to the API invoker.

9.	The API invoker authenticates to the AEF by establishing a TLS session with the AEF.

10.	With successful authentication to the AEF, the API invoker shall initiate invocation of a 3GPP northbound API with the AEF by sending service operation request with the requested date type, and the corresponding data processing purpose. The access token received from the CCF-B is sent along with the northbound API invocation request as per OAuth 2.0.

11.	The AEF validates the access token using CCF-B certificate or a key pre-shared with CCF-B.

12.	After successful verification of the access token of the API invoker, the requested northbound API is invoked and the appropriate response shall be returned to the API invoker.

********* NEXT CHANGE

### 6.19.3	Evaluation

This solution addresses part of KI#2 (i.e. The CAPIF should support authorization of the API invoker in CAPIF interconnection scenarios).

The benefit of this solution is that it realizes API invoker authorization via enhancing existing client credentials flow.

Impacts to CCF-A:

If the CCF-A finds that the expected service/service operation/service API cannot be provided by AEF in its domain and finds that the expected service/service operation matches the discovered service API from CCA-B, the CCF-A sends the authorization related request to the CCF-B.

If the CCF-A finds that the expected service/service operation/service API is previously published by CCF-B, the CCF-A sends the authorization related request to the CCF-B.

If the CCF-A finds that the expected service/service operation/service API cannot be provided by AEF in its domain and finds that the expected service/service operation matches the discovered service API from CCA-B, the CCF-A sends the authorization related request to the CCF-B.

If the CCF-A finds that the expected service/service operation/service API is previously published by CCF-B, the CCF-A sends the authorization related request to the CCF-B.

Impacts to CCF-B:

CCF-B is required to receive authorization request from the CCF-A.

CCF-B is required to send the token to the API invoker via the CCF-A.

CCF-B is required to receive authorization request from the CCF-A.

CCF-B is required to send the token to the API invoker via the CCF-A.

********* END OF CHANGES