3GPP TSG-SA #108	SP-250808

Prague, Czech Republic, 10-13 June 2025	(was SP-250784)

Source:	Qualcomm Incorporated

Title:	Way Forward on AIoT device credentials storage

Document for:	Discussion/Approval

Agenda Item:	3.3

Work Item / Release:	Ambient_IoT-SEC/ Rel-19

Abstract of the contribution: Proposes a way forward for AIoT device credentials storage.

# 1.	Proposal

It is proposed that SA#108 endorses the following guidance for AIoT device credentials storage:

1) 	when the AIoT system is defined as private network e.g. SNPN, the AIoT device credentials storage follows 3GPP defined requirements, the exact mechanism is out of scope of 3GPP (similar to Annex I.2.2 of TS 33.501). This bullet means that no interconnection exists between AIoT systems and PLMNs ;

2) 	when the AIoT system is defined as public network i.e. PLMN, the AIoT device credentials storage shall use UICC.

NOTE 0: Revisiting the above requirements in bullet 1 and/or 2 can be anticipated in future releases following the normal working procedures.

NOTE 1: In case UICC is used, the exact form factor and whether it is removable, non-removable or integrated is out of scope of 3GPP.

The above guidance, if endorsed, can be sent to SA3 and other involved WGs with LS from SA#108, or documented in the meeting minutes of SA#108.