3GPP TSG-SA3 Meeting #98-LI	s3i250404

# Florence, Italy, 15th Jul 2025 - 18th Jul 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0754 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications and consolidation of provisioning details |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD_US) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI19 |  |  |  |  |  | Date: |  |  | 2025-07-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The existing provisioning clauses on provisioning repeat the same information in multiple places. In order to reduce duplication and errors, the common criteria should be put in the provisioning clause with only the NF specific extensions in the NF clauses. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Common criteria are consolidated to provisioning clause. Some of the tables do not match the existing XSD, so the tables are corrected. Additional clarifications are provided for the usage of some of the provisioning fields including language alignments with the ETSI documents defining the provisioning interface. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will continue to be redundancies in the document, the use of some of the fields will remain unclear and the language in this document will not align with the ETSI document defining the interface used for provisioning. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2, 5.2.2.1, 5.2.2.2, 5.2.2.2.1, 5.2.2.2.2, 5.2.2.2.3, 5.2.2.3, 5.2.2.3.1, 5.2.2.3.2, 5.2.5, 5.2.6, 6.2.2.1, 6.2.2.1.1, 6.2.3.1.2, 6.2.3.3.1, 6.2.3.4, 6.2.5.1, 6.3.2.1, 6.3.3.1.1, 6.3.3.1.2, 6.3.3.1.3, 7.3.1.2.1, 7.3.1.2.2, 7.3.1.3, 7.9.1.2, 7.9.1.2.1, 7.9.1.2.2, 7.9.1.2.3, 7.13.1.2, 7.13.2, 7.13.2.1, 7.13.2.1.1, 7.13.2.2, 7.13.2.2.1, 7.14.1.1, 7.15.1.1, 7.16.1.1, 7.17.1.1, 7.18.1.1, 7.19.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS 33.128 CR 0762, 0768 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Contributions 0762 and 0768 reference tables and clauses updated by this CR. This contribution does not rely on 0762 or 0768 and the approval of those CRs should not affect this contribution. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | s3i250312 |  |  |  |  |  |  |  |  |

## **** START OF FIRST CHANGE (MAIN DOCUMENT) ****

### 5.2.2	Usage for realising LI_X1

#### 5.2.2.1	General

For the purposes of realising LI_X1 between the LIPF and a POI, MDF or TF, the LIPF plays the role of the ADMF as defined in ETSI TS 103 221-1 [7] reference model (clause 4.2), and the POI, MDF or TF plays the role of the NE.

Unless otherwise specified, the messages and information elements referenced in the clauses below are defined in ETSI TS 103 221-1 [7].

In general, and unless otherwise specified, the LIPF shall:

-	When the provisioning of an IRI-POI/IRI-TF/MDF2 is needed to meet the requirements of the warrant, send an ActivateTask (and subsequent ModifyTask if/as needed) with the DeliveryType set to "X2Only" and the ListOfDIDs containing at least one DID for an X2 or LI_HI2 delivery destination over LI_X1 to each of the relevant functions. The messages sent to the MDF2 shall have the DeliveryType of the MediationDetails set to "HI2Only".

-	When the provisioning of a CC-POI/CC-TF/MDF3 is needed to meet the requirements of the warrant, send an ActivateTask (and subsequent ModifyTask if/as needed) with the DeliveryType set to "X3Only" and the ListOfDIDs containing at least one DID for X3 or LI_HI3 delivery destination over LI_X1 to each of the relevant functions. The messages sent to the MDF3 shall have the DeliveryType of the MediationDetails set to "HI3Only".

When both the above are required to meet the requirements of the warrant, the ADMF shall send each independently to each relevant function.

Unless otherwise specified, the LIPF shall:

-	Configure at least one delivery destination with the relevant DeliveryType in the POI, MDF, or TF by sending a CreateDestination as described in ETSI TS 103 221-1 [7] clause 6.3.1.

-	Provision the MDFs that will be listed as the delivery destinations for a specific interception prior to the provisioning of any POIs/TFs.

When it is required to cease interception, the ADMF shall send a DeactivateTask message to each relevant function, unless the Task has already been removed by other means (e.g. by the use of the ImplicitDeactivationAllowed flag, see ETSI TS 103 221-1 [7] clause 6.2.1.2).

Other deployments compliant with ETSI TS 103 221-1 [7] may be used subject to local agreement.

#### 5.2.2.2	Provisioning of the MDF2 and MDF3

##### 5.2.2.2.1	General

When required to meet the requirements of the warrant, the LIPF shall provision the MDF2 and MDF3 over LI_X1 using the X1 protocol. Table 5.2.2.2-1 shows the details of the LI_X1 ActivateTask message used for provisioning the MDF2 and MDF3 subject to any additional requirements specified in specific provisioning clauses.

##### 5.2.2.2.2	ActivateTask message for the MDF2 and MDF3

Table 5.2.2.2.2-1: ActivateTask message for the MDF2 and MDF3

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| XID | XID assigned by LIPF. | M |
| TargetIdentifiers | One or more of the target identifiers listed in the relevant clause. | M |
| DeliveryType | Shall be set to "X2Only" if sent to the MDF2, "X3Only" if sent to the MDF3. | M |
| ListOfDIDs | Delivery endpoints of the LI product sent over LI_HI2 or LI_HI3. At least one shall be present. These delivery endpoints shall be configured using the CreateDestination message as described in ETSI TS 103 221-1 [7] clause 6.3.1 prior to first use. | M |
| ListOfMediationDetails | Sequence of Mediation Details, See table 5.2.2.2.3-1. | M |

##### 5.2.2.2.3	MediationDetails for the MDF2 and MDF3

Table 5.2.2.2.3-1: Mediation Details for the MDF2 and MDF3

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| LIID | Lawful Intercept ID associated with the task. | M |
| DeliveryType | Set to "HI2Only" when sent to the MDF2 and "HI3Only" when sent to the MDF3. | M |
| ListOfDIDs | Delivery endpoints of the LI product sent over LI_HI2 or LI_HI3 for this LIID. Shall be included if deviation from the ListofDIDs in the ActivateTask message is necessary. If included, the ListOfDIDs in the Mediation Details shall be used instead of any delivery destinations authorised by the ListOfDIDs field in the ActivateTask Message. | C |
| ServiceScopingOptions | Shall be included to Identify the service(s) and associated service-related delivery settings for this LIID. May include more than one instance of this parameter to allow for different combinations of subparameters associated with a single LIID. This parameter is defined in ETSI TS 103 221-1 [7] Annex C table C.2. | C |

#### 5.2.2.3	Provisioning of the POIs and TFs

##### 5.2.2.3.1	General

When required to meet the requirements of the warrant, the LIPF shall provision the POIs and TFs required to intercept all information authorized by the warrant over LI_X1 using the X1 protocol. Table 5.2.2.3.2-1 shows the details of the LI_X1 ActivateTask message used for provisioning the POIs and TFs that shall be present, subject to any additional requirements specified in specific provisioning clauses.

##### 5.2.2.3.2	ActivateTask message for the POIs and TFs

Table 5.2.2.3.2-1: ActivateTask message for the POIs and TFs

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| XID | XID assigned by LIPF. | M |
| TargetIdentifiers | One or more of the target identifiers listed in the relevant clause. | M |
| DeliveryType | Set to "X2Only" when sent to the IRI-POI/IRI-TF or to "X3Only" when sent to the CC-POI/CC-TF as needed to meet the requirements of the warrant. "X2andX3" may be used if specified in the specific provisioning clauses. | M |
| ListOfDIDs | Delivery endpoints of the LI product sent over LI_X2 or LI_X3. At least one shall be present. These delivery endpoints shall be configured using the CreateDestination message as described in ETSI TS 103 221-1 [7] clause 6.3.1 prior to first use. | M |
| ListOfServiceTypes | Shall be included when the task should only intercept specific CSP service types as described in clause 5.2.4. This parameter is defined in ETSI TS 103 221-1 [7] clause 6.2.1.2. | C |

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

### 5.2.5	Usage for realising LI_T2 and LI_T3

For the purposes of realising the triggering interfaces (LI_T2 between an IRI-TF and a triggered IRI-POI and LI_T3 between a CC-TF and a triggered CC-POI), the TF plays the role of the "ADMF" as defined in the ETSI TS 103 221-1 [7] reference model (clause 4.2), and the triggered POI plays the role of the "NE".

In case the TF receives an error from the triggered POI, the TF shall send a ReportTaskIssue message to the LIPF. In such case, the failure of LI shall not impact the target's or other users' services.

Whenver a relevant event is detected; the TF shall provision the relevant POI by sending an ActivateTask as defined in ETSI TS 103 221-1 [7] clause 6.2.1 over the triggering interface using the X1 protocol. Table 5.2.5-1 shows the details of the ActivateTask message used for provisioning a triggered POI subject to any additional requirements specified in specific provisioning clauses.

Table 5.2.5-1: ActivateTask message for Triggered POI

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| XID | XID assigned by triggering function. | M |
| TargetIdentifiers | One or more of the target identifiers listed in the relevant clause. The target identifier sent to the POI may be different from the target identifier specified in the warrant. | M |
| DeliveryType | Set to "X2Only" when sent to the IRI-POI or to "X3Only" when sent to the CC-POI as needed to meet the requirements of the warrant."X2andX3" may be used if specified in the specific provisioning clauses. | M |
| ListOfDIDs | Delivery endpoints of the LI product sent over LI_X2 or LI_X3. At least one shall be present. These delivery endpoints shall be configured using the CreateDestination message as described in ETSI TS 103 221-1 [7] clause 6.3.1 prior to first use. | M |
| CorrelationID | Correlation ID to assign to the LI product generated by the POI for this task. Shall be populated with the same Correlation ID used for the xIRI associated to this task generated by the POIs in the NF where the TF is located. | M |
| ProductID | Shall be set to the XID of the Task Object associated with the interception at the TF. The triggered POI shall use this value as the XID for LI product generated by this task. | M |
| ListOfServiceTypes | Shall be included when the task should only intercept specific CSP service types as described in clause 5.2.4. This parameter is defined in ETSI TS 103 221-1 [7] clause 6.2.1.2. | C |

When the TF determines that it is required to remove a Task at a particular POI (e.g. having detected the end of a session) it shall send a DeactivateTask message for the relevant Task to that POI, unless the Task has already been removed by other means (e.g. by the use of the ImplicitDeactivationAllowed flag, see ETSI TS 103 221-1 [7] clause 6.2.12).

When the TF receives a DeactivateTask message or ModifyTask message from the LIPF, the TF shall send DeactivateTask or ModifyTask messages to all applicable triggered POIs for all tasks associated to the Task object in the message from the LIPF.

When the TF reports the status of a Task via a GetTaskDetailsResponse or GetAllDetailsResponse, the TF shall also report the details of each 'delegated' Task that the TF is maintaining at an POI as a result of that Task. The details are given using the DelegatedTaskStatus structure described in table 5.2.5-1 below, which is placed in the TaskStatusExtensions element of the TaskStatus structure in the response (see ETSI TS 103 221-1 [7] clause 6.4.2.2).

Table 5.2.5-1: DelegatedTaskStatus definition

| ETSI TS 103 221-1 field name | Description | M/C/O |
| --- | --- | --- |
| ListOfDelegatedTasks | List of DelegatedTask structures (see table 5.2.5-2). | M |

Table 5.2.5-2: DelegatedTask definition

| ETSI TS 103 221-1 field name | Description | M/C/O |
| --- | --- | --- |
| NEID | NE Identifier (see ETSI TS 103 221-1 [7] clause 6.1) of the triggered POI where the TF is maintaining the relevant Task. | M |
| TaskDetails | Contains a copy of the relevant Task, as maintained by the TF at the triggered POI. | M |
| TaskStatus | Copy of the last TaskStatus information received from the triggered POI regarding the relevant Task, if available. | C |
| LastTaskStatusTime | Time at which the TaskStatus information was received. Shall be present if TaskStatus is supplied. | C |
| NEIPAddress | IPAddress of the triggered POI encoded as per ETSI TS 103 280 [97] clause 6.15, if known. | C |

The TF shall support reporting a TaskIssue to the LIPF via LI_X1 when the TF detects that there is a mismatch between the expected and actual tasking state at a triggered POI that the TF is managing. The TF may additionally attempt to take automatic remedial action. The use of either feature shall be configurable by the CSP.

### 5.2.6	Void

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

#### 6.2.2.1	Provisioning over LI_X1

##### 6.2.2.1.1	General

The IRI-POI present in the AMF and the MDF2 are provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2 and the details below.

The POI in the AMF shall support the following target identifier formats in the ETSI TS 103 221-1 [7] messages (or equivalent if ETSI TS 103 221-1 [7] is not used):

-	SUPIIMSI.

- 	SUPINAI.

-	PEIIMEI.

-	PEIIMEISV.

-	GPSIMSISDN.

-	GPSINAI.

Table 6.2.2.1-1 contains additional details for the ActivateTask message for the IRI-POI in the AMF..

Table 6.2.2.1-1: Additional details for the ActivateTask message for the IRI-POI in the AMF

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| TaskDetailsExtensions/ IdentifierAssociationExtensions | This field shall be included if the IRI POI is required to generate AMFIdentifierAssociation and AMFIdentifierDeassociation records (see clause 6.2.2.2.1). If the field is absent, AMFIdentifierAssociation and AMFIdentifierDeassociation records shall not be generated. See table 6.2.2.1-2. | C |

Table 6.2.2.1-2: IdentifierAssociationExtensions Parameters

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| TaskDetailsExtensions/EventsGenerated | One of the following values: - IdentifierAssociation - All See clause 6.2.2.2.1 for the interpretation of this field. | M |

The MDF2 listed as the delivery endpoint for the LI_X2 generated by the IRI-POI in the AMF shall be provisioned as described in clause 5.2.2.

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

##### 6.2.3.1.2	Provisioning of the IRI-POI, IRI-TF and CC-TF in the SMF

The IRI-POI, IRI-TF and CC-TF present in the SMF are provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2 with the additional details in table 6.2.3.1.2-1.

The POI/TF in the SMF shall support the following target identifier formats in the ETSI TS 103 221-1 [7] messages (or equivalent if ETSI TS 103 221-1 [7] is not used):

-	SUPIIMSI.

-	SUPINAI.

-	PEIIMEI.

-	PEIIMEISV.

-	GPSIMSISDN.

-	GPSINAI.

Table 6.2.3.1.2-1 contains additional details for the ActivateTask message for the IRI-POI, IRI-TF and CC-TF in the SMF.

Table 6.2.3.1.2-1: ActivateTask message for SMF IRI-POI, CC-TF and IRI-TF

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| TaskDetailsExtensions/ HeaderReporting | Header reporting-specific tag to be carried in the TaskDetailsExtensions field of ETSI TS 103 221-1 [7]. See table 6.2.3.9.2-1. Unless there is a CSP/LEA agreement to not report packet header information, this field shall be present to enable packet header information reporting. | C |

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

#### 6.2.5.1	Provisioning over LI_X1

The IRI-POI present in the SMSF is provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2 with the additional details in table 6.2.5.1-1.

The IRI-POI in the SMSF shall support the following target identifier formats in the ETSI TS 103 221-1 [7] messages:

-	SUPIIMSI.

-	SUPINAI.

-	PEIIMEI.

-	PEIIMEISV.

-	GPSIMSISDN.

-	GPSINAI.

If service scoping is to be performed at the IRI-POI in the SMSF, the IRI-POI in the SMSF shall support the following CSP service types (see clauses 4.4.2 and 5.2.4):

-	Messaging.

Table 6.2.5.1-1 shows additional details of the LI_X1 ActivateTask message used for provisioning the IRI-POI in the SMSF.

Table 6.2.5.1-1: Additional details for the ActivateTask message for the IRI-POI in the SMSF

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| TaskDetailsExtensions/ SMSFExtensions | This field shall be included if the delivery of the full TPDU is not authorised. See table 6.2.5.1-2. | C |

Table 6.2.5.1-2: SMSFProvisioningExtensions Parameters

| Field Name | Description | M/C/O |
| --- | --- | --- |
| TruncateTPUserData | If included, the truncatedSMSTPDU field of the sMSTPDUData (as described in table 6.2.5.3-3) structure shall be used when applicable (see text below table). If absent, the sMSTPDU field of the sMSTPDUData structure shall be used. | C |

If the TruncateTPUserData field of the LI_X1 ActivateTask message is included, the IRI-POI in the SMSF shall use the truncatedSMSTPDU field in xIRI generated at the IRI-POI in the SMSF for SMS-SUBMIT and SMS-DELIVER TPDUs, otherwise, the sMSTPDU field shall be used.

The MDF2 listed as the delivery endpoint for the LI_X2 generated by the IRI-POI in the SMSF shall be provisioned as described in clause 5.2.2 with the additional details in table 6.2.5.1-3. If SMS Content delivery is not authorized, the MDF2 shall be provisioned with the TruncateTPUserData included, otherwise it shall be left absent.

Table 6.2.5.1-3: Additional details for the ActivateTask message for the MDF2

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| TaskDetailsExtensions/ SMSFExtensions | This field shall be included if the delivery of the full TPDU is not authorised. See table 6.2.5.1-2. | C |

Table 6.2.5.1-4: Void

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

#### 6.3.2.1	Provisioning over LI_X1

The IRI-POI present in the MME is provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2 with the additional details shown in table 6.3.2.1-1.

The POI in the MME shall support the following target identifier formats:

-	IMSI (using the IMSI target identifier format from ETSI TS 103 221-1 [7]).

-	MSISDN (using the E164Number target identifier format from ETSI TS 103 221-1 [7]).

-	ME Identity (using the IMEI target identifier format from ETSI TS 103 221-1 [7]).

Table 6.3.2.1-1 shows additional details for the LI_X1 ActivateTask message used for provisioning the IRI-POI in the MME.

Table 6.3.2.1-1: Additional Details for ActivateTask message for the IRI-POI in the MME

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| TaskDetailsExtensions/ IdentifierAssociationExtensions | This field shall be included if the IRI-POI is required to generate MMEIdentifierAssociation and MMEIdentifierDeassociation records (see clause 6.3.2.2.1). If the field is absent, MMEIdentifierAssociation and MMEIdentifierDeassociation records shall not be generated. See table 6.2.2.1-2. | C |

Table 6.3.2.1-2:

Void

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

##### 6.3.3.1.1	General

If the warrant is for IRI and CC, then the LI functions in the SGW/PGW shall be provisioned in accordance with clause 6.3.3.1.2 for non-CUPS architecture and clause 6.3.3.1.3 for CUPS architecture, the MDF2 shall be provisioned in accordance with clause 6.3.3.1.4, and the MDF3 shall be provisioned in accordance with clause 6.3.3.1.5.

If the warrant is for IRI only, the IRI-POI in the SGW/PGW shall be provisioned in accordance with clause 6.3.3.1.2 for non-CUPS architecture and clause 6.3.3.1.3 for CUPS architecture and the MDF2 shall be provisioned in accordance with clause 6.3.3.1.4.

If approach 1 described in clause 6.2.3.9 is used for packet header information reporting:

-	For non-CUPS architecture, the IRI-POI in the SGW/PGW shall be provisioned in accordance with clause 6.3.3.1.2 and the MDF2 shall be provisioned in accordance with clause 6.3.3.1.4.

-	For CUPS architecture, the IRI-TF in the SGW-C/PGW-c shall be provisioned in accordance with clause 6.3.3.1.3 and the MDF2 shall be provisioned in accordance with clause 6.3.3.1.4.

If approach 2 described in clause 6.2.3.9 is used for packet header information reporting:

-	For non-CUPS architecture, the CC-POI in the SGW/PGW shall be provisioned in accordance with clause 6.3.3.1.2, the MDF2 shall be provisioned in accordance with clause 6.3.3.1.4, and the MDF3 shall be provisioned in accordance with clause 6.3.3.1.5.

-	For CUPS architecture, the CC-TF in the SGW-C/PGW-C shall be provisioned in accordance with clause 6.3.3.1.3, the MDF2 shall be provisioned in accordance with clause 6.3.3.1.4, and the MDF3 shall be provisioned in accordance with clause 6.3.3.1.5.

The LI functions in the SGW/PGW and ePDG, the MDF2 and the MDF3 shall support the following target identifier formats in the ETSI TS 103 221-1 [7] messages (or equivalent if ETSI TS 103 221-1 [7] is not used):

-	IMSI.

-	MSISDN (using the E164Number target identifier format from ETSI TS 103 221-1 [7]).

-	IMEI.

In the case of EPC-5GC interworking via combined SMF+PGW-C and UPF+PGW-U, the LI functions in the SMF+PGW-C, MDF2 and MDF3 shall support the following target identifier formats in the ETSI TS 103 221-1 [7] messages (or equivalent if ETSI TS 103 221-1 [7] is not used):

-	SUPINAI.

-	SUPIIMSI.

-	IMSI.

-	GPSINAI.

-	GPSIMSISDN.

-	MSISDN (using the E164Number target identifier format from ETSI TS 103 221-1 [7]).

-	PEIIMEISV.

-	PEIIMEI.

-	IMEI.

When the target identifier is an IMSI, the LI functions in the SMF+PGW-C shall also trigger when events associated to a SUPI in the form of an IMSI with a value matching the provisioned IMSI target identifier value are detected. Likewise, when the target identifier is a SUPIIMSI, the LI functions in the SMF+PGW-C shall also trigger when events associated to an IMSI with a value matching the provisioned SUPIIMSI target identifier value are detected.

When the target identifier is an MSISDN, the LI functions in the SMF+PGW-C shall also trigger when events associated to a GPSI in the form of an MSISDN with a value matching the provisioned MSISDN target identifier value are detected. Likewise, then the target identifier is a GPSIMSISDN, the LI functions in the SMF+PGW-C shall also trigger when events associated to an MSISDN with a value matching the provisioned GPSIMSISDN target identifier value are detected.

When the target identifier is an IMEI, the LI functions in the SMF+PGW-C shall also trigger when events associated to a PEI in the form of an IMEI with a value matching the provisioned IMEI target identifier value are detected. Likewise, then the target identifier is a PEIIMEI, the LI functions in the SMF+PGW-C shall also trigger when events associated to an IMEI with a value matching the provisioned PEIIMEI target identifier value are detected.

NOTE:	When both a 4G identifier and its equivalent 5G identifier are provisioned by means of separate tasks in the LI functions present in the SMF+PGW-C, interception will be triggered independently for each of the two identifiers.

Table 6.3.3.1.1-1 shows the additional details for the provisioning message for the IRI-POI, CC-TF and IRI-TF in the SGW/PGW and ePDG.

Table 6.3.3.1.1-1: Additional details for the ActivateTask message for the IRI-POI and CC-POI in the SGW/PGW and ePDG

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| TaskDetailsExtensions/ HeaderReporting | Header reporting-specific tag to be carried in the TaskDetailsExtensions field of ETSI TS 103 221-1 [7]. See table 6.2.3.9.2-1. Unless there is a CSP/LEA agreement to not report packet header information, this field shall be present to enable packet header information reporting. | C |

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

##### 6.3.3.1.2	Non-CUPS Architecture

When the EPS is implemented using non-CUPS architecture, the IRI-POI and CC-POI present in the SGW/PGW and ePDG are provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2 with the additional details shown in table 6.3.3.1.1-1. A single task may be used.

For non-CUPS Architecture, the DeliveryType of "X2andX3" may be used.

Table 6.3.3.1.2-1: Void

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

##### 6.3.3.1.3	CUPS Architecture

When the EPS is implemented using CUPS architecture, the IRI-POI, IRI-TF and CC-TF present in the SGW-C/PGW-C and the IRI-POI and CC-POI present in the ePDG are provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2 with the additional details shown in table 6.3.3.1.1-1.

Table 6.3.3.1.3-1: Void

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

#### 7.3.1.2.1	Target positioning service

For the LALS target positioning service (TS 33.127 [5] clause 7.3.3.2) the IRI-POI provided by the LI-LCS Client is directly provisioned over LI_X1 by the LIPF using the LI_X1 protocol as described in clause 5.2.2 additional details in Table 7.3.1.2.1-1.

Based on national regulatory requirements and CSP policy, the TaskDetailsExtensions may also include the QoS parameters (specified in OMA-TS-MLP-V3_5-20181211-C [20]) for the use on the Le interface towards the LCS Server/GMLC. Alternatively, the QoS parameters may be statically configured in the LI-LCS Client.

The POI in the LI-LCS Client shall support the following target identifier formats in the ETSI TS 103 221-1 [7] messages (or equivalent if ETSI TS 103 221-1 [7] is not used):

- SUPIIMSI.

- SUPINAI.

- GPSIMSISDN.

- GPSINAI.

- IMSI.

- MSISDN (E164Number target ID format, per ETSI TS 103 221-1 [7]).

- IMPU.

Table 7.3.1.2.1-1 shows the additional details of the LI_X1 ActivateTask message used for the LI-LCS Client provisioning for the target positioning service.

The LI_X1 DeactivateTask shall be issued by the LIPF to terminate the target positioning service and withdraw the associated provisioning data, except for the Immediate target positioning service in which case the LI_X1 DeactivateTask is not used.

Table 7.3.1.2.1-1: Additional details for the ActivateTask message for LI-LCS Client target positioning provisioning

| ETSI TS 103 221-1 field name | Description | M/C/O |
| --- | --- | --- |
| TaskDetailsExtensions/ LALSLILCSTargetProvisioning | See LALSLILCSTargetProvisioningExtensions, table 7.3.1.2.1-2. | M |

Table 7.3.1.2.1-2: LALSLILCSTargetProvisioningExtensions

| Field Name | Description | M/C/O |
| --- | --- | --- |
| PositioningServiceType | Specifies the type of positioning request. "Immediate" or "Periodic". | M |
| PositioningPeriodicity | Time interval between the positioning requests in case of Periodic positioning, in seconds. Shall be present if the PositioningServiceType is "Periodic". | C |
| PositioningParameters | Set of optional parameters for MLP SLIR message, per OMA-TS-MLP-V3_5-20181211-C [20]: - requested location type (clause 5.3.60). - requested response type (clause 5.3.112.1). - max location age (clause 5.3.65). - response timing required (clause 5.3.106). - response timer (clause 5.3.107). - horizontal accuracy with QoS class (clause 5.3.44). - altitude accuracy with QoS class (clause 5.3.6). - motion state request (clause 5.3.70). | O |

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

#### 7.3.1.2.2	Triggered location service

For the LALS triggered location service (TS 33.127 [5] clause 7.3.3.3) the LTF, as an IRI-TF, is provisioned by the LIPF using the LI_X1 protocol as described in clause 5.2.2 with the additional details in table 7.3.1.2.2-1.

The POI in the LI-LCS Client shall support the following target identifier formats in the ETSI TS 103 221-1 [7] messages (or equivalent if ETSI TS 103 221-1 [7] is not used):

- SUPIIMSI.

- SUPINAI.

- GPSIMSISDN.

- GPSINAI.

- IMSI.

- MSISDN (E164Number target ID format, per ETSI TS 103 221-1 [7]).

- IMPU.

An ActivateTask for an LTF may be issued by the LIPF if and only if at least one of the identifiers in the above list was specified in the warrant.

Table 7.3.1.2.2-1 defines the additional details for the LI_X1 ActivateTask message used for the LTF provisioning for the Triggered Location service.

Table 7.3.1.2.2-1: Additional details for the ActivateTask message for LTF triggered location service provisioning

| ETSI TS 103 221-1 field name | Description | M/C/O |
| --- | --- | --- |
| TaskDetailsExtensions/LALSLTFProvisioning | See LALSLTFProvisioningExtensions, table 7.3.1.2.2-2. | M |

Table 7.3.1.2.2-2: LALSLTFProvisioningExtensions

| Field Name | Description | M/C/O |
| --- | --- | --- |
| LILCSClientAddress | The IP address of the LI-LCS Client for triggering. | M |
| PositioningParameters | Set of optional parameters for MLP SLIR message, per OMA-TS-MLP-V3_5-20181211-C [20]: - requested location type (clause 5.3.60). - requested response type (clause 5.3.112.1). - max location age (clause 5.3.65). - response timing required (clause 5.3.106). - response timer (clause 5.3.107). - horizontal accuracy with QoS class (clause 5.3.44). - altitude accuracy with QoS class (clause 5.3.6). - motion state request (clause 5.3.70). | O |

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

#### 7.3.1.3	Triggering over LI_T2

An LTF, as an IRI-TF, provisioned as described in clause 7.3.1.2.2, triggers the LI-LCS Client (which plays the role of a triggered IRI-POI) using the LI_T2 protocol as described in clause 5.2.5 with the additional details in table 7.3.1.3-1. The LTF shall populate the "TaskDetailsExtensions" in the LI_T2 "ActivateTask" message with  the positioning parameters mapped from information the LTF receives from the LIPF over the LI_X1. The LI_T2 "ActivateTask" message header may include a correlation ID from the triggering xIRI, if available.

The LI-LCS Client shall deactivate the task on its own upon issuing the final xIRI for the trigger. There is no DeactivateTask operation on the LI_T2 for the LI-LCS Client.

Table 7.3.1.3-1 shows the additional details of the LI_T2 ActivateTask message used by the LTF to trigger LI-LCS Client for the triggered location service.

The target identifier send in the ActivateTask message shall shall correspond to one of the target identifiers in the xIRI observed by the LTF, and shall be one of the identifiers provided in the ActivateTask for the LTF (clause 7.3.1.2.2).

Table 7.3.1.3-1: Additional details for the ActivateTask message from LTF to LI-LCS Client for the triggered location service triggering

| ETSI TS 103 221-1 field name | Description | M/C/O |
| --- | --- | --- |
| CorrelationID | Correlates the requested location to the triggering xIRI, if available. NOTE: While this parameter is mandatory in table 5.2.5-1, it is conditional in messages sent from the LTF to the LI-LCS client. | C |
| TaskDetailsExtensions/LALSLILCSTargetProvisioning | See LALSLILCSTargetProvisioningExtensions, table 7.3.1.2.1-2. For this message, PositioningServiceType shall be set to "Immediate" and the the optional PositioningParameters shall be set using the PositioningParameters received in the PositioningParameters field of the LALSLTFProvisioning message received in the ActivateTask message sent to the LTF for this task. | O |

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

#### 7.9.1.2	Provisioning over LI_X1

##### 7.9.1.2.1	General

The IRI-POI in the AAnF (AKMA Anchor Function), the IRI-TF in the AAnF, and the MDF2 shall be provisioned as described in the following clauses.

Details of provisioning of an IRI-POI at a network internal AF (Application Function) making use of AKMA services of the AAnF is in general service specific and not part of the present clause. Generally, triggering, rather than provisioning, could in some cases be necessary for the AF. An application independent generic triggering mechanism is defined in clause 7.9.1.2.3.

Provisioning of CC-intercept at the AF is service specific and not covered in the present document.

##### 7.9.1.2.2	Provisioning of the IRI-POI and IRI-TF in AAnF

The IRI-POI and IRI-TF present in the AAnF are provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2.

The IRI-POI and IRI-TF in the AAnF shall support the following target identifier formats:

-	SUPI, given in either SUPIIMSI or SUPINAI formats as defined in ETSI TS 103 120 [6] clause C.2.

Table 7.9.1.2.2-1:

Void

##### 7.9.1.2.3	Triggering of the IRI-POI in AF

The IRI-POI present in the AF shall be triggered by the IRI-TF present in the AAnF over LI_T2 using the X1 protocol as described in clause 5.2.5 with the additional details in table 7.9.1.2.3-1. An AAnF can provide services for several different types of applications. Triggering could be service/application specific, which can affect whether or not certain conditional fields are included in the xIRI described in clause 7.9.1.4 below.

For all AFs a priori known to match the scope of the warrant, when the IRI-TF in the AAnF detects that an A-KID has been associated with a SUPI (see clause 7.9.1.3.2), it shall send an ActivateTask message to the IRI-POI present in the AF. The same shall apply if the AAnF detects that the A-KID of a target changes due to primary authentication. For AFs not a priori known at the AAnF, the ActivateTask message shall instead be sent when the IRI-TF in the AAnF detects that the AF performs an AKMA application key get associated with the A-KID.

NOTE:	AFs providing services based on SUPI could, as an alternative to triggering, be directly provisioned by the LIPF as part of service-specific provisioning. This is however outside the scope of the present document.

Table 7.9.1.2.3-1: Additional details for the ActivateTask message for triggering the IRI-POI in the AF

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| TargetIdentifiers | A-KID associated with the AKMA Anchor Key (see table 7.9.1.3.3-2 below). | M |
| implicitDeactivationAllowed | Shall be set to "True". | M |

Table 7.9.1.2.3-2: Target Identifier Types for LI_T3

| Identifier type | Owner | ETSI TS 103 221-1 [7] TargetIdentifier type | Definition |
| --- | --- | --- | --- |
| A-KID | 3GPP | TargetIdentifierExtension / AKID. | AKID (see XSD schema) |

When the IRI-POI present in the AF detects that a UE has requested the use of a targeted A-KID, it shall continue to generate xIRI events for that A-KID until it detects that the UE has requested the use of a different A-KID, at which point it shall implicitly deactivate the previous Task. In addition, the AAnF may at any time issue a DeactivateTask message against the Task, at which point the AF shall cease interception of the A-KID and remove the Task as per ETSI TS 103 221-1 [7] clause 6.2.3.

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

#### 7.13.1.2	Provisioning of the POIs and TFs in the RCS Server and the POIs in the HTTP Content Server, File Transfer Localization Function and S-CSCF by the LIPF

The IRI-POI, CC-POI, IRI-TF and CC-TF present in the RCS Server, the IRI-POI and CC-POI in the HTTP Content Server, and File Transfer Localization Function and the IRI-POI in the S-CSCF, are provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2 with the additional details shown in table 7.13.1.2-1.

The POIs and TFs in the RCS Server and the IRI-POIs in the S-CSCF shall support the following target identifier formats in the ETSI TS 103 221-1 [7] messages (or equivalent if ETSI TS 103 221-1 [7] is not used).

-	IMPU.

-	IMPI.

-	IMEI.

-	PEIIMEI.

The POIs in the HTTP Content Server and File Transfer Localization Function shall support the following additional target identifier formats in the ETSI TS 103 221-1 [7] messages (or equivalent if ETSI TS 103 221-1 [7] is not used).

-	SIPURI.

-	TELURI.

-	GPSIMSISDN.

-	GPSINAI.

-	IMSI.

-	SUPIIMSI.

-	SUPINAI.

-	Email Address.

Table 7.13.1.2-1 shows the additional details of the LI_X1 ActivateTask message used for provisioning the IRI-POI, CC-POI, IRI-TF and CC-TF in the RCS Servers and the IRI-POI and CC-POI in the HTTP Content Server and File Transfer Localization Function and the IRI-POI in S-CSCF.

Table 7.13.1.2-1: Additional details for the ActivateTask message for the IRI-POI, CC-POI, IRI-TF and CC-TF in the RCS Servers and the IRI-POI and CC-POI in the HTTP Content Server and S-CSCF

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| XID | XID assigned by LIPF. If the CC-TF or IRI-TF is also being tasked for the same interception, the same XID shall be used. The same XID shall be used at the RCS Servers, the S-CSCF and the HTTP Content Server for the same interception. | M |

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

### 7.13.2	Triggering of the IRI-POI and CC-POI in the HTTP Content Server

#### 7.13.2.1	Triggering of the IRI-POI in the HTTP Content Server over LI_T2

##### 7.13.2.1.1	LI_T2 interface Specifics

In order to allow the IRI-POI in the HTTP content server to detect all events related to files uploaded or downloaded by a target, the IRI-TF in the RCS Server sends a trigger to the IRI-POI present in the HTTP Content Server with the necessary information over the LI_T2 interface.

When the IRI-TF in the RCS Server detects that a file is being uploaded or downloaded by a target UE it shall send an activation message to the IRI-POI in the HTTP Content Server over the LI_T2 interface. The activation message shall contain the correlation identifiers that the IRI-POI in the HTTP Content Server shall use with the xIRI. This can be achieved by sending an ActivateTask message as described in clause 5.2.5.

Table 7.13.2.1.1-1: Void

Table 7.13.2.1.1-2: Target Identifier Types for LI_T2

| Identifier type | Owner | ETSI TS 103 221-1 [7] TargetIdentifier type | Definition |
| --- | --- | --- | --- |
| RCS Content URI (See Note) | 3GPP | TargetIdentifierExtension / RCSContentURI | RCSContentURI (see XSD schema) |
| NOTE: If the TargetIdentifier used is an RCS Content URI, only one RCS Content URI shall be included per ActivateTask message. |  |  |  |

#### 7.13.2.2	Triggering of the CC-POI in the HTTP Content Server over LI_T3

##### 7.13.2.2.1	LI_T3 interface Specifics

To support the use-cases where the IRI-POI in the HTTP Content Server does not get the identity of the user involved in the file-transfer (and therefore, the CC-POI in the HTTP Content Server cannot perform the interception based on the target identity provisioned by the LIPF), the CC-TF present in the RCS Server sends a trigger to the CC-POI present in the HTTP Content Server. When the CC-TF in the RCS Server detects that a file is being uploaded or downloaded by a target UE, it shall send an activation message to the CC-POI in the HTTP Content Server over the LI_T3 interface. The activation message shall contain the correlation identifiers that the CC-POI in the HTTP Content Server shall use with the xCC. This can be achieved by sending an ActivateTask message as described in clause 5.2.5.

Table 7.13.2.2.1-1: Void

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

#### 7.14.1.1	Provisioning of IRI-POI in EES

The IRI-POI present in the EES is provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2.

The POI in the EES shall support the identifier types given in table 7.14.1.1-1.

Table 7.14.1.1-1: TargetIdentifier Types for Edge Computing

| Identifier | Owner | ETSI TS 103 221-1 [7] TargetIdentifier type | Definition |
| --- | --- | --- | --- |
| gPSIMSISDN | ETSI | GPSIMSISDN | See ETSI TS 103 221-1 [7] |
| gPSINAI | ETSI | GPSINAI | See ETSI TS 103 221-1 [7] |
| eECID | 3GPP | TargetIdentifierExtension | See XSD schema |

Table 7.14.1.1-2: Void

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

#### 7.15.1.1	Provisioning of IRI-POI in 5GMS AF

The IRI-POI present in the 5GMS AF is provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2.

The POI in the 5GMS AF shall support the target identifier types given in table 7.15.1.1-1.

Table 7.15.1.1-1: TargetIdentifier types for 5G media streaming

| Identifier | Owner | ETSI TS 103 221-1 [7] TargetIdentifier type | Definition |
| --- | --- | --- | --- |
| gPSIMSISDN | ETSI | GPSIMSISDN | See ETSI TS 103 221-1 [7] |
| gPSINAI | ETSI | GPSINAI | See ETSI TS 103 221-1 [7] |

Table 7.15.1.1-2: Void

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

#### 7.16.1.1	Provisioning of IRI-POI in NWDAF

The IRI-POI present in the NWDAF is provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2.

The POI in the NWDAF shall support the target identifier types given in table 7.16.1.1-1.

Table 7.16.1.1-1: TargetIdentifier types for data analytics

| Identifier | Owner | ETSI TS 103 221-1 [7] TargetIdentifier type | Definition |
| --- | --- | --- | --- |
| SUPIIMSI | ETSI | SUPIIMSI | See ETSI TS 103 221-1 [7] |
| SUPINAI | ETSI | SUPINAI | See ETSI TS 103 221-1 [7] |

Table 7.16.1.1-2: Void

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

#### 7.17.1.1	Provisioning of IRI-POI in 5G DDNMF

The IRI-POI present in the 5G DDNMF is provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2.

The IRI POI in the 5G DDNMF shall support the target identifier types given in table 7.17.1.1-1.

Table 7.17.1.1-1: TargetIdentifier types for data analytics

| Identifier | Owner | ETSI TS 103 221-1 [7] TargetIdentifier type | Definition |
| --- | --- | --- | --- |
| sUPIIMSI | ETSI | SUPIIMSI | See ETSI TS 103 221-1 [7] |
| sUPINAI | ETSI | SUPINAI | See ETSI TS 103 221-1 [7] |
| gPSIMSISDN | ETSI | GPSIMSISDN | See ETSI TS 103 221-1 [7] |
| gPSINAI | ETSI | GPSINAI | See ETSI TS 103 221-1 [7] |

Table 7.17.1.1-2: Void

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

#### 7.18.1.1	Provisioning of IRI-POI in 5G PIN Server

The IRI-POI present in the 5G PIN Server is provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2.

The POI in the 5G PIN Server shall support the target identifier types given in table 7.18.1.1-1.

Table 7.18.1.1-1: TargetIdentifier types for PIN

| Identifier | Owner | ETSI TS 103 221-1 [7] TargetIdentifier type | Definition |
| --- | --- | --- | --- |
| gPSIMSISDN | ETSI | GPSIMSISDN | See ETSI TS 103 221-1 [7] |
| gPSINAI | ETSI | GPSINAI | See ETSI TS 103 221-1 [7] |
| pINClientID | 3GPP | TargetIdentifierExtension | See XSD schema |
| identityToken | 3GPP | TargetIdentifierExtension | See XSD schema |
| pINID | 3GPP | TargetIdentifierExtension | See XSD schema |

Table 7.18.1.1-2: Void

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

### 7.19.1	Provisioning over LI_X1

The IRI-POI present in the common charging function (e.g. CDF, CHF) (see TS 32.240 [147] figure 4.2.1.1) is provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2 with the additional details from table 7.19.1-1.

The charging function shall not be provisioned unless the warrant covers all services for which charging records may be generated, and reporting of location information is authorised.

The IRI-POI in the charging function shall support the following target identifiers formats in ETSI TS 103 221-1 [7] messages:

-	SUPIIMSI

-	SUPINAI

-	IMSI

-	GPSIMSISDN

-	GPSINAI

-	E164Number

-	SIP-URI

Table 7.19.1-1 shows the additional details of the LI_X1 ActivateTask message used for provisioning the IRI-POI in the charging function.

Table 7.19.1-1: Additional details for the ActivateTask message for the IRI-POI in the charging function

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| TaskDetailsExtensions/ ChargingDataEvents | This field shall be included if the IRI POI is required to generate ChargingDataEvent records (see clause 7.19.3.2). If the field is absent, ChargingDataEvent records shall not be generated. | C |

Table 7.19.1-2: ChargingDataEventsExtensions Parameters

| Field Name | Description | M/C/O |
| --- | --- | --- |
| ChargingDataEventsGenerated | One of the following values: - DefaultChargingDataInformation - TranslatedChargingDataInformation See clause 7.19.3.1 for the interpretation of this field. | M |

## **** END OF ALL CHANGES ****