3GPP SA3LI#98	S3i250429

# 15-18 July 2025, Florence (Italy)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0758 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to NIDD clauses |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Nokia, Ministère Economie et Finances) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI19 |  |  |  |  |  | Date: |  |  | 2025-07-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Rest of the clauses in TS 33.128 provide a table that depicts the ActivateTask used for LI_X1, the NIDD clause fails to have such a table. Additionally, not all cases of IRI-POI only are listed. IWK-SCEF is not mentioned at some places. |  |  |  |  |  |  |  |  |
| Summary of change: |  | ActivateTask tables are added for LI at NEF and LI at SCEF/IWK-SCEF clauses. Missing case of IRI-POI only is added. IWK-SCEF is mentioned where it needs to be mentioned. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missed LI aspects. Will not aligned within the document and also to stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7.1.1, 7.7.1.2, 7.8.1.1, 7.8.1.2, 7.8.2.1.1, 7.8.2.2, 7.8.2.4. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3i250318 |  |  |  |  |  |  |  |  |

### ** First Change **

#### 7.7.1.1	General

For NIDD using NEF:

-	If delivery type for the warrant is "IRI and CC", then the IRI-POI and the CC-POI in the NEF, the MDF2 and MDF3 shall be provisioned.

-	If delivery type for the warrant is "IRI", then the IRI-POI in the NEF and the MDF2 shall be provisioned.

-	Delivery type "CC" is not applicable to the warrant.

For device triggering, MSISDN-less MO SMS, Parameter Provisioning, AF session with QoS provision and 5G LAN Parameter Provisioning:

-	the delivery type for the warrant is "IRI"; the IRI-POI in the NEF and the MDF2 shall be provisioned.

### ** Next Change **

#### 7.7.1.2	Provisioning of the IRI-POI and CC-POI in NEF

The IRI-POI and CC-POI present in the NEF are provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2.

The POI in the NEF shall support the following target identifier formats in the ETSI TS 103 221-1 [7] messages (or equivalent if ETSI TS 103 221-1 [7] is not used):

-	SUPIIMSI.

-	SUPINAI.

-	GPSIMSISDN.

-	GPSINAI.

NOTE:	For Parameter Provisioning and 5G LAN Parameter Provisioning, only GPSIMSISDN and GPSINAI are applicable.

Table 7.7.1.2-1 shows the minimum details of the LI_X1 ActivateTask message used for provisioning the IRI-POI and CC-POI present in the NEF.

Table 7.7.1.2-1: ActivateTask message for activating IRI-POI and CC-POI present in the NEF

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| XID | XID assigned by LIPF. The same value shall be used for provisioning the IRI-POI, CC-POI, MDF2 and MDF3. | M |
| TargetIdentifiers | One of the target identifiers listed in the clause above. | M |
| DeliveryType | Set to "X2Only", or "X2andX3" as needed to meet the requirements of the warrant. | M |
| ListOfDIDs | Delivery endpoints of LI_X2 or LI_X3. These delivery endpoints shall be configured using the CreateDestination message as described in ETSI TS 103 221-1 [7] clause 6.3.1 prior to first use. | M |
| NOTE: As stated in clause 7.7.1.1, the provisioning of CC-POI and MDF3 is not required for device triggering, MSISDN-less MO SMS, Parameter Provisioning. AF session with QoS provision and 5G-LAN Parameter Provisioning. |  |  |

The ModifyTask and DeactivateTask messages that the LIPF may send to the IRI-POI present in the NEF shall include the XID of the Task created by the above ActivateTask message.

### ** Next Change **

#### 7.8.1.1	General

For NIDD using SCEF:

-	If delivery type for the warrant is "IRI and CC", then the IRI-POI and the CC-POI in the SCEF/IWK-SCEF, the MDF2 and MDF3 shall be provisioned.

-	If delivery type for the warrant is "IRI", then the IRI-POI in the SCEF and the MDF2 shall be provisioned.

-	Delivery type "CC" is not applicable to the warrant.

For device triggering, MSISDN-less MO SMS, Parameter Provisioning and AS session with QoS provision:

-	the delivery type for the warrant is "IRI"; the IRI-POI in the SCEF and the MDF2 shall be provisioned.

NOTE:	The provisioning of IRI-POI and CC-POI in the IWK-SCEF is required only in the VPLMN.

### ** Next Change **

#### 7.8.1.2	Provisioning of the IRI-POI and CC-POI in the SCEF/IWK-SCEF

The IRI-POI and CC-POI present in the SCEF/IWK-SCEF are provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2.

The POI in the SCEF/IWK-SCEF shall support the following target identifier formats in the ETSI TS 103 221-1 [7] messages (or equivalent if ETSI TS 103 221-1 [7] is not used):

-	IMSI.

-	MSISDN.

-	External Identifier.

NOTE 1:	For Parameter Provisioning, only MSISDN and External Identifier are applicable.

NOTE 2:	Provisioning of IRI-POI and CC-POI in the IWK-SCEF is required only in the VPLMN.

Table 7.8.1.2-1 shows the minimum details of the LI_X1 ActivateTask message used for provisioning the IRI-POI and CC-POI present in the SCEF/IWK-SCEF.

Table 7.8.1.2-1: ActivateTask message for activating IRI-POI and CC-POI present in the SCEF/IWK-SCEF

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| XID | XID assigned by LIPF. The same value shall be used for provisioning the IRI-POI, CC-POI, MDF2 and MDF3. | M |
| TargetIdentifiers | One of the target identifiers listed in the clause above. | M |
| DeliveryType | Set to "X2Only", or "X2andX3" as needed to meet the requirements of the warrant. | M |
| ListOfDIDs | Delivery endpoints of LI_X2 or LI_X3. These delivery endpoints shall be configured using the CreateDestination message as described in ETSI TS 103 221-1 [7] clause 6.3.1 prior to first use. | M |
| NOTE: As stated in clause 7.8.1.1, the provisioning of CC-POI and MDF3 is not required for device triggering, MSISDN-less MO SMS, Parameter Provisioning and AS session with QoS provision. |  |  |

The ModifyTask and DeactivateTask messages that the LIPF may send to the IRI-POI present in the NEF shall include the XID of the Task created by the above ActivateTask message.

### ** Next Change **

##### 7.8.2.1.1	General

The IRI-POI present in the SCEF/IWK-SCEF shall send the xIRIs over LI_X2 for each of the events listed in TS 33.127 [5] clause 7.8.2.3, the details of which are described in the following clauses. Each event will be based on PDN Connection between SCEF and target UE, except in case of Unsucessful Procedure.

### ** Next Change **

#### 7.8.2.2	Generation of xCC at CC-POI in SCEF over LI_X3

The CC-POI present in the SCEF/IWK-SCEF shall send xCC over LI_X3 for each NIDD packet.

Each X3 PDU shall contain the contents of the user plane packet (i.e. NIDD) using an unstructured payload.

The CC-POI present in the SCEF/IWK-SCEF shall set the payload format to indicate the appropriate payload type (i.e. unstructured payload) as described in ETSI TS 103 221-2 [8] clause 5.4.

### ** Next Change **

#### 7.8.2.4	Generation of CC over LI_HI3

When xCC is received over LI_X3 from the CC-POI in the SCEF/IWK-SCEF, the MDF3 shall populate the ETSI TS 102 232-1 [9] @LI-PS-PDU.payload.cCPayloadSequence.cCContents.threeGPP33128DefinedCC field with a CCPayload.pDU.nIDDCCPDU and send it over LI_HI3 interface to LEMF without undue delay.

The ETSI TS 102 232-1 [9] @LI-PS-PDU.pSHeader.timeStamp field shall be set to the time that the SCEF/IWK-SCEF observed the data (i.e. the timestamp field of the xCC). The LIID and CID fields shall correctly reflect the target identity and communication session to which the CC belongs.

### ** End of all Changes **