3GPP TSG-SA3 Meeting #98-LI	s3i250367

# Florence, Italy, 15th Jul 2025 - 18th Jul 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0770 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment of PositioningMethod types with TS 29.572 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Ofcom (CH)) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI18 |  |  |  |  |  | Date: |  |  | 2025-07-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current table and ASN.1 definition of the PositioningMethod types do not include all the values specified in the latest versions of TS 29.572 table 6.1.6.3.6-1. The Sidelink positioning methods are added in the corresponding table and the ASN.1 schema. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The definition is extended to include the Sidelink positioning methods in the corresponding table and the ASN.1 schema. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with TS 29.572; values specified in TS 29.572 table 6.1.6.3.6-1 for the Sidelink poistioning methods will not be supported over the LI interfaces. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.3.2.52; Attachment TS33128Payloads.asn |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is associated with the following changes in the Forge: Merge request: https://forge.3gpp.org/rep/sa3/li/-/merge_requests/339 Commit hash: d3721a22345061782165ddbee3d22c0307ecc669 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** FIRST MAIN DOCUMENT CHANGE ***

##### 7.3.3.2.52	Enumeration: PositioningMethod

The PositioningMethod represents the method used to determine the location of the UE, the PositioningMethod is derived from the data present in the PositioningMethod type defined in TS 29.572 [24] clause 6.1.6.3.6.

Table 7.3.3.2.52-1 contains the details of the PositioningMethod type.

Table 7.3.3.2.52-1: Enumeration for PositioningMethod

| Enumeration value | Description |
| --- | --- |
| cellID(1) | Cell ID positioning method. |
| eCID(2) | Enhanced cell ID methods based on LTE signals. |
| oTDOA(3) | Observed time difference of arrival positioning based on LTE signals. |
| barometricPressure(4) | Positioning method based on barometric Pressure Sensor. |
| wLAN(5) | WLAN positioning. |
| Bluetooth(6) | Bluetooth positioning. |
| mBS(7) | Terrestrial Beacon System (TBS) positioning based on MBS signals. |
| motionSensor(8) | Positioning method based on motion Sensor. |
| dLTDOA(9) | Downlink Time Difference of Arrival (DL-TDOA) based on NR signals. |
| dLAOD(10) | Downlink Angle-of-Departure (DL-AoD) based on NR signals. |
| multiRTT(11) | Multi-Round Trip Time Positioning (Multi-RTT based on NR signals). |
| nRECID(12) | NR enhanced cell ID methods (NR E-CID) based on NR signals. |
| uLTDOA(13) | Uplink Time Difference of Arrival (UL-TDOA) based on NR signals. |
| uLAOA(14) | Uplink Angle of Arrival (UL-AoA), including the Azimuth of Arrival (A-AoA) and the Zenith of Arrival (Z-AoA) based on NR signals. |
| networkSpecific(15) | Network specific position methods. |
| sLTDOA(16) | Sidelink Time Difference of Arrival (TDOA) based on Sidelink NR PC5 radio signals. |
| sLTOA(17) | Sidelink Time Of Arrival based on NR PC5 radio signals. |
| sLAOA(18) | Sidelink Angle-of-Arrival based on NR PC5 radio signals. |
| sLRT(19) | Sidelink Round Trip based on NR PC5 radio signals. |

*** END OF MAIN DOCUMENT CHANGES ***

START OF ATTACHMENT CHANGE 1

diff --git a/33128/r19/TS33128Payloads.asn b/33128/r19/TS33128Payloads.asn

index c3b96c04eba5c3c080f1f40bf6144ec1706343ec..49a9575e76bffc92228086e1ad9af2f82f4bb846 100644

--- a/33128/r19/TS33128Payloads.asn

+++ b/33128/r19/TS33128Payloads.asn

@@ -8397,7 +8397,11 @@ PositioningMethod ::= ENUMERATED

nRECID(12),

uLTDOA(13),

uLAOA(14),

-    networkSpecific(15)

+    networkSpecific(15),

+    sLTDOA(16),

+    sLTOA(17),

+    sLAOA(18),

+    sLRT(19)

}

-- TS 29.572 [24], clause 6.1.6.3.7

END OF ATTACHMENT CHANGE 1