3GPP SA3#98-LI	S3i250439

# July 1518, 2025; Florence, Italy

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.928 | CR | 0021 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TR 33.928 addition – LIPF provisioning logic for CHF and CDF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Nokia) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI19 |  |  |  |  |  | Date: |  |  | 2025-07-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The LI capabilities at the charging function were introduced to TS 33.127 and TS 33.128. Those functions introduce an IRI-POI at the CHF, V-CHF, and hence, requires the provisioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | LIPF provisioning logic is added to TR 33.928 to support the LI at the charging function when all of the service types to which the charging would apply. Additional changes to add a missing table for the PTC clause is made. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TR 33.928 will not be aligned to TS 33.128. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.2, 5.7.2, 5.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR 0020 must be considered while implementing this CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3i250328, S3i250392 |  |  |  |  |  |  |  |  |

### ** First Change ***

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

CDF	Charging Data Function

CHF	Charging Function

V-CHF	Visted CHF

### *** Next Change ***

#### 5.3.2.2	Service-based LI provisioning logic in LIPF

The flow-chart in figure 5.3.2.2-1 shows a top-level logic within the LIPF to branch off into separate processes according to the service type defined in the present document.

Figure 5.3.2.2-1: Top-level view of LIPF logic in handling the service type

The illustration shown in figure 5.3.2.2-1 applies even when multiple service types are applicable to warrant. Based on the LI functionality defined in the present document:

-	For the service type of Data, it is assumed that the NFs in the packet core network are involved and hence, provide the IRI and CC interception.

-	For the service type of Voice, it is assumed that the NFs in the IMS domain are involved and hence, provide the IRI and CC interception.

-	For the service type of Messaging (that includes SMS and MMS), the NFs in the packet core network, IMS or MMS Proxy Relay are involved and hence, provide the IRI and CC interception. The interception of SMS has only the IRIs. For, the service type of Messaging, the LI provisioning for the service type RCS is also done (see below) if supported and applicable to the target.

-	For the service type of PTC, the PTC Server is involved and hence, provides the IRI and CC interception.

-	For the service type of LALS, the LI-LCS Client provides the IRI interception, and the CC interception does not apply to LALS.

For the service type RCS, the RCS Server, the HTTP Content Server, the File Transfer Localization Function are involved and hence, provide the IRI and CC interception.

When the service type "Messaging" is explicitly specified in a warrant, the provisioning for the service type RCS is also performed by the LIPF, if the latter is supported and applicable to the target. If the warrant explicitly includes both "Messaging" and "RCS" as the service types, the LI provisioning for the service type RCS is done only once.

The UDM and HSS are also the NFs that have the IRI-POI and the provisioning of IRI-POI in UDM and HSS is independent of the service type indicated in the warrant as long as the target is not indicated as a non-local ID. The provisioning of IRI-POI in the UDM and HSS for a target identifier is done only once.

The IRI-POI in CHF and CDF are provisioned when certain conditions are met as illustrated in figure 5.3.2.1-x below:

Figure 5.3.2.1-x: Illustration of logic showing the preconditions for LI at charging function

LIPF logic for provisioning the IRI-POI in CHF and CDF when the conditions illustrated in figure 5.3.2.1-x are met is illustrated in clause 5.X.

When multiple service types are applicable for a target identifier, the LIPF may provision the LI function in a NF only once including all the applicable service types. Alternatively, the LIPF may determine the services that need to be intercepted at the LI function present in a NF and then provision that LI function for all services together.

When charging records are received if the charging related LI needs are different for those multiple intercepts, the MDF2 will have to determine whether the charging related records can be delivered to the LEMF following the pre-conditions checks similar to the illustration shown in figure 5.3.2.1-x.  Such a pre-condition check is not needed, if the LIPF provisions the IRI-POIs in CHF and CDF along with the other associated POIs, TFs separately for the each of those intercepts.

As a deployment option, the LIPF may choose to use a separate XID for provisioning the IRI-POIs in CHF and CDF and the MDF2 for charging related LI. In this case, repeated provisioning of NFs for multiple intercepts on the same target id or MDF2 checking for the pre-conditions is not necessary.

##### ** Next change **

### 5.7.2	LIPF logic for service type of PTC

Figure 5.7.2-1 illustrates the LIPF logic for the provisioning of IRI-POI/CC-POI in the PTC server.

Figure 5.7.2-1: LIPF logic for provisioning the IRI-POI/CC-POI in PTC Server for the service type of PTC

When the Chat Group ID is used as the target ID, the PTC server happens to intercept when the indicated chat group ID is used for a PTC chat session.

Table 5.7.2-x provides the scope of NF domain that provides the IRI-POI/CC-POI functions for the service type of PTC.

Table 5.7.2-x: Scope of NF providing the LI functions for PTC

| NF with the LI function | Roaming |  |  |
| --- | --- | --- | --- |
|  | Not | Outbound | Inbound |
| HSS (target local) | IRI-POI | IRI-POI | n/a |
| PTC Server | IRI-POI | IRI-POI | n/a |
| PTC Server | CC-POI | CC-POI | n/a |

NOTE 1:	The use of "n/a" in the above table implies that the LI function is not applicable to the NF for the indicated scenario.

NOTE 2:	The LIPF is not aware of the above role played by the NFs in providing the LI functions nor the roaming situations of the target or the party communicating with the target non-local ID.

NOTE 3:		MDF2 and MDF3 which are also involved in providing the LI functions are not shown in the tables above.

#### ** Next change **

## 5.X	Charging

### 5.X.1	Scope of interception

The illustrations shown in this clause is LI for charging function that includes the delivery of target’s charging information stored at the CHF or V-CHF (5GC based packet core) or CDF (EPC based packet core).

### 5.X.2	LIPF logic for service type of charging

Figure 5.X.2-1 illustrates the LIPF logic for the provisioning IRI-POI in CHF and the CDF.

Figure 5.X.2-1: LIPF logic the provisioning of IRI-POI in CHF and CDF

As illustrated in figure 5.11.2-1, in support of interworking between 4G and 5G networks, the IRI-POI in CDF and CHF are provisioned observing the following logic:

- The target id is IMEI or PEI (IMEI only).

- The target id is IMSI or SUPI (IMSI only).

- The target id is MSISDN or GPSI (MSISDN).

When the target id is SUPI (NAI only) or GPSI (NAI only), only the IRI-POI in CHF is provisioned.

Table 5.X.2-1 provides the scope of NF domain providing the LI functions for charging function.

Table 5.X.2-1: Scope of NF domain providing the LI functions for charging function

| NF with the LI function | Not-roaming | Roaming |  |
| --- | --- | --- | --- |
|  |  | VPLMN | HPLMN |
| CHF | IRI-POI | IRI-POI | IRI-POI |
| CDF | IRI-POI | IRI-POI | IRI-POI |

NOTE 1:	The LIPF is not aware of the roaming situation of the target.

NOTE 2:	MDF2 which is also involved in providing the LI function is not shown in the tables above.

NOTE 3:	The CHF in the VPLMN is referred to as V-CHF. CDF in the VPLMN is referred to as CDF.

** End of all Changes **