3GPP TSG-SA WG6 Meeting #68	S6-253616

Gothenburg, Sweden, 25th – 29th August 2025	(revision of S6-253339)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0052 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on ML model information discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It is not clear in which condition the AIMLE server need to continuously train the model |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clarification that if the received ML model does not satisfy the ML model requirement, then the AIMLE server determine to continously train the ML model. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The unclear description will remain in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.11.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.11.3	ML model information discovery

Figure 8.11.3-1 illustrates the procedure of ML model information discovery.

Pre-condition:

1.	Either AIMLE consumer has requested to discover ML model or AIMLE server decides to discover model for training.

Figure 8.11.3-1: ML model information discovery

1.	The AIMLE server sends an ML model information discovery request to the ML repository. The request contains information elements as described in Table 8.11.4.3-1.

2.	Upon receiving the ML model information discovery request, the ML repository verifies if the requestor is authorized to discover the ML model(s) identified by the ML model ID. Further, it verifies whether the requestor is present in the list of allowed vendors or not. If the ML repository consumer is authorized, the ML repository processes the request and discovers the model based on ML model ID, ADAE Analytics ID, Base Model ID, and/or ML model interoperability information.

If the discovery request is for transfer of learning, the ML repository identifies all candidate models matching the domain and required accuracy level.

The ML repository sends the response message to the ML repository consumer. The response includes information of the discovered ML model.

NOTE:	When the ML model is within training or re-training phase, the ML model is not available for discovery. The details are indicated within the Result IE in Table 8.11.4.4-1. The training and re-training phase is used during federated learning so that AIML enablement consumer can rely on model repository to get the model details.

After the AIMLE server receives the response from the ML repository, the AIMLE server determines whether the received ML model satisfy the ML model requirement of the AIMLE server. If not, the AIMLE server needs to continuously train the model to satisify the ML model requirement based on the AIMLE server capability of supporting continuous training and ML model profile (Indication of continuous model training, continuous training model parameter).

* * * End of Change * * * *