3GPP TSG-SA5 Meeting #162 	S5-253243

Stor-Göteborg, Sweden, 25th August 2025 - 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0715 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.552 Aggregated Performance Measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_Trace_MDT_QoE-OAM |  |  |  |  |  | Date: |  |  | 2025-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | With current subcounter and filter, the result is a list of measurements of each subcounter/filter value. However, there are use cases where an operator would like to receive the aggregated measurement result when filter is specified. This could reduce the output dimensionality. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding aggregated measurement method |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unable to receive aggregated measurement result |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2, 4.3, 4.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

### 4.2.2	Filter naming

The Filter shall be in the form: NameValue(s)

The Filter Name shall be an alphanumeric string specified in the performance measurement definition. The Filter Name shall always end with an alphabet character. The Filter Value shall be a numeric string.

For the following Filters:

-	QOS, where QOS represents the mapped 5QI or QCI value.

-	SNSSAI, where SNSSAI represents the S-NSSAI.

-	PLMN, where PLMN represents the PLMN ID.

It is possible to specify a Filter in the form of Filter Name with single Filter Value, or with multiple Filter Values. When multiple Filter Values are provided for a given Filter Name, Filter Values shall be separated by '|'.  Filter Value ranges shall be defined in the form of 'x-y'. Combination of using '|' and ranges for the Filter Values of a given Filter Name is possible. The order of the Filter Values is not important. Any other vendor-specific multiple value form is allowed for the combination. It is possible to have multiple Filter Values enclosed in square brackets '[...]' to explicitly indicate that the measurement result shall be aggregated across the specified values.

The followings are examples of possible resulting performance measurement type with:

-	no Filters: DRB.PdcpF1DelayDl

-	single Filter with single value: DRB.PdcpF1DelayDl_QOS2

-	single Filter with multiple values: DRB.PdcpF1DelayDl_QOS1|3-9

-	single Filter with aggregated values: DRB.PdcpF1DelayDl_QOS[2-5]

*** START OF NEXT CHANGE ***

## 4.3	Measurement type naming

The measurement type is constructed as follows:

-	"<familyName>.<measurementName>.<Subcounter1>.<…>.<SubcounterM>_<Filter1>_…_<FilterN>" for a list of measurement results of a combination of Subcounters and Filters

-	"<familyName>.<measurementName>.<Subcounter1>.<…>.<SubcounterM>" for a list of measurement results of Subcounters only

-	"<familyName>.<measurementName>_<Filter1>_…_<FilterN>" for an aggregated measurement result or a list of measurement results of Filters only

-	"<familyName>.<measurementName>" for a measurement result without any Subcounter or Filter

-	"<familyName>" for a list of measurement results of all applicable measurementNames without any Subcounter or Filter

*** START OF NEXT CHANGE ***

## 4.x	Measurement result

The measurement result(s) shall be clearly defined in each performance measurement definition, including its type and value range.

A single measurement result is expected for each performance measurement without subcounter or filter. When a subcounter or filter is specified, a list of measurement results corresponding to each applicable value shall be provided. If an aggregated filter value is specified, the individual measurement results corresponding to each applicable value shall be aggregated into a single aggregated measurement result.

The followings are examples of possible resulting performance measurement outputs:

-	no subcounter or filter: DRB.AirIfDelayDl

<measTypes>

DRB.AirIfDelayDl

</measTypes>

<measValue measObjLdn="GnBFunction=0,Cell=Gbg-994">

<measResults> R1 </measResults>

<!-- R1 is the only measurement result -->

</measValue>

-	single subcounter: DRB.AirIfDelayDist.Bin

If only 2 Bin values are available, the output would be structured as a list of 2 measurement results:

<measTypes>

DRB.AirIfDelayDist.Bin1

DRB.AirIfDelayDist.Bin2

</measTypes>

<measValue measObjLdn="GnBFunction=1,Cell=Gbg-995">

<measResults> R1 R2 </measResults>

<!-- R1 is the measurement result for BIN = 1 -->

<!-- R2 is the measurement result for BIN = 2 -->

</measValue>

-	single subcounter and single Filter with multiple values: DRB.AirIfDelayDist.Bin_QOS3-5

In this case, the Bin is the first dimension. And the QOS is the second dimension. If only 2 Bin values are available, the output would be structured as a list of 2x3 measurement results:

<measTypes>

DRB.AirIfDelayDist.Bin1_QOS3

DRB.AirIfDelayDist.Bin1_QOS4

DRB.AirIfDelayDist.Bin1_QOS5

DRB.AirIfDelayDist.Bin2_QOS3

DRB.AirIfDelayDist.Bin2_QOS4

DRB.AirIfDelayDist.Bin2_QOS5

</measTypes>

<measValue measObjLdn="GnBFunction=2,Cell=Gbg-996">

<measResults>R1 R2 R3 R4 R5 R6</measResults>

<!-- R1 is the measurement result for BIN = 1, QoS = 3 -->

<!—- R2 is the measurement result for BIN = 1, QoS = 4 -->

<!—- R3 is the measurement result for BIN = 1, QoS = 5 -->

<!-- R4 is the measurement result for BIN = 2, QoS = 3 -->

<!—- R5 is the measurement result for BIN = 2, QoS = 4 -->

<!-- R6 is the measurement result for BIN = 2, QoS = 5 -->

</measValue>

-	single subcounter and single Filter with aggregated values: DRB.AirIfDelayDist.Bin_QOS[3-5]

If only 2 Bin values are available, the output would be structured as a list of 2 measurement results:

<measTypes>

DRB.AirIfDelayDist.Bin1_QOS[3-5]

DRB.AirIfDelayDist.Bin2_QOS[3-5]

</measTypes>

<measValue measObjLdn="GnBFunction=3,Cell=Gbg-997">

<measResults>R1 R2</measResults>

<!-- R1 is the measurement result for BIN = 1 and QoS>= 3 and QoS <= 5. -->

<!-- R2 is the measurement result for BIN = 2 and QoS>= 3 and QoS <= 5. -->

</measValue>

*** END OF CHANGE ***