3GPP TSG-SA5 Meeting #162 	S5-253238

Stor-Göteborg, Sweden, 25th August 2025 - 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.622 | CR | 0574 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.622 Corrections on Triggering Event |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_Trace_MDT_QoE-OAM |  |  |  |  |  | Date: |  |  | 2025-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to 3GPP TS 32.422, the Triggering Event is a conditionally mandatory parameter used to initiate or stop a Trace Recording Session in the core network. In contrast, within RAN nodes (UTRAN, E-UTRAN, and NG-RAN), no triggering events are defined. The Trace Recording Session is always aligned with the Trace Session, and therefore the concept of a Triggering Event does not apply in the RAN context. However, 3GPP TS 28.622 currently specifies triggeringEvents as a mandatory parameter in the TraceConfig data structure. This creates an inconsistency with the behavior described in TS 32.422 for RAN deployments. This inconsistency warrants clarification or correction to ensure proper alignment between the trace configuration model in TS 28.622 and the trace control procedures described in TS 32.422. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Make triggeringEvents as a conditional mandatory parameter in TraceConfig |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specification, which may lead to incorrect implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.30.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

### 4.3.57	TraceConfig <<dataType>>

#### 4.3.57.1	Definition

This <<dataType>> defines the configuration parameters of IOC TraceJob which are specific for Trace or any combination of Trace.

The attribute listOfNeTypes specifies the network elements to be traced. The optional attribute listOfInterfaces allows to specify the individual interfaces of the network elements to be recorded.

The attribute traceDepth allows to configure the level of detail of the information which shall be recorded.

For trace the reporting is event based, where the triggering event is configured with attribute triggeringEvent. For each triggering event the first and last message (start/stop triggering event) to record  are specified.

#### 4.3.57.2	Attributes

| Attribute name | S | isReadable | isWritable |  | isInvariant |  | isNotifyable |
| --- | --- | --- | --- | --- | --- | --- | --- |
| listOfInterfaces | O | T | T | F |  | T |  |
| listOfNeTypes | CM | T | T | F |  | T |  |
| traceDepth | M | T | T | F |  | T |  |
| triggeringEvents | CM | T | T | F |  | T |  |

#### 4.3.57.3	Attribute constraints

| Name | Definition |
| --- | --- |
| listOfNeTypes | This attribute shall be present only for Trace with Signalling Based Activation |
| triggeringEvents | This attribute shall be present if Trace in core network is supported. |

#### 4.3.57.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions.

*** END OF CHANGE ***