3GPP TSG-SA WG6 Meeting #68	S6-253785

Stor-Göteborg, Sweden 25th – 29th August 2025	(revision of S6-2523340, 253532, 253704, 253724)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0747 | rev | 4 | Current version: | 19.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on EES determination based on UE location and route Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-App |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It is not clear how EEC determine which EES to connect to when EEC receives a list of EES and the EEC is not aware of the UE serving Satellite ID, that is the EEC only provide “predicted/expected UE location, Prediction expiration time” to the ECS. Thus, the description of how the EEC determine which EES to connect to is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description of the EEC determines the EES based on UE current location and corresponding current time and received EES Satellite coverage availability information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EES determination based on UE location and route Procedure will remain incorrect and unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

11.1.2	EES determination based on UE location and route Procedure

This procedure and the corresponding EES profile and Service provisioning request are applicable for EDN deployment with gNB on satellite.

A satellite EDN may be deployed as described in Annex A.2.5. The service provisioning procedure for satellite EDN is the same as clause 8.3.3.2.2 and clause 8.3.3.2.3 with the following clarifications:

-	When EEC sends the service request message to the ECS and the request message contains the UE location, predicted/expected UE location, Prediction expiration time, minimum availability time duration for satellite access by the EEC (UE), the ECS determines a list of EES where the Satellite coverage availability information in EES profile matches the UE location, predicted/expected UE location, Prediction expiration time, minimum availability time duration for satellite access by the EEC (UE) and EES Satellite coverage availability information. Alternatively, the ECS may query an external server with the UE current location and UE predicted/expected location to obtain a list of Satellite IDs and Trajectory IDs corresponding to the UE location and the UE trajectory, then the ECS determines a list of EES based on the Satellite IDs and Trajectory IDs in EES profiles by matching with the Satellite IDs and Trajectory IDs obtained from an external server for the UE. The “Expected AC Geographical Service Area” in the AC profile, as specified in clause 8.2.2, is used by the ECS to request the external server (managed by the satellite service provider or operator) for such satellite IDs and trajectory IDs.

-	When ECS sends the service provisioning response which contains a list of EES information which EES is deployed on MEO or LEO satellites, the EEC determines the EES where the Satellite ID corresponding to the EES in the EDN configuration information matches the UE serving Satellite ID. If the UE serving satellite ID is not available, the EEC may determine the EES based on available information e.g., UE current location, corresponding current time, received ASCAI and how long UE can stay under coverage, etc.

-	In addition, when a serving satellite is providing EES and EAS, and due to discontinuous coverage, the same satellite comes back for serving, then the EEC connects to the same EES and EAS (i.e. to which the EEC had previously connected to) based on available information e.g., UE current location, corresponding current time, received ASCAI and how long UE can stay under coverage, etc without needing to discover EES and EAS endpoints again.

This procedure is also utilized by the ACR procedures specified in clause 8.8 where EES determination is needed during service provisioning.

* * * End of Changes * * * *