| 3GPP TR 23.700-15 V1.0.0 (2025-12) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Use of sensing results for Vertical Applications; (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	9

3.3	Abbreviations	9

4	Key issues	9

4.1 	Key issue #1: Application enablement architecture enhancements to support utilization and exposure of sensing results	9

4.1.1	Description	9

4.1.2	Open Issues	10

4.2	Key issue #2: Enhance UAV service utilizing sensing results	10

4.2.1	Description	10

4.2.2	Open Issues	11

4.3	Key issue #3: Key issue on use of sensing results for spatial maps	11

4.3.1	Description	11

4.3.2	Open Issues	11

4.4	Key issue #4: utilizing sensing results for HD map in V2X	12

4.4.1	Description	12

4.4.2	Open issues	12

5	Architectural Requirements and Assumptions	12

6	Solutions	12

6.1	Mapping of solutions to key issues	12

6.2	Solution #1: Functional Architecture to Support Sensing Application Relevant Services	13

6.2.1	Solution Description	13

6.2.1.0	General	13

6.2.1.1	Functional Model	13

6.2.1.1.1	General	13

6.2.1.1.2	Network Functional Model	14

6.2.1.2	Functional Entities Description	14

6.2.1.2.1	General	14

6.2.1.2.2	SEAL Sensing Enabler client	14

6.2.1.2.3	SEAL Sensing Enabler server	14

6.2.1.3	Reference Points Description	14

6.2.1.3.1	General	14

6.2.2	Architecture impacts	14

6.2.3	Solution evaluation	15

6.3	Solution #2: Support the sensing results exposure based on the sensing subscription request	15

6.3.1	Solution description	15

6.3.2	Procedures	15

6.3.3	Solution evaluation	17

6.4	Solution #3: Support of exposure of sensing results	17

6.4.1	Procedure of exposure of sensing results	17

6.4.1.1	Procedure of supporting sensing analysis for ADAES	18

6.4.2	Solution evaluation	20

6.5	Solution #4: Enhancements of UAV services utilizing the sensing results	20

6.5.1	Solution description	20

6.5.1.1	Procedure of detecting UAVs utilizing sensing results	20

6.5.2	Architecture Impacts	21

6.5.3	Solution evaluation	21

6.6	Solution #5: sensing based tracking dynamic UAVs	21

6.6.1	Solution description	21

6.6.2	Procedures	21

6.6.2.1	Subscription for host UAV dynamic information	22

6.6.2.2	 UAE server management of dynamic area for sensing	22

6.6.2.3	UAE server obtaining dynamic information from sensing results	22

6.6.2.4	Notification of host UAV dynamic information	23

6.7	Solution #6: Sensing based DAA service	24

6.7.1	Solution description	24

6.7.2	Procedures	24

6.7.2.1	Configuration of sensing based DAA policies to the UAE layer	24

6.7.2.1.1	Management of DAA support configuration	24

6.7.2.1.2	Sensing based DAA support configuration procedure	25

6.7.2.2	Sensing based DAA event reporting	25

6.8	Solution #7: Sensing Coverage Information Exposure for Supporting UAV Services	26

6.8.1	Solution Description	26

6.8.1.0	General	26

6.8.1.1	Procedure	27

6.8.1.2	Information Flows	27

6.8.2	Architecture impacts	27

6.8.3	Solution evaluation	28

6.9	Solution #8: Use Sensing Results for Creating Spatial Map	28

6.9.1	Solution Description	28

6.9.1.0	General	28

6.9.1.1	Procedure	28

6.9.1.2	Information Flows	29

6.9.2	Architecture impacts	29

6.9.3	Solution evaluation	29

6.10	Solution #9: high level architecture and procedures for use of sensing results for spatial map	29

6.10.1	Description	29

6.10.2	Procedures	30

6.10.2.1	Create spatial map with sensing capabilities	30

6.10.2.2	Spatial map subscription and notification with sensing capabilities	31

6.10.3	Solution evaluation	33

6.11	Solution #10: Sensing results for spatial maps	33

6.11.1	Solution description	33

6.11.2	Procedures	33

6.11.2.1	Impact to Create spatial map procedure	33

9.3.1.2	Procedure	34

9.3.1.3	Information flows	35

9.3.1.3.1	Create spatial map request	35

6.11.2.2	Impact to Update spatial map procedure	36

9.3.4	Update spatial map	37

9.3.4.1	General	37

9.3.4.2	Procedure	37

9.3.4.3	Information flows	37

9.3.4.3.1	Update spatial map request	37

6.11.3	Solution evaluation	38

6.12	Solution #11: Use Sensing Results to Enhance HD Map for V2X Services	38

6.12.1	Solution Description	38

6.12.1.0	General	38

6.12.1.1	Procedure	39

6.12.1.2	Information Flows	40

6.12.2	Architecture impacts	40

6.12.3	Solution evaluation	40

6.13	Solution #12: Sensing-based event trigger monitoring procedure	40

6.13.1	Solution Description	40

6.13.2	Procedures	41

6.14	Solution #13: Sensing Requests Process and Sensing Result Reuse	42

6.14.1	Solution Description	42

6.14.1.1	General	42

6.14.1.2	Procedure	42

6.14.1.3	Information Flows	43

6.14.2	Architecture impacts	43

6.14.3	Solution evaluation	43

6.15	Solution #14: Sensing UAVs combining location and sensing results	43

6.15.1	Solution description	43

6.15.1.1	Procedure for sensing UAVs with a UE module	43

6.15.2	Architecture Impacts	44

6.15.3	Solution evaluation	45

6.16	Solution #15: Presence Detect of UAV in No Drone Zone	45

6.16.1	Solution Description	45

6.16.1.1	General	45

6.16.1.2	Procedure	45

6.16.1.3	Information Flows	46

6.16.2	Architecture impacts	46

6.16.3	Solution evaluation	46

6.17	Solution #16: Converged Trajectory Tracking	46

6.17.1	Solution Description	46

6.17.1.1	General	46

6.17.1.2	Procedure	46

6.17.1.3	Information Flows	48

6.17.2	Architecture impacts	48

6.17.3	Solution evaluation	48

6.18	Solution #17: Sensing enabler service procedure for spatial map object detection and tracking	48

6.18.1	Solution description	48

6.18.1.0	General	48

6.18.1.1	Procedure	48

6.18.1.2	Information Flows	50

6.18.1.2.1	Sensing enabler service subscribe request	50

6.18.1.2.2	Sensing enabler service subscribe response	52

6.18.1.2.3	Sensing enabler service notification	53

6.18.2	Architecture impacts	53

6.18.3	Solution evaluation	53

7	Deployment scenarios	53

7.1	General	53

7.x	Deployment model #x: <Title>	54

8	Business Relationships	54

9	Overall evaluation	54

10	Conclusions	54

10.1	General conclusions	54

10.2	Conclusions of key issue #x	54

Annex A (informative): Change history	55

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

# x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

5G wireless sensing is a technology to acquire information about characteristics of the environment and/or objects within the environment. It has significant potential to enhance a large number of vertical applications, so it is important to study how to utilize sensing results in existing vertical application enablement services in alignment with requirements specified in 3GPP TS 22.137 [3], existing SEAL architecture specified in 3GPP TS 23.434 [2] and the study on Integrated Sensing and Communication specified in 3GPP TR 23.700-14 [7].

This technical report will identify key issues, corresponding application architecture requirements and related solutions with recommendations for normative work.

# 1	Scope

The present document is a technical report which identifies potential enhancements to  application enablement services by utilizing Sensing results and proposes architectural requirements, key issues, and solutions for the potential application enablement architecture enhancements.

The study takes into consideration the existing SEAL architecture specified in 3GPP TS 23.434 [2] to investigate how to support the potential enhanced application enablement services by utilizing Sensing results.

Furthermore, the study also provides recommendations for the normative work.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[3]	3GPP TS 22.137, "Service requirements for Integrated Sensing and Communication; Stage 1".

[4] 		3GPP TS 23.255, "Application layer support for Uncrewed Aerial System (UAS); Functional architecture and information flows;".

[5]	3GPP TS 23.286, "Application layer support for Vehicle-to-Everything (V2X) services; Functional architecture and information flows; "

[6]	3GPP TR 22.837: "Feasibility Study on Integrated Sensing and Communication (Release 19)".

[7]	3GPP TR 23.700-14: "Study on Integrated Sensing and Communication (Release 20)".

[8]	3GPP TS 23.437: "Service Enabler Architecture Layer for Verticals (SEAL); Spatial map and Spatial anchors".

[9]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications ".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

For the purposes of the present document, the terms given in 3GPP TS 22.137 [3] apply.

Sensing result

For the purposes of the present document, the terms given in 3GPP TS TS 23.255 [4] apply.

UAS Service Supplier (USS)

UAV

For the purposes of the present document, the terms given in 3GPP TS TS 23.286 [5] apply.

V2X service

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

SEAL	Service Enabler Architecture Layer for Verticals

UAE	UAS Application Enabler

UAS	Uncrewed Aerial System

UAV	Uncrewed Aerial Vehicle

USS	UAS Service Supplier

V2X	Vehicle-to-Everything

VAE	V2X Application Enabler

VAL	Vertical Application Layer

# 4	Key issues

## 4.1 	Key issue #1: Application enablement architecture enhancements to support utilization and exposure of sensing results

### 4.1.1	Description

3GPP TR 22.837[6] and TS 22.137[3] specify a set of service requirements about sensing which may have potential impacts on the application enabler layer.

-	Subject to operator’s policy, the 5G network shall be able to provide secure means to report sensing results to a trusted third-party requesting information about a target object when specific requested conditions are met;

-	Subject to operator’s policy and regulation, the 5G system shall be able to provide secure means for a trusted third-party to receive sensing results with contextual information;

-	Subject to user’s consent, regulation and operator’s policy, the 5G network may provide secure means to expose to a trusted third-party the combined sensing result derived from the joint processing of the 3GPP sensing data and non-3GPP sensing data.

-	Subject to operator’s policy, the 5G network may provide secure means for the operator to expose information towards trusted third-party on whether a given sensing service is available and the estimated quality of the given service for a certain geographic area and time.

A vertical application may provide the application enabler layer with specific information defining the conditions for the situation of interest. When such conditions are met, the application enabler layer shall be able to provide the vertical application with a report together with the relevant information.

The information required to define the situation of interest may include, but is not limited to, basic parameters such as a specific geographical area, state information, or conditions (e.g., speed, distance). Furthermore, depending on the requirements of the vertical application, more detailed and complex conditional information may be specified, such as indicators of a disaster in a specific area, tracking of the movement of a target UE over a defined time period together with environmental status information around the UE, or detection of unauthorized UE entry into a specific area, etc.

Thus, in 5G wireless sensing each vertical application may specify a single sensing service requirement (e.g. object detection) or a combination of multiple requirements (e.g. detect-and-avoid, no-transmit zone enforcement), and request them to the application enabler layer. Therefore, the application enabler layer shall be able to handle these application requirements, and shall dynamically provide the necessary capabilities to fulfil sensing requirements.

In addition, in the present document, KI#2, KI#3, and KI#4 address the utilization of sensing results for UAV, spatial map, and V2X services, respectively. It is necessary to study whether common function(s) on utilizing sensing result can be captured from these specific services.

### 4.1.2	Open Issues

Based on the above analysis, the following aspects need to be addressed in the application enabler layer:

1.	Whether and how to enhance the application enablement architecture and related functions to support the mapping of sensing service requirements of vertical applications.

2.	Whether and how the existing exposure mechanisms (e.g., subscription, notification, etc) defined in the SEAL architecture and functions are sufficient to meet vertical application requirements.

3.	Whether and how to process the collected sensing results (e.g., sensing exposure from the core network), to generate application enabler layer enhanced sensing results, and to expose them to vertical applications.

4.	Whether and how to resolve conflicts in sensing service requirements among different vertical applications or under resource limitations.

5.	Whether new application enabler architecture is needed to support the above functions.

6.	Whether common function(s) can be identified when working on the solutions related to the utilization of sensing result for specific services, e.g. UAV (KI#2), spatial map (KI#3), and V2X (KI#4) services.

NOTE:	The open issues on utilization of sensing result for specific services  (e.g., UAV, V2X, Metaverse, etc) will be covered in separate KIs.

## 4.2	Key issue #2: Enhance UAV service utilizing sensing results

### 4.2.1	Description

3GPP TS 22.137 [3] has specified sensing service requirements about UAVs. Such as "The 5G system shall be able to provide sensing service to detect, and/or track one or more objects (e.g., UAVs, birds) and the environment around the object(s). "

In Rel-19, the application enabler (i.e. UAE) has been developed to support UAS which specified in 3GPP TS 23.255 [4] in which communications between UAVs, monitoring of UAV location deviation and reporting of UAV events have been studied. Especially for Detect And Avoid (DAA) service, which is to detect the potential collision between UAVs, is a key service of UAS to accomplish the safe and efficient UAV management. However, the DAA service depends on the PC5 communication, and the current situation is that the PC5 communication is not large-scale commercially used in UAV industry and even some UAVs are not equipped with 3GPP UE modules (e.g. USIM).

Furthermore, according to clause 7.8 of 3GPP TS 23.255 [4], an UAE service of tracking dynamic UAVs in an application defined area relative to a host UAV is specified. For this service, the UAE layer needs to provide the dynamic information (i.e. other UAVs’ location information) to the application layer and/or the host UAV. Currently, this service is performed by invoking location management service in SEAL layer. The location management service localizes an UAV only when the UE is equipped an UE module (e.g. USIM), and it is impossible to obtain the location information of an UAV without such UE module.

In addition, it is essential to manage UAV flight in No Drone Zone. This requires that once UAVs enter No Drone Zone, the intrusion can be detected, triggering notification immediately, e.g., sending warning to the UAV controller or UTM.

In fact, sensing results can be utilized to assist in detecting UAV, tracking UAV and avoiding the collision when there is any object during UAV trajectory or when UAV is entering a No Drone Zone, especially when the UAV is without the UE module. So, it would make sense to study how to utilize related sensing results to support the DAA, UAV trajectory tracking services, UAV flight management in No Drone Zone.

### 4.2.2	Open Issues

Based on the above analysis, the following open issues need to be studied:

-	How to enhance the UAS application enabler to support enhanced trajectory tracking for UAVs utilizing the sensing results.

-	How to enhance the UAS application enabler to support enhanced DAA service/functionalities for UAVs utilizing the sensing results.

-	How to enhance the UAS application enabler to support No Drone Zone avoidance (detect and notify) utilizing the sensing results.

## 4.3	Key issue #3: Key issue on use of sensing results for spatial maps

### 4.3.1	Description

This KI aims to study how to use sensing results to manage object information in spatial maps. Objects in spatial maps can be stationary or mobile. As a result, tracking mobile objects within spatial maps can be more efficient with the use of sensing technology. A sensing service can be triggered to generate sensing results for an area of interest in a spatial map and the sensing results can be used to determine and identify objects within the spatial map. As an object moves, sensing results can be used to track the object and allow a spatial map server the ability to keep the spatial map current. The information obtained from sensing results for the spatial map can then be exposed to third-party consumers.

In Rel-19, the processed sensor data is stored at VAL layer (e.g., in database at VAL layer) and the SEAL SM server gets access to such database to create a spatial map via application specific way which is out of scope.

### 4.3.2	Open Issues

The key issue will study:

1.	Whether and how the enabler layer can use sensing results for determining the presence of objects (e.g. humans, animals, and vehicles) within an area of interest in a spatial map.

2.	Whether and how the enabler layer can associate a sensing result with an object within a spatial map.

3.	Whether and how the enabler layer can localize objects and detect object movements within a spatial map from sensing results.

4.	How the enabler layer can support SM server with creating SM.

NOTE:	In Rel-20, exposure of sensing results (with or without the sensing contextual information) to support sensing services will be provided by 3GPP core network to enabler layer.

## 4.4	Key issue #4: utilizing sensing results for HD map in V2X

### 4.4.1	Description

According to application layer support for Vehicle-to-Everything (V2X) services specified in 3GPP TS 23.286 [5], the V2X application specific server can be responsible for managing HD maps and providing the HD map information to the V2X application specific client on V2X UE. As per a proximity range set by the application layer, the VAE layer support providing the dynamic information (i.e. location information) required for HD maps management to the V2X application specific server. During this procedure, VAE needs to invoke location management service to get dynamic UEs list and further obtain their location. However, in current transportation system, there are many traditional vehicles which may be not equipped with UE module, or GNSS module, so it is difficult to localize their location by location management service provided by SEAL. Moreover, the environment information around vehicle (e.g., pedestrian/animal objects) also is important for safe operation, but there is no an efficient method to obtain those information.

According to the sensing service requirement specified in 3GPP TS 22.137 [3], the 5G system shall be able to provide sensing service to detect, and/or track one or more objects and the environment around the object(s). This service could be used to enhance HD map service to provide the location information of traditional vehicles and environment information around host vehicle to improve system safety.

Hence, it would make sense to study how to utilize sensing results for HD map in V2X.

### 4.4.2	Open issues

Based on the above analysis, the following open issue need to be studied:

1.	How to enhance VAE layer to provide enhanced HD map including traditional vehicles information with considering sensing results.

2.	How to enhance VAE layer to provide enhanced HD map including environment information (e.g. pedestrian/animal objects, etc) with considering sensing results.

# 5	Architectural Requirements and Assumptions

This clause provides the architectural requirements and assumptions.

# 6	Solutions

## 6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 | x |  |  |  |
| Sol #2 | x |  |  |  |
| Sol #3 | x |  |  |  |
| Sol #4 |  | x |  |  |
| Sol #5 |  | x |  |  |
| Sol #6 |  | x |  |  |
| Sol #7 |  | x |  |  |
| Sol #8 |  |  | x |  |
| Sol #9 |  |  | x |  |
| Sol #10 |  |  | x |  |
| Sol #11 |  |  |  | x |
| Sol #12 | x |  |  |  |
| Sol #13 | x |  |  |  |
| Sol #14 |  | x |  |  |
| Sol #15 |  | x |  |  |
| Sol #16 |  | x |  |  |
| Sol #17 |  |  | x |  |
| Sol #... |  |  |  |  |

## 6.2	Solution #1: Functional Architecture to Support Sensing Application Relevant Services

### 6.2.1	Solution Description

#### 6.2.1.0	General

There are some common requirements from difference sensing application relevant services, for example, sensing data/result collection, sensing result aggregation, sensing result exposure. Enhancements to the application enablement layer architecture are needed to introduce common functionalities to fulfil these comment requirements.

The following clauses specify generic functional model of SEAL Sensing Enabler for support application relevant sensing services.

#### 6.2.1.1	Functional Model

##### 6.2.1.1.1	General

The functional architecture enhancement for the SEAL Sensing Enabler is based on the generic functional model specified in clause 6.2 of 3GPP TS 23.434 [2]. It is organized into functional entities to describe an architecture enhancement which addresses the support for sensing aspects for vertical applications.

##### 6.2.1.1.2	Network Functional Model

Figure 6.2.1.1.2-1: Network functional model

Figure 6.2.1.1.2-1 illustrates the network functional model with SEAL Sensing Enabler. In the vertical application layer, the VAL client communicates with the VAL server over VAL-UU reference point. The SEAL functional entity with sensing enabler function on the server is grouped into SEAL Sensing Enabler server. The SEAL Sensing Enabler server consists of a common set of services and reference points. The SEAL Sensing Enabler server offers its services to the vertical application layer (VAL), VAE/UAE, and other SEAL servers.

NOTE:	In this release, the SEAL sensing client is not used for sensing data collection.

Editor's Note:	Whether the SEAL sensing client is needed and the usage of it are FFS.

#### 6.2.1.2	Functional Entities Description

##### 6.2.1.2.1	General

The SEAL Sensing Enabler functional entities with sensing application relevant service functions are described in the following subclauses.

##### 6.2.1.2.2	SEAL Sensing Enabler client

The SEAL Sensing Enabler client interacts with the SEAL Sensing Enabler server.

NOTE:	In this release, the SEAL sensing client is not used for sensing data collection.

##### 6.2.1.2.3	SEAL Sensing Enabler server

The SEAL Sensing Enabler server functional entity provides for sensing application relevant services supported within the vertical application layer.

#### 6.2.1.3	Reference Points Description

##### 6.2.1.3.1	General

The reference points for the functional model for sensing application relevant service are described in the following subclauses.

Editor’s Note:	The description of reference points is FFS.

### 6.2.2	Architecture impacts

Editor's Note:	The architecture impacts of the solution is FFS.

### 6.2.3	Solution evaluation

Editor's Note:	The evaluation of the solution is FFS.

## 6.3	Solution #2: Support the sensing results exposure based on the sensing subscription request

### 6.3.1	Solution description

This solution aims to address the issues identified in Key Issue 1 and provides a possible procedure to perform the sensing results exposure.

Editor's note: This solution is built on functional architecture described in solution X for key issue 1. The sensing capabilities in the enabler layer may be provided by a separate SEAL sensing server or directly by the specific SEAL/VAL enabler server. The decision on whether a separate SEAL sensing server is needed will be based on the evaluation of the solutions and FFS.

In this solution, after acquiring the sensing requirements from the sensing service consumer, a Sensing enabler server will send sensing request to a 5GC NF(e.g. NEF). After it receives the sensing results from the 5GC NF, it determines whether the sensing requirements (e.g. the required accuracy of sensing result) are met, if not, it may further select other sensing service supplier(s) to acquire more sensing results, then it may generate enhanced sensing results based on the received sensing results from the 5GC NF and other sensing service supplier(s) and expose them to the sensing service consumer.

In this solution, the sensing service supplier which can provide sensing results can be another 5GC NF or vertical application server.

Editor's note:	Whether the sensing service supplier can be 3rd party AF is FFS.

### 6.3.2	Procedures

The high-level procedure of sensing service exposure is shown in Figure 6.3.2-1.

Pre-conditions:

1.	The Sensing enabler server has acquired sensing capability information of one or more Sensing Service Supplier(s).

2.	The Sensing enabler server has acquired Sensing Authorization information for Sensing Service from CN.

Figure 6.3.2-1: Sensing service exposure procedure

1.	The VAL Server/Client acting as sensing consumer performs sensing service subscription procedure by sending sensing service subscription request and receiving sensing service subscription response. During the sensing service subscription procedure, the Sensing enabler server can acquire the sensing requirements of the VAL Server/Client such as VAL ID, Sensing Service Area of interest, Sensing service type, Sensing target object type, Accuracy requirements, QoS requirements and so on. The Sensing enabler server performs authorization check and determines whether the Sensing Service Area of interest, sensing service type and Sensing target object type is authorized according to the acquired Sensing Authorization information for Sensing Service from CN. Then it sends sensing service subscription response including the authorization information (e.g.,authorized Sensing Service Area, authorized sensing service type and authorized Sensing target object type) to the sensing consumer. If the sensing consumer is authorized to acquire one-time sensing results reporting, the sensing service subscription response includes the allowed time period of triggering sensing request.

2.	Sensing enabler server sends sensing request to 5GC NF(e.g.,NEF) and receives sensing response. The Sensing enabler server determines the content of the sensing request including the accuracy and QoS requirements in the received sensing service subscription request and the authorized requesting information after authorization check in step1.

Editor's note:	The parameters in the sensing request and response are FFS and should be determined based on SA2’s progress.

3.	Sensing enabler server receives sensing results from 5GC NF(e.g.,NEF) and determine whether the sensing requirements are met.

4.	If the sensing requirements are met,  skip to step 8, if not, the Sensing enabler server selects other Sensing Service Supplier(s) according to the sensing capability information of Sensing Service Supplier(s)  and sensing requirement as received in step1 (e.g. by matching sensing capability information of Sensing Service Supplier to the sensing subscription parameters received from the sensing consumer).

5.	The Sensing enabler server sends sensing request to each selected Sensing Service Supplier and receives sensing response. Different Sensing Service Supplier may be given different sensing request parameters.

Editor's note:	The parameters in the sensing request and response are FFS.

6.	Each Sensing Service Supplier sends sensing results to the Sensing enabler server based on the sensing request  parameters respectively.

7.	The Sensing enabler server further generate enhanced sensing results(e.g.,with higher accuracy) based on the received sensing results from the 5GC NF and other Sensing Service Supplier(s).

8.	The Sensing enabler server sends the (enhanced) sensing results to the VAL Server/Client.

### 6.3.3	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 6.4	Solution #3: Support of exposure of sensing results

This solution addresses the KI#1.

### 6.4.1	Procedure of exposure of sensing results

This solution proposes a new enabler (i.e., Sensing Enabler Server) to support the exposure of sensing results to the consumers (e.g., VAL server). The Sensing Enabler server may interact with other SEAL enabler to get the value-added sensing results.

Figure 6.4.1-1 illustrates the high-level procedure of exposure of sensing results.

Figure 6.4.1-1: Procedure of exposure of sensing results

1.	The VAL server sends a sensing service request to the Sensing Enabler server, including the service type (e.g., UAV detection, dynamic HD map, Vehicle tracking, etc.), service ID, sensing target object information (e.g., UAV ID, UE ID, Vehicle ID), sensing service requirements for the target object (e.g., the target area, sensing accuracy, sensing resolution, maximum sensing service latency, etc.), the reporting conditions (e.g., periodic reporting or event-triggered reporting), the time duration for the sensing, etc.

2.	The Sensing Enabler server checks whether the VAL server is authorized to invoke such request, may be based on e.g. the pre-configurations or operator policies.

3.	If the request is authorized, the SM server may invoke the sensing related service request to the 3GPP CN based on the service request received in step 1 and obtain the sensing results from the 3GPP CN.

Editor's note:	The exposure of sensing results from 3GPP CN depends on SA2’s progress. What will be exposed to the consumers is FFS.

4.	The Sensing Enabler server may interact with other SEAL enablers (e.g., SEALDD, ADAES) to get more sensing results (e.g., the transmission latency, the sensing accuracy analysis) utilizing the obtained sensing results from the 3GPP CN, in case the 3GPP core network will not provide.

-	The Sensing Enabler server may interact with ADAES to get the sensing accuracy analysis and the prediction analysis for the target object utilizing the obtained sensing results from the 3GPP CN, and the details can refer to the clause 6.4.1.1;

-	The Sensing Enabler server may interact with SEALDD as specified in clause 9.7 of 3GPP TS 23.433 [9] to get the e.g., the transmission latency for the target object with UE ID utilizing the obtained sensing results from the 3GPP CN.

Editor's note: Whether the UE ID for the target object will be exposed by 3GPP CN is FFS.

5.	The Sensing Enabler may analyse and aggregate the sensing results obtained in step 3 and step 4 to generate the value-added sensing results (e.g. the statistic results in a location/time/direction/height granularity), and then expose to the VAL server if the results can fulfil the requested sensing service requirements.

6.	The Sensing Enabler sends the value-added sensing results to the VAL server via sensing service response message, including all of target objects and the value-added sensing information.

#### 6.4.1.1	Procedure of supporting sensing analysis for ADAES

Figure 6.4.1.1-1 illustrates the high-level procedure when the Sensing enabler, acting as a consumer, asks for sensing accuracy analytics from ADAES.

Pre-conditions:

-	A-ADRF may have stored the historical sensing accuracy results for the requested sensing objects.

Figure 6.4.1.1-1: Procedure of supporting sensing analysis for ADAES

1.	The consumer (i.e. Sensing enabler) initiates a subscription request to ADAE server for sensing accuracy prediction/stats, including an analytics ID (e.g. "sensing accuracy prediction" or "sensing accuracy sustainability"), an analytics type (e.g. sensing accuracy statistics or prediction for a given location and/or for a given UE). The request may include also the target area, the service ID, the object ID, time validity, accuracy threshold, the obtained sensing results from the 3GPP CN, sensing service requirements (e.g. sensing accuracy, sensing resolution, maximum latency, missed detection probability, false alarm probability, etc.), etc.

2.	The ADAE server authorizes the service request based on e.g. the pre-configurations or the operator policies.

3.	 If the authorization is approved, the ADAE server sends a sensing accuracy analytics subscription response to the consumer.

4.	The ADAE server optionally requests sensing accuracy historical analytics/results from A-ADRF for the corresponding sensing objects or the target service area.

5.	Based on the request, the ADAE server receives the sensing accuracy historical analytics/results from A-ADRF for the corresponding sensing objects or target service area.

6.	The ADAE server utilizes the obtained sensing results from the consumer (i.e. Sensing enabler) and abstracts or correlates the analytics/results from steps 6 and then generates the analytics on the sensing accuracy for the requested sensing objects considering the service requests. Depending on the analytics ID in step 1, the ADAE server can indicate whether the sensing accuracy is sustainable or is predicted to be downgraded or upgraded.

7.	The ADAE server sends the sensing accuracy analytics notification to the consumer.

### 6.4.2	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 6.5	Solution #4: Enhancements of UAV services utilizing the sensing results

### 6.5.1	Solution description

This solution addresses the KI#2.

The following high-level solution principles apply to this solution:

-	A high-level procedure for sensing request and exposure is proposed with a new Sensing enabler in the application enablement layer.

-	The solution assumes the 3GPP core network will expose the sensing results related to UAVs to the AF (e.g., VAL server).

#### 6.5.1.1	Procedure of detecting UAVs utilizing sensing results

Figure 6.5.1.1-1 illustrates the high-level procedure of detecting UAVs utilizing sensing results.

Pre-condition:

-	The following Sensing enabler is a new enabler that implements the sensing functions in the application enablement layer.

Figure 6.5.1.1-1: Procedure of detecting UAVs utilizing sensing results

1.	The VAL server sends a UAV detect request to the UAE to detect the UAVs in a certain area, including the service type (i.e., UAV detection, UAV tracking), service ID, sensing service requirements for the detected UAVs (e.g., location accuracy, velocity, latency, sensing resolution, etc.), the target area, the reporting conditions (e.g., periodic reporting or event-triggered reporting), the reporting time intervals if the request is periodic, time duration, optional tracked object information (e.g., UAV ID, normal flight trajectory) if the object needs to be tracked, etc.

2.	The UAE checks whether the VAL server is authorized to invoke such request based on e.g. the pre-configurations or the operator policies.

3.	If the request is authorized, the UAE sends the UAV related sensing results request to the sensing enabler to ask for the UAV related sensing results.

4.	The sensing enabler may invoke the sensing related service request to the 3GPP CN based on the service requests received in step 1 and obtain the sensing results from the 3GPP CN via NEF if the SM server is an untrusted AF or interact with the SF entity in 5GC directly as a trusted AF.

Editor's note:	The exposure of sensing results from 3GPP CN depends on SA2’s progress. What will be exposed to the consumers is FFS.

5.	Upon received the exposure from network layer, the sensing enabler may determine the detected UAVs that entering the target area and then the sensing enabler may interact with LMS to track the detected UAVs with UE modules and obtain the location trajectory (including the velocity, direction, altitude, etc.) for them in the requested time duration as specified in clause 9.3.5 of 3GPP TS 23.434 [2].

6.	The sensing enabler aggregates and combines the sensing results obtained in step 4 and step 5 to determine all of UAVs entering the target area as well as their flight trajectories, and then the sensing enabler compares the detected UAV flight trajectory with normal flight trajectory included in step 1 to identify if the UAVs entering the target area are authorized or not.

7.	The sensing enabler notifies the UAE for the UAV related sensing results, including all UAVs entering the target area and the unauthorized UAVs that not allowed entering the target area.

8.	The UAE sends the analysed sensing results to the VAL server via UAV detect response message, including all UAVs entering the target area and the unauthorized UAVs that not allowed entering the target area.

### 6.5.2	Architecture Impacts

This solution proposes a new sensing enabler to support the sensing service.

For the new architecture for the sensing enabler, please check the solution#1 for the KI#1.

### 6.5.3	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 6.6	Solution #5: sensing based tracking dynamic UAVs

### 6.6.1	Solution description

This solution intends to solve the first open issue of key issue #2. This solution is to enhance tracking dynamice UAV service by utilizing sensing results.

### 6.6.2	Procedures

This feature utilizes the following procedures:

-	Step 1: UAS Application Specific Server or the host UAV subscription for host UAV’s dynamic information with UAE server.

-	Step 2: UAE server tracking host UAV’s UE location with support from SEAL’s location management server.

-	Step 3: UAE server management of dynamic area for sensing

-	Step 4: UAE server obtaining dynamic information from sensing results

-	Step 5: UAE server notification of host UAV’s dynamic information to the UAS Application Specific Server and/or to the host UAV.

NOTE:	Step 1 and Step 2 are same to the existing procedures defined in clause 7.8.2.1 in 3GPP TS 23.255 [4]. Step 5 is enhanced to include UVA dynamic information obtained by sensing results based on the procedure in 7.8.2.4 in 3GPP TS 23.255 [4].

#### 6.6.2.1	Subscription for host UAV dynamic information

This procedure intends to obtain the host UAV dynamic information, the procedure in clause 7.8.2.1 in 3GPP TS 23.255 [4] is reused.

#### 6.6.2.2	 UAE server management of dynamic area for sensing

Figure 6.6.2.2-1 describes the procedure for management of dynamic area for sensing.

Pre-condition:

-	UAE server 1 has received an updated location of the host UAV as per procedure specified in 3GPP TS 23.434 [5].

Figure 6.6.2.2-1: Management of dynamic area for sensing

1.	Dynamic area for sensing creation/update is triggered (e.g. notified of the UE location of host UAV or notified a location update) via the step 4 in clause 7.8.2.1 in 3GPP TS 23.255 [4] for the UAV ID of the host UAV.

2.	UAE server 1 determines the dynamic area for sensing based on the location of host UAV and the proximity range defined by application by its implementation.

#### 6.6.2.3	UAE server obtaining dynamic information from sensing results

Figure 6.6.2.3-1 describes the procedure of obtaining dynamic information from sensing results.

Pre-condition:

-	UAE server 1 is configured with NEF information of their supported region of operation.

Figure 6.6.2.3-1: Obtaining dynamic information from sensing results

1.	The UAE server 1 determines the NEF(s) operating in the dynamic area for sensing.

2.	The UAE server 1 request and obtain sensing results from 5GC.

3.	The UAE server 1 obtain dynamic information of UAVs by considering all sensing results from 5GC.

Editor's Note: the detail of how to request and obtain sensing results from 5GC depends on SA2.

#### 6.6.2.4	Notification of host UAV dynamic information

Pre-conditions:

-	UAS Application Specific Server has performed subscription as per procedure in clause 7.8.2.1 in 3GPP TS 23.255 [4] with UAE server 1.

-	UAE server 1 has prepared the host UAV dynamic information as per procedure in clause 7.8.2.3.3 in 3GPP TS 23.255 [4].

Figure 6.6.2.4: Notification for host UAV dynamic information

1.	The UAE server 1 sends notification of host UAV dynamic information to the subscribed entity (i.e. UAS Application Specific Server and/or to the subscribed UAE client of the host UAV). The UAE server can aggregate the dynamic information obtained by sensing and other methods. The notification includes the aggregated information of all the objects in the application defined proximity range of the host UAV and the location of the host UAV.

2.	The UAS Application Specific Server or the UAE client of the host UAV updates the host UAV dynamic information with the host UAV dynamic information received in step 1. The UAE client provides the host UAV dynamic information to the UAS Client.

Editor's note: How to define objects in the notification message is FFS. It may be associated to an UAV identity or not.

## 6.7	Solution #6: Sensing based DAA service

### 6.7.1	Solution description

This solution intends to solve the second open issue of key issue #2, i.e. enhance DAA service by utilizing sensing results. This solution is based on the procedures in clause 7.7.2 of 3GPP TS 23.255 [4].

### 6.7.2	Procedures

#### 6.7.2.1	Configuration of sensing based DAA policies to the UAE layer

##### 6.7.2.1.1	Management of DAA support configuration

Figure 6.7.2.1.1-1 illustrates the DAA support management procedure where the UAE server receives an application request for managing the DAA application policy from the UAS application specific server.

Pre-conditions:

1.	The UAV has received its UAS ID from the UAS application specific server.

2.	The UAV has performed the UAS UE registration procedure.

Figure 6.7.2.1.1-1: DAA support management procedure

1.	The UAS application specific server sends to the UAE server a DAA support management request. The request includes the UAV (UAE client) identifier and the DAA application policy. If the procedure is for LDGS assisted DAA, then the request may include a list of UAE client identifiers (LDGS IDs) and/or geographical area so UAE server may select the relevant UAE client(s). The DAA application policy includes parameters for the control of LDGS assisted DAA. DAA application policy could includes parameters for the control of sensing assisted DAA.

2.	The UAE server shall send to the UAS application specific server a DAA support management response with a positive or negative acknowledgement of the request.

3.	The UAE server shall timestamp and store the DAA application policy and execute the DAA configuration according to clause 7.7.2.1.2 and sensing based DAA support configuration procedure according to clause 6.7.2.1.2 if DAA application policy include parameter for the control of sensing assisted DAA.

4.	After execution of DAA configuration, the UAE server shall send a DAA support management complete to the UAS application specific server.

##### 6.7.2.1.2	Sensing based DAA support configuration procedure

Figure 6.7.2.1.2-1 illustrates the sensing based DAA support configuration procedure. This procedure enables the configuration of the UAE server, based on a request from UAS application specific server to configure the sensing based DAA application policy.

Pre-conditions:

1.	UAE server has performed the DAA support management procedure according to clause 6.7.2.1.1.

Figure 6.7.2.1.2-1: Sensing based DAA support configuration procedure

1.	The UAE server obtains and initiates tracking the UAV location from the location management server as specified in 3GPP TS 23.434.

2.	The UAE server determines the dynamic area for sensing based on the location of the UAV and the sensing based DAA application policy.

3.	The UAE server sends request and obtain sensing results of the area determined in Step 2.

4.	The UAE detects UAVs in approaching based on the obtained sensing results and sensing based DAA application policy.

Editor's Note: how to request and obtain sensing results depends on SA2.

#### 6.7.2.2	Sensing based DAA event reporting

Figure 6.7.2.2-1 illustrates the procedure of sensing based DAA event reporting.

Pre-conditions:

1.	UAE server has detected UAVs in approaching as specified in 6.7.2.1.2.

Figure 6.7.2.2-1: Sensing based DAA event reporting

1.	The UAE server shall send a sensing based DAA event information to the UAS application specific server with information about one or more UAVs in approaching as detected and the corresponding timestamp.

2.	The UAS application specific server provides a sensing based DAA event information acknowledge to the UAE server.

Editor's Note 1: UAE server how to consider sensing based detection result and other detection methods jointly to determine DAA event is FFS

Editor's Note 2: The message name of sensing based DAA event information report depends on what information is used to trigger the report and it is FFS

## 6.8	Solution #7: Sensing Coverage Information Exposure for Supporting UAV Services

### 6.8.1	Solution Description

#### 6.8.1.0	General

The Spatial Map Server can benefit from sensing (including 3rd Party sensing services) data or results to build high-quality Spatial Map.

As sensing capabilities rely on the CSP’s infrastructure, which may provide full coverage. And the rollout of the CSP’s sensing service takes time, coverage remains incomplete during the deployment phase. Given that sensing capabilities are critical for the safety of UAVs and automotive systems, comprehensive sensing coverage information is essential for these sectors to develop robust services and ensure operational safety. Currently, there is no mechanism for CSP to expose the sensing coverage information.

The following clauses specify sensing coverage information exposure for supporting UAV services.

#### 6.8.1.1	Procedure

Figure 6.8.1.1-1: Sensing coverage information exposure

1.	The Consumer (VAL Server, SEAL Server) sends check sensing coverage information request to the SEAL Sensing Server.

2.	The SEAL Sensing Server authenticates the request, checks the stored sensing coverage information, and determines the next actions (e.g., to collect sensing coverage information from sensing entities).

3.	The SEAL Sensing Server responds to the VAL server for the check sensing coverage information request.

Editor's Note:	Whether the SEAL sensing server gets initial sensing coverage information from CN between 3 and 4, will be decided based on the stable conclusion of SA2.

4.	The SEAL Sensing Server interacts with the 3rd party sensing server(s) to collect the sensing coverage information of the 3rd party sensing data.

Editor's Note:	Whether the revision of the architecture is needed for the 3rd party sensing server is FFS.

5.	The SEAL Sensing Server aggregates the sensing coverage information from different sensing entities received in step 4.

6.	The SEAL Sensing Server sends check sensing coverage information response to the VAL server with the aggregated sensing coverage information.

Editor's Note:	Further changes to this procedure are FFS.

#### 6.8.1.2	Information Flows

Editor's Note:	The information flows of the solution is FFS.

### 6.8.2	Architecture impacts

Editor's Note:	The architecture impacts of the solution is FFS.

### 6.8.3	Solution evaluation

Editor's Note:	The evaluation of the solution is FFS.

## 6.9	Solution #8: Use Sensing Results for Creating Spatial Map

### 6.9.1	Solution Description

#### 6.9.1.0	General

The Spatial Map Server can benefit from sensing (including 3rd Party sensing services) data or results to build high-quality Spatial Map.

The following clauses specify use combination of sensing results for creating spatial map.

#### 6.9.1.1	Procedure

Pre-conditions:

1.	The Spatial Map Server is triggered to send the sensing result request due to a spatial map create or update procedure as described in clause 6.11.

Figure 6.9.1.1-1: Sensing results for creating spatial map

1.	The Spatial Map Server sends sensing result request for consolidated sensing results from the SEAL Sensing Server.

2.	The SEAL Sensing Server authenticates the request, and determines the sensing entities (e.g., 5GC NFs, 3rd party sensing server) for collection of sensing data.

3.	The SEAL Sensing Server responds to the SM server for the sensing result request.

4.	The SEAL Sensing Server interacts with the NEF to collect the sensing result measured by the 3GPP network. The SEAL Sensing Server determines whether the sensing result fulfils the request from the consumer provided in step 1, and determines whether additional sensing data is needed to enhance the sensing result.

5.	The SEAL Sensing Server interacts with the 3rd party sensing server to collect third party sensing data measured.

Editor's Note:	Whether the revision of the architecture is needed for the 3rd party sensing server is FFS.

6.	The SEAL Sensing Server consolidates the sensing data/results from different sources in steps 4-5 to generate sensing result (e.g. draft spatial map).

7.	The SEAL Sensing Server sends sensing result response to the SM server with the consolidated sensing results.

8.	The Spatial Map Server creates spatial map based on the consolidated sensing result.

#### 6.9.1.2	Information Flows

Editor's Note:	The information flows of the solution is FFS.

### 6.9.2	Architecture impacts

Editor's Note:	The architecture impacts of the solution is FFS.

### 6.9.3	Solution evaluation

Editor's Note:	The evaluation of the solution is FFS.

## 6.10	Solution #9: high level architecture and procedures for use of sensing results for spatial map

### 6.10.1	Description

This solution resolves solution of KI#3 on use of sensing results for spatial maps.

This solution is based on following principles:

1.	No significant changes are made to the spatial map management procedures defined in 3GPP TS 23.437 [8];

2.	A separated SEAL Sensing service server is provided by Enabler layer and interacts with Spatial Map server via a new SEAL-X interference.

The functional model for the SEAL Sensing server is based on the generic functional model specified in clause 6 of 3GPP TS 23.434 [4] and the generic architecture of sensing service concluded in this study. The functional model enhanced to Spatial map for this solution is illustrated in Figure 6.10.1-1.

Figure 6.10.1-1: On-network spatial map functional model for use of sensing

NOTE:	In this solution, SEAL SM client/server in the architecture and procedures refer to SEAL Spatial Map client/server specified in 3GPP TS 23.437 [8].

This solution contains two basic procedures:

1.	Create spatial map with sensing capabilities.

2.	Spatial map subscription and notification with sensing capabilities.

### 6.10.2	Procedures

#### 6.10.2.1	Create spatial map with sensing capabilities

Figure 6.10.2.1-1 below illustrates the procedure for creating a spatial map when sensing capabilities are introduced. For the request from VAL server or SEAL SM client, as a spatial map consumer. SEAL SM server creates a spatial map by utilizing sensing capabilities provided by SEAL Sensing server.

Pre-conditions:

1.	The VAL Server or SEAL SM client is authorized to use APIs provided by the SEAL SM server.

2.	The SEAL SM server can interact with SEAL Sensing server via SEAL-X interface.

3.	The SEAL Sensing server is authorized to use Sensing related capability APIs provided by sensing related function in the 3GPP core network or authorized to use the NEF exposed Sensing related capability.

NOTE:	In this solution, it is assumed that neither the VAL Server nor the SEAL SM Server has sufficient knowledge to understand the detailed use of sensing-related capabilities/parameters provided by the 3GPP core network, whether directly or via the NEF. Alternatively, If the VAL Server/SEAL SM Server knows that sensing service can be provided by SEAL sensing server, the VAL Server/SEAL SM Server may include a sensing indicator in the request. If so, the SEAL SM server will use the sensing indicator to decide whether to utilize sensing capabilities.

Editor's Note:	How the VAL server discovers which SEAL sensing server can provide sensing service it needs is FFS.

Figure 6.10.2.1-1: Create spatial map procedure

1.	The requester (e.g., VAL server or SEAL SM client) sends a Create Spatial Map Request message to the SEAL SM server to create a spatial map. The request includes the same IEs as specified in clause 9.3.1.3.1 of 3GPP TS 23.437 [8]. The VAL Server/SEAL SM Server may include a sensing service configuration as described in clause 6.11.

2.	The SEAL SM server decides to utilize sensing capabilities to assist the creation of the spatial map based on local policy. If SEAL SM server decides to utilize sensing capabilities, it sends a Get Sensing Results Request to SEAL Sensing server. The request includes requester ID, security credentials and three-dimensional area of interest.

Editor's note:	the detailed IEs included in the Get Sensing Result Request is FFS.

3.	The SEAL Sensing server sends a Sensing Service Request either directly to a sensing-related function in the 3GPP core network or to this function via the NEF (e.g., using the Nnef_Sensing_Subscribe service operation). The request includes the Target Sensing Area information. This information is derived from a three-dimensional area of interest and can be handled by 3GPP core network.

Editor's note:	the Sensing Service Request may be used to trigger the sensing service provided by 3GPP core network, or used to request the sensing result. How the SEAL sensing server utilizes the sensing service will be based on SA2 progress and is FFS.

4.	The sensing-related function in the 3GPP core network sends the Sensing Service Response either directly to the SEAL Sensing server or via the NEF. The response includes the sensing result (with or without the sensing contextual information).

Editor's note:	the detailed IEs included in the step 3 and step 4 are based on the output of SA2 and are FFS.

5.	The SEAL Sensing server sends the Get Sensing Result Response to the SEAL SM server. The response includes the List of objects information in the Area of interest obtained from the sensing service.

6.	The SEAL SM server creates the spatial map as per step 2 in clause 9.3.1.2 of 3GPP TS 23.437 [8]. In this step, the processed sensor data may be sourced from step 5 instead of the VAL layer.

7.	The SEAL SM server sends response message to the requester as per step 3 in clause 9.3.1.2 of 3GPP TS 23.437 [8].

8.	If the response in step 7 indicates "in-progress", the SEAL SM server notifies the requester with the spatial map ready notification message as per step 4 in clause 9.3.1.2 of 3GPP TS 23.437 [8].

#### 6.10.2.2	Spatial map subscription and notification with sensing capabilities

Figure 6.10.2.2-1 below illustrates the procedure for Spatial map subscription and notification with sensing capabilities. It is based on the Subscribe spatial map event specified in clause 9.3.6.2.1 of 3GPP TS 23.437 [8] and Notify spatial map event specified in clause 9.3.6.2.2 of 3GPP TS 23.437 [8].

Pre-conditions:

1.	The VAL Server or SEAL SM client is authorized to use APIs provided by the SEAL SM server.

2.	The SEAL SM server can interact with SEAL Sensing server via SEAL-Xsensing interface.

3.	The SEAL Sensing server is authorized to use Sense ing related capability APIs provided by sensing related function in the 3GPP core network or authorized to use the NEF exposed Sense ing related capability.

Figure 6.10.2.2-1: Create spatial map with periodic update notification procedure

The following step 1 to step 8 are Spatial map subscription procedure.

1.	The VAL server (or SEAL SM client) sends a Subscribe Spatial Map Event Request to the SEAL SM server to subscribe spatial map event.. The request includes the same IEs as specified in clause 9.3.6.2.1 of 3GPP TS 23.437 [8].

2.	The SEAL SM server processes the Subscribe Spatial Map Event Request as per step 2 in clause 9.3.6.2.1 of 3GPP TS 23.437 [8]. If the "Change of objects event" is subscribed by requester (VAL server or SEAL SM client), the SEAL SM server may subscribe the Sensing Results from SEAL Sensing server.

3.	The SEAL SM server sends a Get Sensing Results Subscription Request to SEAL Sensing server. The request includes requester ID, security credentials, and may include one or more IEs in the Target spatial map(s) and/or List of events.

Editor's note 1:	the detailed IEs included in the Get Sensing Result Request is FFS.

4.	The SEAL Sensing server processes the Sensing subscription for this Get Sensing Results Subscription Request.

5.	The SEAL Sensing server sends a Sensing Service Request either directly to a sensing-related function in the 3GPP core network or to this function via the NEF (e.g. Nnef_Sensing_Subscribe service). The request may includes:

a.	the Target Sensing Area information. This information is derived from the Target spatial map(s) (e.g. Area of interest) and can be handled by 3GPP core network.

b.	If periodic or event triggered sensing result notification is supported by the 3GPP core network, the SEAL Sensing server may also utilize them based on the Notification intervals included in the List of events.

c.	If Detection or Tracking specific object is supported by the 3GPP core network, the SEAL Sensing server may also translates the information included in the specified objects IE in the List of events to a list of IDs that can be handled by  the 3GPP core network and includes them in this Sensing Service Request.

Editor's note 2:	the Sensing Service Request may be used to trigger the sensing service provided by 3GPP core network, or used to request the sensing result. How the SEAL sensing server utilizes the sensing service will be based on SA2 progress and is FFS.

6.	The sensing-related function in the 3GPP core network sends the Sensing Service Response either directly to the SEAL Sensing server or via the NEF.The response indicates the result of Sensing Service subscription.

Editor's note 3:	the detailed IEs included in the step 5 and step 6 are based on the output of SA2 and are FFS.

7.	The SEAL Sensing server sends a Get Sensing Results Subscription response to the SEAL SM server to indicate the result of Sensing Results Subscription.

8.	The SEAL SM server sends a Subscribe Spatial Map Event Response to the VAL server (or SEAL SM client) as per step 3 in clause 9.3.6.2.1 of 3GPP TS 23.437 [8].

The following step 9 to step 12 are Spatial map notification procedure.

9.	The SEAL Sensing server detects a sensing event that requires the use of sensing capabilities, e.g. a specific time, then the SEAL Sensing server interacts with 3GPP core network to get the sensing result. The interaction may be also based on the information included in the Get Sensing Results Subscription Request, e.g. if periodic notification is required, the 3GPP core network may notify the sensing result to the SEAL Sensing server periodically.

10.	The SEAL Sensing server sends a Sensing Event Notification to the SEAL SM server. This notification includes the information obtained from step 9.

11.	The SEAL SM server sends a Spatial Map Event Notification to the requester as per step 2 in clause 9.3.6.2.2 of 3GPP TS 23.437 [8].

### 6.10.3	Solution evaluation

This solution utilizes sensing results to enhance the spatial maps service specified in 3GPP TS 23.437 [8].

Compared to Rel-19, the SEAL SM Server in this solution can now obtain not only processed sensor data stored at the VAL layer but also sensing results from 3GPP core network.

In this solution, neither the VAL Server nor the SEAL SM Server have sufficient knowledge of the sensing-related capabilities/parameters provided by the 3GPP core network, whether directly or via the NEF. Therefore a SEAL Sensing server is proposed to provide the following functionalities:

a)	mapping the service specific requirement, e.g. the Area of interest, to the sensing parameters used by APIs exposed by 3GPP core network;

It is proposed to include the functionalities a) as common function(s) in enabler layer to utilize sensing result.

## 6.11	Solution #10: Sensing results for spatial maps

### 6.11.1	Solution description

This solution addresses Key Issue #3, open issues #2 and #4 on how the sensing enabler layer can support SM server with creating a spatial map and to associate a sensing result with an object in a spatial map. The solution proposes enhancements to Create spatial map and Update spatial map procedures in 3GPP TS 23.437 [8].

The enhancements enable a VAL server or SM client to specify sensing service configuration when requesting to create or update a spatial map. The SM server uses the information in the sensing service configuration to send requests to a sensing service to obtain objects for the area of interest. Within the sensing service configuration, there are sensing configuration parameters the SM server can use for sending the requests to the sensing service. An Object label classification policy is provided to enable the SM server to label sensing results received from the sensing service with object IDs.

### 6.11.2	Procedures

#### 6.11.2.1	Impact to Create spatial map procedure

The Create spatial map procedure and information flows in 3GPP TS 23.437 [8] are enhanced (highlighted in bold italics) as follows.

Editor's Note:	How a VAL server discovers which SEAL sensing server can provide sensing service it needs is FFS.

#### 9.3.1.2 Procedure Figure 9.3.1.2-1 depicts the procedure for creating a spatial map. For the request from VAL server or SEAL SM client, as a spatial map consumer, SEAL SM server creates a spatial map. A spatial map can be structured in layers where each layer signifies a specific aspect of the spatial information (e.g. three dimensional space of area of interest, objects). Figure 9.3.1.2-1: Create spatial map procedure 1) The requestor (e.g., VAL server or SEAL SM client) sends a request message to the SEAL SM server to create a spatial map. The request includes requestor ID, security credentials, three-dimensional area of interest, information to be included in the spatial map such as allowed entities list defining which entities are permitted to discover and access the spatial map. The request may include spatial map layering information parameters and may also include augmented layer information that can be requested with the spatial map. The request may include a required media format for the spatial map. A Sensing service configuration may be provided to configure the SM server to use a sensing service to obtain object information for the spatial map. NOTE: SEAL SM server can obtain augmented layer information and can provide the obtained augmented layer information to the requestor in the response message. 2) The SEAL SM server authorizes the requestor, and validates the request. The SEAL SM server produces a requested spatial map using layering information and processed sensor data, according to the requested spatial map media format. The SEAL SM server does not create a spatial map if the requested spatial map media format is not supported and can create a spatial map based on local configuration if no spatial map media format is included in the request. a) If the augmented layer information indicates to include VAL UE information, the SEAL SM server fetches the list of VAL UEs in the area of interest from LM server using clause 9.3.10 of 3GPP TS 23.434 [4] and/or from NEF as specified in 3GPP TS 23.502 [5]. b) If the augmented layer information indicates to include spatial anchors information within area of interest, the SEAL SM server fetches the list of spatial anchors in the area of interest as specified in clause 8.3.4. c) If a sensing service configuration is provided, the SM server triggers a sensing service to obtain object information using information in the sensing service configuration. NOTE: Processed sensor data is stored at the VAL layer (e.g., in database at VAL layer) or within the SM server (e.g., when sensing service configuration is provided). When processed sensor data is stored at the VAL layer, the SEAL SM server gets access to such database to create a spatial map via application specific way which is out of scope. When processed sensor data is stored within the SM server, the SM server will use the sensing service configuration to configure and manage sensing service(s) to obtain object information for the spatial map. ML models are used to derive object information from sensing results and the object information is saved within the SM server. Editor’s Note: The procedure of triggering of sensing service by the SM server is FFS. 3) The SEAL SM server sends response message to the requestor with a indication of success, in-progress or fail. If the requested spatial map has been created sussesfully, the response message includes assigned spatial map ID and information which includes three-dimensional space defined by the spatial map and can include a list of spatial map layers with their corresponding layer ID, objects belong to the layer, etc.. If the response indicates "in-progress", an assigned spatial map ID is included in the message. Further, if the response indicates "in-progress" and the requestor has not provided a notification target address, then the requestor subscribes for spatial map event to receive notification when the spatial map is created. Otherwise, the response includes an indication of failure and may include a reason for the failure and indication of supported spatial map media formats if the failure is related to an unsupported spatial map media format. 4) If the response indicates "in-progress", the SM server detects when the SEAL SM server has sufficient information to map all the objects related to area of interest and the spatial map is ready to provide service (e.g. employed for localization). If the requestor has provided a notification target address in the request or has subscribed for spatial map events, the SEAL SM server notifies the requestor with the spatial map ready notification message. The notification includes parameters as specifed in Table 9.3.6.3.3-1. #### 9.3.1.3 Information flows ##### 9.3.1.3.1 Create spatial map request Table 9.3.1.3.1-1 descibes the information elements from VAL server (or SEAL SM client) to SM server for create spatial map request. Table 9.3.1.3.1-1: Create spatial map request Information element; Status; Description. Requestor identifier; M; The identifier of the requestor (e.g., VAL server or VAL User or VAL UE). Security credentials; M; The security credentials of the requestor.. Application service ID; M; ID of the requesting VAL application service. Area of interest; M; Three-dimensional area information to produce the spatial map. Notification Target Address; O; Notification target address (e.g. URL, URI, IP) where the notifications should be sent.. Spatial map media format; O; The media format requested for the spatial map.. Information to include; O; Information to be included in the spatial map. > Allowed entities list; O; Identity of entities (e.g. VAL client, VAL server or VAL service identity) which are permitted to discover and access the spatial map. > List of spatial map layers; O; List of spatial map layer information. Each element includes the information described below.. >> Spatial map layer information; O; Information to specify the corresponding spatial map layer specific information. > Augmented layer information; O; List of required augmented layer information. This IE can contain multiple values. Possible values are: - VAL_USERS; - SPATIAL_ANCHORS.. Sensing service configuration; O; Configuration information to provide configuration parameters for the SM server to trigger sensing service to obtain object information for the spatial map. The contents of the configuration is described in Table 9.3.1.3.1-y.. NOTE 1: For the current release, the spatial map layer information is implementation specific and out of scope. Table 9.3.1.3.1-y: Sensing service configuration Information element; Status; Description. Sensing service provider IDs; O; The identifier of the sensing service providers the SM server should use to trigger sensing service.. Sensing area; O; Three-dimensional area information of the sensing area.. Sensing resolution; O; A resolution to provide to the sensing service.. Sensing schedule; O; A schedule to trigger sensing service.. Sensing refresh interval; O; An interval to refresh SM object information provided by a sensing service.. Sensing output requirements; O; A listing of object characteristics required from the sensing service. Object characteristics describes the size, shape, orientation, speed, and absolute and relative location of an object obtained from sensing service.. Object label classification policy; O; A policy to assist with the labeling of sensing results obtained from the sensing service. The policy provides e.g. a list of ML models that derives application object information from sensing results received by the SM server.The ML models are trained to take sensing result inputs and outputting labels for the identification of the objects from sensing results.. NOTE 2: Sensing service configuration values are within the range defined in Table 9.3.1.3.1-1. Editor’s Note: The configuration parameters Sensing output requirements, Sensing resolution, Sensing refresh interval, and Sensing service provide IDs may need alignment with what is exposed by the sensing service.

#### 6.11.2.2	Impact to Update spatial map procedure

The Update spatial map procedure and information flows in 3GPP TS 23.437 [8] are enhanced (highlighted in bold italics) as follows.

### 9.3.4 Update spatial map #### 9.3.4.1 General The update spatial map procedure enables the consumers to request the SEAL SM Server to update a spatial map. #### 9.3.4.2 Procedure Figure 9.3.4.2-1 depicts the procedure to update the spatial map. For the request from the spatial map consumer (VAL server or SEAL SM client), SEAL SM server updates the spatial map as requested. Figure 9.3.4.2-1: Update spatial map procedure 1) The requestor (e.g., VAL server or SEAL SM client) sends a request message to the SEAL SM server to update a spatial map. The request includes requestor ID, security credentials, spatial map ID, information on what to update such as updated spatial map layering information and updated spatial map coverage area. As a part of update, the request may include augmented layer information (e.g., VAL users, spatial anchors). The request may include a list of UE-object association information for the SEAL SM server to identify associations between UEs and objects in a spatial map. A sensing service policy may be provided to configure the SM server to use a sensing service to obtain object information for the spatial map. 2) The SEAL SM server authorizes the requestor and validates the request. The SEAL SM server updates the spatial map as requested. If a sensing service policy is provided, the SM server triggers a sensing service to obtain object information with the information in the sensing service policy. Editor’s Note: The procedure of triggering of sensing service by the SM server is FFS. 3) The SEAL SM server sends response message to the requestor with result of the request and updated spatial map information with the spatial map ID. SEAL SM server may include the requested augmented layer information in the response message. #### 9.3.4.3 Information flows ##### 9.3.4.3.1 Update spatial map request Table 9.3.4.3.1-1 descibes the information elements for update spatial map request from VAL server or SEAL SM client to SEAL SM server. Table 9.3.4.3.1-1: Update spatial map request Information element; Status; Description. Requestor identifier; M; The identifier of the requestor (e.g., VAL server or VAL User or VAL UE). Security credentials; M; The security credentials of the requestor.. Application service ID; M; ID of the requesting VAL application service. Spatial map ID; M; ID of the spatial map to update. Spatial map update information; M; The spatial map Information to update as described in described in Table 7.3.3.1-1. Only IEs which need to be updated are included.. > Augmented layer information; O; List of required augmented layer information. This IE can contain multiple values. Possible values are: - VAL_UE; - SPATIAL_ANCHORS.. List of UE-object association information; O; List of UE-object association information to identify associations between UEs and objects in a spatial map as described in Table 7.3.3.x-1.. Sensing service policy; O; A policy to provide configuration parameters for the SM server to trigger sensing service to obtain object information for the spatial map. The contents of the policy is described in Table 9.3.1.3.1-y.. NOTE: At least one of the information elements shall be provided..

### 6.11.3	Solution evaluation

This solution addresses Key Issue #3, open issues #2 and #4 on how the sensing enabler layer can support SM server with creating a spatial map and to associate sensing results to objects in a spatial map, respectively. The solution proposes to enhance spatial map create and update procedures described in 3GPP TS 23.437 [8].

The enhancements enable a VAL server or SM client to specify sensing service configuration when requesting to create or update a spatial map. The SM server uses the information in the sensing service configuration to send requests to a sensing service to obtain sensing results for an area of interest and uses ML models to apply application layer labels for the identification of objects from the sensing results.

## 6.12	Solution #11: Use Sensing Results to Enhance HD Map for V2X Services

### 6.12.1	Solution Description

#### 6.12.1.0	General

Sensing plays an important role in smart transportation. Most modern cars are equipped with onboard sensors – radar, lidar, and cameras just to mention a few – which help the car to get an overview of its immediate surroundings. However, onboard systems fail to provide information further away and cannot “see around the corner”. An sensing system can complement onboard sensory data with more “global” data that cannot be obtained by sensors mounted on the vehicle. Another automotive-related example is the detection of pedestrians or animals on highways.

Application enablement layer can aggregate the sensing demands of a large number of vehicles, enabling unified subscription to sensing data from different sensing entities. By tracking or predicting vehicle locations in real time, this layer transforms the geographically scoped sensing data into a vehicle-centric dynamic view, and simplifies the vehicle's data consumption process and significantly enhances processing efficiency and system responsiveness.

The following clauses specify use sensing results to enhance HD map for supporting V2X services.

#### 6.12.1.1	Procedure

Figure 6.12.1.1-1: Sensing result subscription for enhance HD map

1.	The Consumer (e.g., VAL server/client, VAE server/client) sends sensing result subscription request to the SEAL Sensing Server for device-centric sensing result. The request contains the context information, e.g. planned route of the vehicle.

2.	The SEAL Sensing Server authenticates the request, checks the stored sensing data, and determines the next actions (e.g., to collect sensing data from sensing entities, to collect device location information/analytics, to generate device-centric sensing result).

3.	The SEAL Sensing Server responds to the VAL server for the sensing result subscription request.

4.	The SEAL Sensing Server interacts with the 5GC NF (e.g. NEF) for receiving notifications with the sensing data from the 3GPP network. The SEAL sensing server determines whether the sensing result fulfils the request from the consumer provided in step 1, and determines whether additional sensing data is needed to enhance the sensing result.

5.	The SAE Server interacts with the 3rd party sensing server for receiving the sensing data from the 3rd party sensing data.

Editor's Note:	Whether the revision of the architecture is needed for the 3rd party sensing server is FFS.

6.	After receiving the sensing data from the sensing entities in steps 4-5, the SEAL Sensing Server interacts with the other SEAL servers for collection of location information/analytics (e.g., LMS for device location information, ADAES for device location analytics).

7.	The SEAL Sensing Server filters the sensing data and location information/analytics, to generate device-centric sensing results.

8.	The SEAL Sensing Server sends sensing result subscription notifications the Consumer with the generated device-centric sensing results.

9.	The Consumer may use the sensing result for e.g., create/update HD map.

#### 6.12.1.2	Information Flows

Editor's Note:	The information flows of the solution is FFS.

### 6.12.2	Architecture impacts

Editor's Note:	The architecture impacts of the solution is FFS.

### 6.12.3	Solution evaluation

Editor's Note:	The evaluation of the solution is FFS.

## 6.13	Solution #12: Sensing-based event trigger monitoring procedure

### 6.13.1	Solution Description

This solution addresses KI#1 and proposes a new procedure to support enhanced sensing-based event trigger monitoring.

For certain vertical applications such as smart home, smart factory, and disaster and safety services, the VAL server may require continuous event-trigger monitoring according to predefined triggering conditions. To support this requirement, the SEAL Sensing Enabler server needs to continuously monitor the triggering conditions and send repeated notifications to the VAL server whenever the conditions are satisfied.

To support this functionality, the Sensing Enabler server interacts with the 3GPP CN (e.g., NEF) to get sensing results. The Sensing Enabler Server interprets the sensing requirements provided by the VAL server into corresponding sensing subscription parameters and establishes a sensing subscription toward the 3GPP CN. The Sensing Enabler Server may receive event-triggered notifications from the 3GPP CN when the network detects that the subscribed triggering conditions are met. Based on the sensing results received from 3GPP CN, the Sensing Enabler server evaluates whether the VAL Server’s triggering conditions are satisfied, generates an enhanced sensing result and exposes it to to the VAL Server.

### 6.13.2	Procedures

Figure 6.13.2-1: Sensing-based event trigger monitoring procedure

1.	The VAL server sends a monitoring subscription request to the SEAL Sensing Enabler server to subscribe for event trigger monitoring. The request may specify sensing requirements of the VAL server including sensing service type (e.g., presence, object detection, localization, motion, identification, event detection, etc), KPI, service area of interest, time, notification triggering conditions, and service duration.

2.	The Sensing Enabler server verifies whether the VAL server is authorized to request the sensing-based monitoring service. If authorized, the Sensing Enabler server stores the subscription information contained in the request message.

3.	The Sensing Enabler server responds to the VAL server that the subscription request is accepted.

4.	The Sensing Enabler server performs a continuous monitoring process to obtain sensing results from 3GPP CN until the subscription expires or an unsubscribe request is received from the VAL server. For this, the Sensing Enabler server maps the sensing requirements received from the VAL server into corresponding subscription parameters of 3GPP CN (e.g., via NEF).

5.	The Sensing Enabler server sends a Sensing Subscribe request to 3GPP CN to subscribe for the sensing results from the network.

6.	3GPP CN processes the subscription request and sends a Sensing subscribe response to the Sensing Enabler server.

7.	When 3GPP CN detects an event satisfies the triggering conditions, it sends a notification to the Sensing Enabler server.

8.	The Sensing Enabler Server evaluates the sensing results received from 3GPP CN to determine whether the triggering conditions of the VAL server are met, and generates an enhanced sensing result to send a notification to the VAL server.

9.	When a triggering condition is satisfied, the Sensing Enabler server sends a notification to the VAL Server.

10	When the VAL server decides to stop the sensing-based monitoring service, it sends an unsubscribe request to the Sensing Enabler server.

11.	The Sensing Enabler server sends a Sensing unsubscribe request to 3GPP CN to stop the sensing service in the network.

12.	3GPP CN sends a Sensing unsubscribe response to the Sensing Enabler server.

13.	The Sensing Enabler server removes the subscription information.

14.	The Sensing Enabler server sends an unsubscribe response to the VAL server.

## 6.14	Solution #13: Sensing Requests Process and Sensing Result Reuse

### 6.14.1	Solution Description

#### 6.14.1.1	General

The following clauses specify SEAL Sensing Server service on support processing sensing requests from different consumers to remove conflict in the request, and reuse of sensing result to save resource for sensing result collection.

#### 6.14.1.2	Procedure

Pre-condition:

-	The requests from different consumers are received at the SEAL sensing server in the same time period, so that the SEAL sensing server can process these requests.

Figure 6.14.1.2-1: Procedure on support sensing requests process and sensing result reuse

1.	Multiple consumers (e.g. VAL server, SEAL server, UAE server, VAE server) send sensing requests separately to the SEAL sensing server. The request contains the requirements on sensing result collection (e.g. target time period for collection sensing result, target area).

2.	Upon receiving the request(s), the SEAL sensing server checks for the relevant authorization for the request(s). If the authorization is successful, the SEAL sensing server determines next action on processing of the request(s).

3.	The SEAL sensing server processes the requests from different consumers to remove conflict, e.g., merge requests into one request for sensing collection (e.g. same sensing results are requested in different requests), split one request into multiple ones (e.g. sensing results required in one request need be collected from different data sources), or grouped the requests into smaller number of requests (e.g. common requirements on sensing results in some of the requests).

4.	The SEAL sensing server checks the cached sensing result and determines whether the required sensing result are available. If the required sensing result is available, go to step 6. Otherwise, go to step 5.

5.	If the required sensing result is not available, the SEAL sensing server interacts with the 5GC NF (NEF) for collecting sensing result.

6.	The SEAL Sensing Server caches the received sensing result received previously and/or received in step 5, and may also caches the request information corresponding to the sensing result. The SEAL Sensing Server processes the collected sensing result based on the request in step 1.

7.	The SEAL Sensing Server sends sensing responses to the consumers.

#### 6.14.1.3	Information Flows

Editor’s Note:	The information flows of the solution is FFS.

### 6.14.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.14.3	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

## 6.15	Solution #14: Sensing UAVs combining location and sensing results

### 6.15.1	Solution description

This solution addresses the KI#2.

In this solution, it is proposed a mechanism where a UAV with a UE module (UAV ID) can be identified as a Sensed object. Once the matching between a sensed object and the UAV ID is done, the sensing results can be used to assist in continuing tracking the UAV ID when network location is not available. Monitoring the position of the UAV ID based on the matched sensed object could be used e.g. to monitor the distance with other sensed objects returned in the sensing response and avoid the collision with them.

The solution is based on the correlation of the position of the UAV ID obtained with location services and the position of the objects sensed by the sensing network, using e.g. a minimum distance algorithm between the position of the UAV ID returned by the location server and the sensed object positions returned by the 5GC.

#### 6.15.1.1	Procedure for sensing UAVs with a UE module

Figure 6.15.1.1-1 illustrates the high-level procedure for sensing UAVs with a UE module.

Pre-condition:

-	The UAV ID is registered and connected in the CN.

Figure 6.15.1.1-1: Procedure for sensing UAV ID

1.	The Sensing Enabler client (e.g. VAL server, or the UAV server) sends a Sensing request to the Sensing enabler, including the request to detect and identify a UAV ID. The VAL server includes the UAV ID (GPSI), the sheduled UAV ID trajectory or target area around the UAV ID, and the sensing time duration. If the VAL server wants to receive in step 5 the matching between the provided UAV ID and a sensed object, the VAL server also includes in the request the indication that the result needs to include the matching between the UAV ID and sensed Object description (e.g., position, velocity). The VAL server may include other sensing request parameters related e.g. with the sensing of objects in the proximity of the UAV.

2.	The sensing enabler invokes the Location service and obtains the network location for the UAV ID.

3.	The sensing enabler invokes the sensing related service request to the 3GPP CN and obtains the sensing results from the 3GPP CN for the area indicated in step 1.

Editor's note:	The exposure of sensing results from 3GPP CN depends on SA2’s progress. What will be exposed to the consumers is FFS.

4.	The Sensing enabler calculates the distances between the returned sensed objects and the network location(s) (e.g., the Sensing enabler utilizes samples, obtained ant the same time, of the sensing results (object(s) positions) and the network location of the UAV ID and calculates for each sample the object with the minimum distance between the object position and the network location), and determines with a certain probability, which is the object that corresponds to the UAV ID. When the probability that a sensed object is the UAV ID is above a certain accuracy level, the Sensing enabler concludes that a sensed object is the UAV ID included in step 1.

5.	The Sensing enabler sends to the VAL server the sensing result, including the description of the matched object (e.g., position, velocity) with the UAV ID, the accuracy of the matching and the corresponding UAV ID.

For subsequent sensing results, when the network location of the UAV ID is available, the Sensing Enabler uses the reported UE location to increase accuracy of the sensing positioning results related to the correlated object. When network location of the UAV ID is not available, the Sensing Enabler uses the sensing positioning results in the report. The sensing Enabler indicates to the VAL server whether network location is contributing to the positioning information included in the sensing report of the object bound to the UAV ID.

### 6.15.2	Architecture Impacts

This solution proposes a new sensing enabler to support the sensing of UAV ID. The new architecture for the sensing enabler is described in the solution for KI#1.

### 6.15.3	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

Editor’s Note:	The evaluation of the solution is FFS.

## 6.16	Solution #15: Presence Detect of UAV in No Drone Zone

### 6.16.1	Solution Description

#### 6.16.1.1	General

The following clauses specify SEAL Sensing Server service on presence detect of UAV in No Drone Zone and notify.

Pre-condition:

-	The UAE server has the No Drone Zone (no UAV flight zone) list of geographical areas.

#### 6.16.1.2	Procedure

Figure 6.16.1.1-1: Presence detect in No Drone Zone and notify

1.	The UAE Server sends presence detect in No Drone Zone subscription request to the SEAL Sensing Server. The request contains the planned flight path of the UAV, and no Drone Zone information. The actual trajectory of the flight may change compared with the planned flight and may get into the No Drone Zone. The UAE Server may update the presence detect in No Drone Zone subscription request if the No Drone Zone is changed.

2.	The SEAL Sensing Server interacts with 5GC NF (NEF) for sensing result.

3	The SEAL Sensing Server detects presence of object in the No Drone Zone, based on the sensing result received from 5GC NF (NEF) in step 2.

4.	The SEAL Sensing Server sends presence in No Drone Zone notification to the UAE Server, with one or more detected object(s), and the location of the object(s).

5.	The UAE Server maps the object and UAV ID(s).

6.	The UAE Server determines actions for the UAV(s), e.g. adjust flight route of the UAV(s).

#### 6.16.1.3	Information Flows

Editor’s Note:	The information flows of the solution is FFS.

### 6.16.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.16.3	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

## 6.17	Solution #16: Converged Trajectory Tracking

### 6.17.1	Solution Description

#### 6.17.1.1	General

Currently, there are two means to for UAE server to track a drone through LM server:

-	The drone may periodically report its location (e.g. GPS location) to LM server.

-	LM server uses network location service to get the locations of the drone.

However, the drone may be hacked or malfunctioning so that it stops reporting locations to LM server/UAE, or the drone may be malfunctioning so that the network location service cannot get the location. Sensing can be used as a complementary mean to track the drone’s trajectory. Since sensing service does not know the UE ID, how to correlate the sensed object to the UE ID is a challenge.

The following clauses specify how the SEAL Sensing Server service can be used to request sensing for mapping with UAV locations to provide object description of the target UAV.

#### 6.17.1.2	Procedure

Pre-condition:

-	UAE Server requests to LM Server for UAV location reporting.

-	The LM Server requests the LM Client (deployed on a UAV) to report location.

-	The 5GCN provides the sensed objects.

Figure 6.17.1.1-1: Converged trajectory tracking

1.	The LM Client (deployed on a UAV) reports the UAV location to LM Server. The LM server provides the UAV location information to the UAE Server. The UAE Server records the trajectory of the UAV as the UAV is changing its position.

2.	The UAE Server sends sensing requests to the SEAL Sensing Server indicating that the response shall include the object(s) whose position correspond to the UAV location. The request includes UAV location information (e.g. UAV trajectory).

3.	The SEAL Sensing Server determines the dynamic sensing area based on the UAV locations received in step 1.

4.	The SEAL Sensing Server interacts with 5GC NF (NEF) for sensing result for the dynamic sensing area determined in step 2.

5.	After getting sensing result from 5GC NF (NEF), the SEAL Sensing Server compares the trajectories and characteristics (e.g. timestamp, position and velocity) of sensed objects and the received UAV trajectory (timestamp-position sequences) and determines the object that maps with the target UAV.

6.	The SEAL Sensing Server sends sensing result periodically to the UAE Server with the mapped object of the target UAV.

7.	The UAE Server sets up/updates the correlation between the received object and the UAV ID, based on the received information from step 5 and additional information from LMS and 5GC NF (GMLC).

8.	The UAE Server sends location information requests periodically by object or UAV ID, to various location information sources (e.g. LM Server, SEAL Sensing Server, 5GC NFs (via NEF)), for location information of the UAV (e.g., to determine potential deviations).

9.	The UAE Server aggregates the location information received from the location information sources (e.g. LM Server, SEAL Sensing Server, 5GC NFs (via NEF)) to generate UAV trajectory.

10.	The UAE Server records the trajectory of the UAV.

#### 6.17.1.3	Information Flows

Editor’s Note:	The information flows of the solution is FFS.

### 6.17.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.17.3	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

## 6.18	Solution #17: Sensing enabler service procedure for spatial map object detection and tracking

### 6.18.1	Solution description

#### 6.18.1.0	General

The Spatial Map server can request sensing enabler services for sensing results, object detection, movement tracking to detect object changes, mobility prediction, estimated duration of staying in-range, etc.

The following clause specifies procedures for sensing enabler services that detect objects, track target spatial map objects, and provide additional value-added service results based on the collected sensing results.

#### 6.18.1.1	Procedure

Pre-conditions:

1.	SEAL Sensing Enabler server has acquired information of Application Specific Layer server(s).

2.	SEAL Sensing Enabler server can identify the object from sensing results and associate it with a target object, utilizing sensing results from 5GC NF and spatial map information in Sensing enabler service subscribe request.

3.	SEAL Sensing Enabler server can track the identified object by monitoring the collected sensing results form 5GC NF.

Editor's note:	How to identify the object from sensing results and track it are dependent on what are included in the sensing result(s), and details are FFS.

Figure 6.18.1.2-1 Sensing enabler service procedure for spatial map object detection and tracking

1.	SM server sends a request message to SEAL Sensing Enabler server to subscribe a sensing enabler service. The request includes, as specified in Table 6.18.1.2.1-1, requestor ID, area of interest and object information in the target spatial map, requested sensing service, etc. The requestor (SM server) may subscribe to one of the following services:

a.	The “Object detection service” specifies that the Sensing Enabler server should notify the requestor of all objects detected in the area of interest. The notification triggering event type for the service may include event based (e.g., promptly) or periodically (with duration and interval time).

b.	The “Object tracking service” specifies that the Sensing Enabler server should notify the requestor when changes to the specified target objects are detected. The Sensing Enabler server monitors changes of target object utilizing sensing result(s) which are collected continuously from the 5GC NF. The notification triggering criteria for the service may include when a target object is added, removed, moved in or out of the given area of interest, or when its position, direction, velocity etc. is changed. The request may also include tracking end condition and notification triggering event type. If the requester only requires detection results for changed objects, the notification filter can specify "Changed objects only". Otherwise, the Sensing Enabler server notifies the SM server of the detection results for all target objects so that it can determine the changes.

c.	The “Enhanced sensing enabler service” specifies that the Sensing Enabler server should provide value-added sensing enabler services such as tracking object mobility prediction (e.g., ADAE analytics) and estimated duration for target object to remain in-range. The Sensing Enabler server can use accumulated tracking results and trajectory records, etc., for the prediction and estimation service.

2.	SEAL Sensing Enabler server authorized the SM server, and validates the request. If authorized, the SEAL Sensing Enabler server adds the requestor to the subscriber list of the requested service.

3.	SEAL Sensing Enabler server sends response message with the result. For the successful subscribe result, a subscription ID is included in the message. Otherwise, the response message may include the failure cause.

4.	SEAL Sensing Enabler server interacts with 5GC NF to collect the required sensing results.

5. 	The Sensing Enabler server determines whether sensing results received from 5GC NF meet the requirements to derive desired results for the subscribed service. If the received sensing results meet the requirements, skip to step 7. Otherwise, the Sensing Enabler server may want to use data from the Application Specific layer/VAL layer server.

6.	SEAL Sensing Enabler server interacts with the Application Specific layer/VAL layer server.

7.	The SEAL Sensing Enabler server generates result(s) for the services to which SM server subscribed. For the “Object detection services”, based on the received sensing results, SEAL Sensing Enabler server identifies the object from sensing results and associate it with an existing spatial map object. For “Object tracking services”, the SEAL Sensing Enabler server determines whether the generated object detection results meet the triggering criteria for change of objects condition. If so, go to step 8. Otherwise, step 4 – step 6 are repeated. If SM server requested “Enhanced sensing enabler service” to obtain additional value-added service results, the Sensing Enabler server generates requested service results using accumulated tracking results and trajectory history, etc., to predict the mobility of the tracked object and calculate the expected duration of the target object's stay within the range. If 5G NF does not support periodic reporting and continuous monitoring of tracking objects, for requests that require continuous tracking of target objects (e.g. for mobility prediction, estimated duration of staying in-range, etc.), step 4 – step 6 are repeated.

NOTE:	In this release, SM specific sensing enabler service procedures are provided. If and when these sensing enabler services are available in 3GPP CN sensing function, usage of the 3GPP CN sensing services will be considered.

8.	The SEAL Sensing Enabler server detects the notification triggering event. The types of notification triggering events for each sensing enabler service are as specified in Table 6.18.1.2.1-2, Table 6.18.1.2.1-3 and Table 6.18.1.2.1-4.

9.	The SEAL Sensing Enabler server notifies the SM server which requested the corresponding sensing enabler service. The notification includes subscription ID and notification information. The notification information may include object information detected as requested, and additional value-added service results such as object mobility prediction and estimated duration of stay. The objects in the notification information may include either all objects detected in the given area of interest, or only include changed objects, depending on the type of service requested (e.g. object tracking service with changed objects only indication). For periodic notification, step 4 – step 9 are repeated.

#### 6.18.1.2	Information Flows

##### 6.18.1.2.1	Sensing enabler service subscribe request

Table 6.18.1.2.1-1 shows the Sensing enabler service subscribe request sent by SEAL SM server to SEAL Sensing Enabler server to subscribe to sensing enabler service.

Table 6.18.1.2.1-1: Sensing enabler service subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (i.e., SM server) |
| Expiry time | O | Proposed subscription expiry time |
| Target spatial map information | M | Information on the spatial map targeted by the sensing enabler service |
| > Spatial map ID | O | ID of the target spatial map |
| > Spatial map coverage | O (NOTE 1) | The three-dimensional space coordinates information of the spatial map. |
| > Area of interest | O (NOTE 1) | Three-dimensional area information to detect and tracking objects. This area can be the same or a subset of the spatial map coverage space. |
| > List of objects | O | List of spatial map object information in the area specified (i.e., spatial map coverage or area of interest). |
| >> Object ID | M | The identifier of the corresponding object |
| >> Object type | O | Type of the object. Stationary or trackable, and other classified type based on the object specific attribute information (e.g., specific type of living things, materials, machineries, etc.). |
| >> Object specific information | O | Object specific attribute information (e.g., size, shape, texture, etc.) |
| >> Object mobility information | M | Specifies position, direction, velocity of the object. |
| >>> Object position | O | 3D position of the object. For stationary object, this IE shall be present. For trackable object, 3D position of it shall be provided while it is being tracked, otherwise no value. |
| >>> Direction (NOTE 2) | O | Direction of the trackable object while it’s being tracked, otherwise no value. |
| >>> Velocity (NOTE 2) | O | Velocity of the trackable object while it’s being tracked, otherwise no value. |
| Requested sensing service type | M | Specifies sensing service type and required parameters. |
| > Object detection service | O (NOTE 3) | Detect objects in the given area of interest. Each element contains the information as specified in Table 6.18.1.2.1-2. |
| > Object tracking service | O (NOTE 3) | Detect changes and tracking of specified target objects in a given area of interest. Each element contains the information as specified in Table 6.18.1.2.1-3. |
| > Enhanced sensing enabler service | O (NOTE 3) | Provides value-added sensing enabler services based on enhanced sensing results, etc. Each element contains the information as specified in Table 6.18.1.2.1-4. |
| Sensing capability requirements | O | Capabilities required to obtain desired sensing results for the requested service (e.g., resolution, capabilities required to determine object specific attributes such as size, shape, texture, etc.) |
| NOTE 1: At least one of the sub-elements shall be provided. NOTE 2: These IEs may be present when object is moving or changes its location frequently. NOTE 3: At least one of the sub-elements shall be provided. |  |  |

Table 6.18.1.2.1-2 describes object detection service details for the sensing enabler service subscribe request specified in Table 6.18.1.2.1-1.

Table 6.18.1.2.1-2: object detection service information

| Information element | Status | Description |
| --- | --- | --- |
| Object type | O | Specifies the type of objects to be detected. |
| Object specific attribute information | O | Specifies object specific attribute information to be detected. |
| Notification triggering event type | O | Specifies when to notify the detected objects, possible values are: - Event based (e.g., promptly); or - Periodically (with duration and interval time) |

Table 6.18.1.2.1-3 describes object tracking service details for the sensing enabler service subscribe request specified in Table 6.18.1.2.1-1.

Table 6.18.1.2.1-3: object tracking service information

| Information element | Status | Description |
| --- | --- | --- |
| List of target object IDs | M | Target object IDs to detect changes, to be tracked |
| Triggering criteria | O | Specifies the condition to determine whether the detected event satisfies requirements of requested sensing service. |
| >Change of objects | O | When objects are added, removed, moved in or out of the given area of interest, changed position, direction, velocity etc. |
| Tracking end condition | O | Specifies the condition until when changed target objects are notified (e.g., untll when no more target object to track is left, or specifies the tracking end period (i.e., duration), etc.) |
| Notification triggering event type | O | Specifies when to notify, possible events are: - Whenever change of object is detected, notify it promptly until tracking end condition is met. - Periodically (with duration and interval time) |
| Notification filter | O | Specifies what to notify. |
| >Changed objects only | O | If specified, notify sensing results of changed objects only, otherwise notify sensing results of all target objects so that SM server can determine the changes. |

Table 6.18.1.2.1-4 describes enhanced sensing service details for the sensing enabler service subscribe request specified in Table 6.18.1.2.1-1.

Table 6.18.1.2.1-4: enhanced sensing service information

| Information element | Status | Description |
| --- | --- | --- |
| Type of service | M | Specifies value-added sensing enabler service. Possible services are: - tracking object mobility prediction. - estimated duration for which target object stays in-range. |
| Target object ID | M | Target object ID being serviced |
| Tracking range of interest | O | Range for target object tracking |
| Notification triggering event type | O | Specifies when to notify, possible events are: - When requested sensing enabler service result is ready, notify it promptly. - Periodically (with duration and interval time) |

##### 6.18.1.2.2	Sensing enabler service subscribe response

Table 6.18.1.2.2-1 shows the Sensing enabler service subscribe response sent by SEAL Sensing Enabler server to SEAL SM server as the response to sensing enabler service subscribe request.

Table 6.18.1.2.2-1: Sensing enabler service subscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result (i.e., success or failure) of the sensing enabler service subscribe request |
| Subscription ID (NOTE) | O | Sensing enabler service subscription identifier |
| Expire time | O | Subscription expiry time |
| Failure cause | O | Indicates reason for the failure. |
| NOTE: This IE is mandatory when Result IE indicates success. |  |  |

##### 6.18.1.2.3	Sensing enabler service notification

Table 6.18.1.2.3-1: Sensing enabler service notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Sensing enabler service subscription identifier |
| Notification information | M | Specifies requested sensing service result. The objects in the notification information may include either all objects detected in the given area of interest, or only include changed objects, depending on the type of service requested (e.g., object tracking service with changed objects only indication). |
| > List of detected objects | O | Object information detected in the given area of interest. Each element includes the information described below. |
| >> Object ID | O | Detected object identifier |
| >> Object type | O | Type of the object. Stationary or trackable, and other classified type based on the object specific attribute information (e.g., specific type of living things, materials, machineries, etc.). |
| >> Object specific attribute information | O | Object specific attribute information (e.g., size, shape, texture, etc.) |
| >> Object mobility information | O | Specifies position, direction, velocity of the detected object. |
| >> Changes to the object | O | Detected object change information requested by object tracking service |
| >> Sensing result | O | Sensing result received from 5G NF or enhanced sensing result |
| > Estimated duration of stay | O | Estimated duration for which the target object stays in the tracking range of interest. |
| > Object mobility prediction | O | Mobility prediction information of the target object. |

### 6.18.2	Architecture impacts

Editor's Note:	The architecture impacts of the solution is FFS.

### 6.18.3	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

Editor's Note:	The evaluation of the solution is FFS.

# 7	Deployment scenarios

## 7.1	General

This clause will provide a general description of the deployment scenarios.

## 7.x	Deployment model #x: <Title>

Provide a description of the deployment scenarios.

# 8	Business Relationships

Provide a description of the involved business relationships.

# 9	Overall evaluation

This clause will provide evaluation of different solutions.

# 10	Conclusions

## 10.1	General conclusions

This clause will provide general conclusions for the study.

## 10.2	Conclusions of key issue #x

This clause will provide conclusions for the specific key issue.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-08 | SA6#68 | S6-253520 |  |  |  | Skeleton | 0.0.0 |
| 2025-09 | SA6#68 |  |  |  |  | Implemented the following pCRs, S6-253521,S6-253522,S6-253723,S6-253759,S6-253760,S6-253776 | 0.1.0 |
| 2025-10 | SA6#69 |  |  |  |  | Implemented the following pCRs,S6-254617, S6-254779, S6-254736, S6-254623, S6-254737, S6-254679, S6-254756, S6-254681, S6-254682, S6-254790, S6-254781, S6-254739, S6-254686 | 0.2.0 |
| 2025-11 | SA6#70 |  |  |  |  | Implemented the following pCRs, S6-255573, S6-255574, S6-255668, S6-255576, S6-255578, S6-255579, S6-255669, S6-255670, S6-255671, S6-255583, S6-255607, S6-255616, S6-255617 | 0.3.0 |
| 2025-12 | SA#110 | SP-251459 |  |  |  | Presentation for information at SA#110 | 1.0.0 |