3GPP TSG-SA WG6 Meeting #70	S6-255449

Dallas, USA, 17th – 21st November 2025	(revision of S6-255040)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0091 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Fix for clause 8.3.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_App |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. In spatial anchors management procedures, all other operations (creation, update, deletion, discovery, subscription) are bulk process except retrieve operation. 2. In step 1 of spatial anchor retrieve procedure, there is “to get information about specific spatial anchors of interest”, it should be bulk process spatial anchors not single one. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fix retrieve operation to bulk process. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Inconsistent with other spatial anchors operation. - Inconsistent with spatial anchor retrieve procedure and information flows. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.6.1, 8.3.6.2, 8.3.6.3.1, 8.3.6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

8.3.6	Spatial anchor retrieve

8.3.6.1	General

Spatial anchor get operation enables a SAn client or VAL server to get details about the specific spatial anchors from the SAn server.

8.3.6.2	Procedure

Figure 8.3.6.2-1 illustrates the procedure for spatial anchor retrieve operation following the request-response model.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server;

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server;

3.	The SAn client or VAL server has discovered the list of spatial anchors as per the procedure defined in clause 8.3.4.

Figure 8.3.6.2-1: Spatial anchor retrieve

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor retrieve request message to the SAn server to get information about specific spatial anchors of interest. The request includes a requestor identifier, security credentials, the identifier of spatial anchors. The request may include information to retrieve multiple spatial anchors.

2.	Upon receiving the request, the SAn server validates if the requestor is authorized for the request. If the requestor is authorized, the SAn server identifies whether spatial anchors exist or not based on identifier of the spatial anchors.

3.	The SAn server sends a spatial anchor retrieve response to the requestor. If the SAn server has identified spatial anchors, the response includes an indication of success and the details about the spatial anchors. Otherwise, the response includes an indication of failure and may include a reason for failure.

8.3.6.3	Information flows

8.3.6.3.1	Spatial anchor retrieve request

Table 8.3.6.3.1-1 describes information elements for the spatial anchor retrieve request from the SAn client or VAL server to the SAn server.

Table 8.3.6.3.1-1: Spatial anchor retrieve request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| List of spatial anchors | M | List of spatial anchors to be retrieved. |
| > Spatial anchor ID | M | Identifier of the spatial anchor to be retrieved. |

8.3.6.3.2	Spatial anchor retrieve response

Table 8.3.6.3.2-1 describes information elements for the spatial anchor retrieve response from the SAn server.

Table 8.3.6.3.2-1: Spatial anchor retrieve response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the spatial anchor retrieve request was successful. |
| > List of spatial anchors | M | List of spatial anchors retrieved. |
| >> Spatial anchor profile | M | Spatial anchor profile as specified in Table 7.3.2.1-1 |
| Failure response | O | Indicates that the spatial anchor retrieve request has failed. |
| > Cause | O | The cause for the request failure. |

# * * * * End of changes * * * *