3GPP TSG-SA WG6 Meeting #69	S6-254539

Wuhan, China 13th – 17th October 2025	(Revision of S6-254032)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.438 | CR | 0020 | rev | 0 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on digital asset discovery request message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_App |  |  |  |  |  | Date: |  |  | 2025-09-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the Digital asset discovery requst message, Requesting digital asset identifier and Digital asset discovery filters are used to discovery the target digital asset. There are the following issues identified: 1) Once Requesting digital asset identifier is determined, digitial asset dicovery filter is not needed because digital asset identifier can identified a digital asset uniquely. Conversly, when filters method is used, it means there is no information on the digital asset identifier. So, they should not be mandatory. 2) Morover, the Digital asset identifier in Digital asset dicovery filter is redundant because there is an IE Request digital asset identifier existing. 3) VAL client type is not needed because it is not included in digital asset profile. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Change the status of both IE Requesting digital asset and IE Digital asset discovery filters as “O” and add a note that at least one of them should be present. 2) Remove Digital asset identifier and VAL client type from the description of Digital asset discovery filters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The IEs setting and description of Digital asset discovery request message are not accurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

8.3.3.1	Digital asset discovery request

Table 8.3.3.1-1 describes the information elements for a digital asset discovery request sent from a requestor (e.g. DA client or VAL server) to a DA server.

Table 8.3.3.1-1: Digital asset discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Digital asset identifier | O (NOTE) | The identifier of the digital asset. |
| Digital asset discovery filters | O (NOTE) | Set of characteristics to determine matching digital asset (e.g., Digital asset name, Digital asset description, location, Digital asset type, Digital asset owner, Allowed user, Digital asset profile). |
| NOTE: At least one of the information elements shall be present |  |  |