3GPP TSG-SA WG6 Meeting #69	S6-254542

Wuhan, China 13th – 17th October 2025	(Revison of S6-254053)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.438 | CR | 0025 | rev | 0 | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on digital asset media |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_App |  |  |  |  |  | Date: |  |  | 2025-09-26 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the Delete digital asset media request message, there is a IE List of digital asset media and it only has a child IE Media URI. However, digital asset media could be a Media URI or Media object according to the Upload digital asset media request message. Hence, Media object is missing. In addition, there could be a list of digital asset media for one digitial asset and media could be Media URI or Media object, but the IE Digital asset media information in digital asset profile is not aligned with this. . |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Add one child IE Media object for List of digital asset media. 2) Update the IE Digital asset media information in digital asset profile as List of Digital asset media. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The usage of digital asset media is not aligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2, 8.4.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

7.2	Digital asset profile

The digital asset profile includes the information elements about the attributes of a single digital asset. Table 7.2-1 shows the information elements for the digital asset profile.

NOTE 1: In this release, the digital asset profile is specified considering Avatar as a digital asset of the user.

Table 7.2-1: Digital asset profile

| Information element | Status | Description |
| --- | --- | --- |
| Digital asset ID | M | Identifier of the digital asset profile |
| Digital asset type | M | Type of the digital asset. (For this release, it contains "Avatar"). |
| List of owners | M | List of identity of the owner user(s) who created the digital asset. |
| List of service provider identifiers | O | List of service providers for the digital asset. |
| access control list | O | List of tuples indicating the requestors allowed to perform operations with this digital asset and the allowed operation types. |
| > allowed user list | O | List of users allowed to access this digital asset |
| >> allowed operations | O | List of operations allowed by this user. Possible operations are (one or more of) : Discovery, Get, Update, Delete |
| >> list of predictive models | O | For each allowed user, list of one or more predictive model for the user, in order to predict user's behaviour. |
| current status | O | Current status (in use or not) of the profile |
| spatial conditions | O | List of spatial conditions (e.g. locations) where this digital asset profile is allowed to be accessed |
| > current location | O | Current location where the profile is being used |
| allowed application list | O | List of application IDs which are allowed to use this digital asset profile |
| associated accessories identifiers | O | List of digital asset identifiers which are accessories (e.g. Hat, watch, shoes) purchased for this digital asset profile |
| expiry time | O | Time until this digital asset profile is valid |
| History information | O | Usage history for this digital asset profile |
| > list of locations | O | List of locations where the digital asset profile is used. |
| > time of use | O | Time information when the digital asset profile is used |
| > list of applications | O | List of the application IDs which used this digital asset profile. |
| List of digital asset media | O | List of digital asset media |
| > Media URI | O (NOTE) | URI where the media is stored |
| > Media object | O (NOTE) | Media object (e.g. JPG file) |
| NOTE: One of the IE shall be included |  |  |

# * * * * Next change * * * *

8.4.3.7	Delete digital asset media request

Table 8.4.3.7-1 describes the information elements for a delete digital asset media request sent from a requestor (e.g. DA client) to a DA server.

Table 8.4.3.7-1: Delete digital asset media request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Digital asset identifier | M | Identifier of the digital asset profile |
| List of digital asset media | M | List of digital asset media to be deleted |
| > Media URI | O (NOTE) | URI where the media is stored |
| > Media object | O (NOTE) | Media object (e.g. JPG file) |
| NOTE: One of the IE shall be included |  |  |