3GPP TSG-SA WG6 Meeting #69	S6-254731

Wuhan, China 13th – 17th October 2025	(revision of S6-254265)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.438 | CR | 0028 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving ENs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_App |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Following ENs are removed: Editor's Note: The association of a subscriber/user identifier for the digital asset is FFS. - Digital asset owner identity can be VAL user identity or VAL UE identity. So, we can add clarification about Digital asset user being VAL user and remove the EN. Editor's Note: Additional parameters (communication parameters, experience requirements) are FFS, - Communication pameters are for SA2 to consider during avatar based communication. So, we can remove such EN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added clarification about digital asset user identity. And removed ENs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ENs will remain in Rel-19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.2.3.1	Digital asset profile create request

Table 8.2.3.1-1 describes the information elements for a digital asset profile create request sent from a requestor (e.g. DA client) to a DA server.

Table 8.2.3.1-1: Digital asset profile create request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Digital asset profile parameters | M | Parameters to create digital asset profile |

Table 8.2.3.1-2: Parameters for digital asset profile

| Information element | Status | Description |
| --- | --- | --- |
| Digital asset owner identifier(s) | M | A list of identifier(s) for the digital asset owner. |
| Digital asset type | M | The type of digital asset, e.g. avatar, wallet, etc. |
| List of service provider identifiers | O | List of service providers for the digital asset. |
| access control list | O | List of tuples indicating the requestors allowed to perform operations with this digital asset and the allowed operation types. |
| > allowed user list | O | List of users allowed to access this digital asset |
| >> allowed operations | O | List of operations allowed by this user. Possible operations are (one or more of) : Discovery, Get, Update, Delete |
| >> list of predictive models | O | For each allowed user, list of one or more predictive model for the user, in order to predict user's behaviour. |
| spatial conditions | O | List of spatial conditions (e.g. locations) where this digital asset profile is allowed to be accessed |
| > current location | O | Current location where the profile is being used |
| allowed application list | O | List of application IDs which are allowed to use this digital asset profile |
| associated accessories identifiers | O | List of digital asset identifiers which are accessories (e.g. Hat, watch, shoes) purchased for this digital asset profile |
| expiry time | O | Time until this digital asset profile is valid |

Editor's Note: The association of a subscriber/user identifier for the digital asset is FFS.

* * * End of Change * * * *