3GPP TSG-SA5 Meeting #164	S5-255696

Dallas, USA, 17 - 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR | 0594 | rev | 3 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.623 Immediate MDT Support for L1 RSRP measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Ericsson, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TraceQoE_OAM |  |  |  |  |  | Date: |  |  | 2025-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN2 has sent a reply LS S5-254621/R2-2506543 in response to S5-252842/R2-2505036 requesting to provide a solution to transfer specified measurements in the context of OAM-centric solution for NW-side data collection. This CR provides the corresponding corrections to align with agreed RAN2 Rel-19 enhancements in TS 37.320. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add configuration parameters "layerOneRsrpPeriodicity" and "eventTriggerConfig" to "ImmediateMdtConfig" <<dataType>> - Introduce <<dataType>> EventTriggerConfig in ComDefs.yaml |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - RAN2 specifications are not aligned with SA5 specifications - RAN2 feature "network side data collection" is not supported by management system |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1973 at commit f62550027f341fd6c478bfe086b76923dc1c2a0f |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is part of the package. It has dependency on TS 32.422 CR0541 and TS 28.622 CR0612. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGE 1 ***

*** OpenAPI/TS28623_ComDefs.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: Common Type Definitions

version: 19.5.0

description: >-

OAS 3.0.1 specification of common type definitions in the Generic NRM

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.623; Generic NRM; Common type definitions

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.623/

paths: {}

components:

schemas:

Float:

description: This datatype is used for writable attribute

type: number

format: float

FloatRo:

description: This datatype is used for readOnly attribute

type: number

format: float

readOnly: true

DateTime:

description: This datatype is used for writable attribute

type: string

format: date-time

DateTimeRo:

description: This datatype is used for readOnly attribute

type: string

format: date-time

readOnly: true

FullTime:

type: string

description: format (e.g., full-time) is from RFC 3339

format: full-time

DateMonth:

type: string

format: date-month

DateMonthDay:

type: string

format: date-mday

DayInYear:

type: object

properties:

month:

$ref: '#/components/schemas/DateMonth'

monthDay:

$ref: '#/components/schemas/DateMonthDay'

Latitude:

type: number

format: float

minimum: -90

maximum: 90

Longitude:

type: number

format: float

minimum: -180

maximum: 180

GeoCircle:

description: This datatype is specifies the geographical area for NTN

type: object

properties:

referenceLocation:

$ref: '#/components/schemas/GeoCoordinate'

distanceRadius:

type: integer

minimum: 1

maximum: 65535

Dn:

description: This datatype is used for writable attribute

type: string

DnRo:

description: This datatype is used for readOnly attribute

type: string

readOnly: true

DnList:

description: This datatype is used for writable attribute

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/Dn'

DnListRo:

description: This datatype is used for readOnly attribute

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/DnRo'

Mcc:

description: This datatype is used for writable attribut

type: string

pattern: '^[0-9]{3}$'

MccRo:

description: This datatype is used for readOnly attribute

type: string

pattern: '^[0-9]{3}$'

readOnly: true

Mnc:

description: This datatype is used for writable attribut

type: string

pattern: '^[0-9]{2,3}$'

MncRo:

description: This datatype is used for readOnly attribute

type: string

pattern: '^[0-9]{2,3}$'

readOnly: true

Nid:

type: string

pattern: '^[A-Fa-f0-9]{11}$'

CagId:

type: string

pattern: '^[A-Fa-f0-9]{8}$'

PlmnId:

description: This datatype is used for writable attribute

type: object

properties:

mcc:

$ref: '#/components/schemas/Mcc'

mnc:

$ref: '#/components/schemas/Mnc'

PlmnIdRo:

description: This datatype is used for readOnly attribute

type: object

properties:

mcc:

$ref: '#/components/schemas/MccRo'

mnc:

$ref: '#/components/schemas/MncRo'

Tac:

type: string

pattern: '(^[A-Fa-f0-9]{4}$)|(^[A-Fa-f0-9]{6}$)'

UtraCellId:

type: integer

EutraCellId:

type: string

pattern: '^[A-Fa-f0-9]{7}$'

NrCellId:

type: string

pattern: '^[A-Fa-f0-9]{9}$'

TimeWindow:

description: This datatype is used for writable attribute

type: object

properties:

startTime:

$ref: '#/components/schemas/DateTime'

endTime:

$ref: '#/components/schemas/DateTime'

TimeWindowRo:

description: This datatype is used for readOnly attribute

type: object

properties:

startTime:

$ref: '#/components/schemas/DateTimeRo'

endTime:

$ref: '#/components/schemas/DateTimeRo'

EventTriggerConfig:

description: This defines the configuration parameters of trigger event used for M10 measurement in NR. For details, please refer to TS 32.422 subclause 5.10.X.

type: object

properties:

eventType:

type: string

enum:

- eventA1

- eventA2

thresholdRAN:

type: integer

minimum: 0

maximum: 127

measurementQuantityRAN:

type: string

enum:

- RSRP

hysteresisRAN:

type: integer

minimum: 0

maximum: 30

timeToTriggerRAN:

type: string

enum:

- 0ms

- 40ms

- 64ms

- 80ms

- 100ms

- 128ms

- 160ms

- 256ms

- 320ms

- 480ms

- 512ms

- 640ms

- 1024ms

- 1280ms

- 2560ms

- 5120ms

GeoCoordinate:

type: object

properties:

latitude:

$ref: '#/components/schemas/Latitude'

longitude:

$ref: '#/components/schemas/Longitude'

altitude:

$ref: '#/components/schemas/Float'

GeoArea:

oneOf:

- type: object

properties:

geoPolygon:

type: array

items:

$ref: '#/components/schemas/GeoCoordinate'

minItems: 1

- type: object

properties:

geoCircle:

$ref: '#/components/schemas/GeoCircle'

GeoAreaToCellMapping:

type: object

properties:

geoArea:

$ref: '#/components/schemas/GeoArea'

associationThreshold:

type: integer

AreaOfInterest:

oneOf:

- type: array

items:

$ref: '#/components/schemas/GeoAreaToCellMapping'

- type: array

items:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/Tai'

minItems: 1

maxItems: 8

- type: array

items:

$ref: '#/components/schemas/NrCellId'

minItems: 1

maxItems: 32

- type: array

items:

$ref: '#/components/schemas/EutraCellId'

minItems: 1

maxItems: 32

- type: array

items:

$ref: '#/components/schemas/UtraCellId'

minItems: 1

maxItems: 32

Fqdn:

type: string

description: This datatype is used for writable attribute

pattern: '^([0-9A-Za-z]([-0-9A-Za-z]{0,61}[0-9A-Za-z])?\.)+[A-Za-z]{2,63}\.?$'

minLength: 4

maxLength: 253

FqdnRo:

type: string

description: This datatype is used for readOnly attribute

pattern: '^([0-9A-Za-z]([-0-9A-Za-z]{0,61}[0-9A-Za-z])?\.)+[A-Za-z]{2,63}\.?$'

minLength: 4

maxLength: 253

readOnly: true

Ipv4Addr:

description: This datatype is used for writable attribute

type: string

pattern: '^(([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])\.){3}([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])$'

example: '198.51.100.1'

Ipv4AddrRo:

description: This datatype is used for readOnly attribute

type: string

pattern: '^(([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])\.){3}([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])$'

example: '198.51.100.1'

readOnly: true

Ipv6Addr:

description: This datatype is used for writable attribute

type: string

allOf:

- pattern: '^((:|(0?|([1-9a-f][0-9a-f]{0,3}))):)((0?|([1-9a-f][0-9a-f]{0,3})):){0,6}(:|(0?|([1-9a-f][0-9a-f]{0,3})))$'

- pattern: '^((([^:]+:){7}([^:]+))|((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))$'

example: '2001:db8:85a3::8a2e:370:7334'

Ipv6AddrRo:

description: This datatype is used for readOnly attribute

type: string

allOf:

- pattern: '^((:|(0?|([1-9a-f][0-9a-f]{0,3}))):)((0?|([1-9a-f][0-9a-f]{0,3})):){0,6}(:|(0?|([1-9a-f][0-9a-f]{0,3})))$'

- pattern: '^((([^:]+:){7}([^:]+))|((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))$'

example: '2001:db8:85a3::8a2e:370:7334'

readOnly: true

Ipv6Prefix:

description: This datatype is used for writable attribute

type: string

allOf:

- pattern: '^((:|(0?|([1-9a-f][0-9a-f]{0,3}))):)((0?|([1-9a-f][0-9a-f]{0,3})):){0,6}(:|(0?|([1-9a-f][0-9a-f]{0,3})))(\/(([0-9])|([0-9]{2})|(1[0-1][0-9])|(12[0-8])))$'

- pattern: '^((([^:]+:){7}([^:]+))|((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))(\/.+)$'

example: '2001:db8:abcd:12::0/64'

Ipv6PrefixRo:

description: This datatype is used for readOnly attribute

type: string

allOf:

- pattern: '^((:|(0?|([1-9a-f][0-9a-f]{0,3}))):)((0?|([1-9a-f][0-9a-f]{0,3})):){0,6}(:|(0?|([1-9a-f][0-9a-f]{0,3})))(\/(([0-9])|([0-9]{2})|(1[0-1][0-9])|(12[0-8])))$'

- pattern: '^((([^:]+:){7}([^:]+))|((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))(\/.+)$'

example: '2001:db8:abcd:12::0/64'

readOnly: true

IpAddr:

description: This datatype is used for writable attribute

oneOf:

- $ref: '#/components/schemas/Ipv4Addr'

- $ref: '#/components/schemas/Ipv6Addr'

- $ref: '#/components/schemas/Ipv6Prefix'

IpAddrRo:

description: This datatype is used for readOnly attribute

oneOf:

- $ref: '#/components/schemas/Ipv4AddrRo'

- $ref: '#/components/schemas/Ipv6AddrRo'

- $ref: '#/components/schemas/Ipv6PrefixRo'

HostAddr:

#  This definition will be deprecated, when all occurances of HostAddr

#  are replaced by Host.

oneOf:

- $ref: '#/components/schemas/Ipv4Addr'

- $ref: '#/components/schemas/Ipv6Addr'

- $ref: '#/components/schemas/Fqdn'

Host:

oneOf:

- $ref: '#/components/schemas/IpAddr'

- $ref: '#/components/schemas/Fqdn'

HostRo:

oneOf:

- $ref: '#/components/schemas/IpAddrRo'

- $ref: '#/components/schemas/FqdnRo'

Uri:

description: This datatype is used for writable attribute

type: string

UriRo:

type: string

description: This datatype is used for readOnly attribute

readOnly: true

TimeInterval:

type: object

properties:

intervalStart:

$ref: '#/components/schemas/FullTime'

intervalEnd:

$ref: '#/components/schemas/FullTime'

AdministrativeState:

type: string

enum:

- LOCKED

- UNLOCKED

default: LOCKED

BasicAdministrativeState:

type: string

enum:

- LOCKED

- UNLOCKED

OperationalState:

type: string

enum:

- ENABLED

- DISABLED

default: DISABLED

readOnly: true

UsageState:

type: string

readOnly: true

enum:

- IDLE

- ACTIVE

- BUSY

AvailabilityStatus:

type: string

enum:

- IN_TEST

- FAILED

- POWER_OFF

- OFF_LINE

- OFF_DUTY

- DEPENDENCY

- DEGRADED

- NOT_INSTALLED

- LOG_FULL

readOnly: true

AttributeNameValuePairSet:

description: >-

The key of this map is the attribute name, and the value the attribute value.

type: object

minProperties: 1

additionalProperties:

nullable: true

AttributeValueChangeSet:

description: >-

The first array item contains the attribute name value pairs with the new values,

and the second array item the attribute name value pairs with the optional old values.

type: array

items:

$ref: '#/components/schemas/AttributeNameValuePairSet'

minItems: 1

maxItems: 2

Filter:

description: >-

The filter format shall be compliant to Jex (TS 32.161).

type: string

JexConditions:

type: string

description: >-

The value of the string shall be a Jex expression that is compliant to the

Jex conditions profile specified in clause 7.6 of TS 32.161.

JexNodeSelectionBasic:

type: string

description: >-

The value of the string shall be a Jex expression that is compliant to the

Jex basic profile specified in clause 7.4 of TS 32.161.

JexNodeSelectionAdvanced:

type: string

description: >-

The value of the string shall be a Jex expression that is compliant to the

Jex advanced profile specified in clause 7.5 of TS 32.161.

SystemDN:

type: string

NotificationId:

type: integer

readOnly: true

NotificationType:

oneOf:

- $ref: 'TS28111_FaultNrm.yaml#/components/schemas/AlarmNotificationTypes'

- $ref: 'TS28532_ProvMnS.yaml#/components/schemas/CmNotificationTypes'

- $ref: 'TS28532_PerfMnS.yaml#/components/schemas/PerfNotificationTypes'

- $ref: 'TS28532_HeartbeatNtf.yaml#/components/schemas/HeartbeatNotificationTypes'

- $ref: 'TS28532_FileDataReportingMnS.yaml#/components/schemas/FileNotificationTypes'

NotificationHeader:

type: object

properties:

href:

$ref: '#/components/schemas/Uri'

notificationId:

$ref: '#/components/schemas/NotificationId'

notificationType:

$ref: '#/components/schemas/NotificationType'

eventTime:

$ref: '#/components/schemas/DateTime'

systemDN:

$ref: '#/components/schemas/SystemDN'

sequenceNo:

type: integer

subscriptionId:

type: string

required:

- href

- notificationId

- notificationType

- eventTime

- systemDN

ErrorResponse:

description: >-

Default schema for the response message body in case the request

is not successful.

type: object

properties:

error:

type: object

properties:

errorInfo:

type: string

Subscription:

type: object

properties:

consumerReference:

$ref: '#/components/schemas/Uri'

timeTick:

type: integer

filter:

$ref: '#/components/schemas/JexConditions'

ErrorResponseGet:

description: >-

Default schema for the response message body in case the Get

is not successful.

type: object

properties:

status:

type: string

type:

type: string

reason:

type: string

title:

type: string

badQueryParams:

type: array

items:

type: string

otherProblems:

type: array

items:

type: object

properties:

status:

type: string

type:

type: string

reason:

type: string

title:

type: string

badQueryParams:

type: array

items:

type: string

required:

- type

required:

- type

ErrorResponseDefault:

type: object

properties:

status:

type: string

type:

type: string

reason:

type: string

title:

type: string

badAttributes:

type: array

items:

type: string

badObjects:

type: array

items:

type: string

otherProblems:

type: array

items:

type: object

properties:

status:

type: string

type:

type: string

reason:

type: string

title:

type: string

badAttributes:

type: array

items:

type: string

badObjects:

type: array

items:

type: string

required:

- type

required:

- type

ErrorResponsePatch:

description: >-

Default schema for the response message body in case the patch

is not successful.

type: object

properties:

status:

type: string

type:

type: string

reason:

type: string

title:

type: string

badOP:

type: string

otherProblems:

type: array

items:

type: object

properties:

status:

type: string

type:

type: string

reason:

type: string

title:

type: string

badOp:

type: string

required:

- type

- badOp

required:

- type

- badOp

<CODE ENDS>

*** END OF CHANGE 1 ***

*** START OF CHANGE 2 ***

*** OpenAPI/TS28623_TraceControlNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: Trace Control NRM

version: 19.4.0

description: >-

OAS 3.0.1 definition of the Trace Control NRM fragment

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.623; Generic NRM,Trace Control NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.623/

paths: {}

components:

schemas:

#-------- Definition of types for name-containments ------

SubNetwork-ncO-TraceControlNrm:

type: object

properties:

TraceJob:

$ref: '#/components/schemas/TraceJob-Multiple'

ManagedElement-ncO-TraceControlNrm:

type: object

properties:

TraceJob:

$ref: '#/components/schemas/TraceJob-Multiple'

#-------Definition of generic IOCs ----------#

#-------- Definition of types used in Trace control NRM fragment------------------

TraceTarget:

type: object

description: Trace target conveying both the type and value of the target ID. For additional details see 3GPP TS 32.422

properties:

traceTargetType:

type: string

enum:

- IMSI

- IMEI

- IMEISV

- PUBLIC_ID

- UTRAN_CELL

- EUTRAN_CELL

- NGRAN_CELL

- ENB

- RNC

- GNB

- SUPI

- N4_SESSION_ID

traceTargetValueList:

type: array

items:

type: string

jobType-Type:

type: string

description: Specifies whether the TraceJob represents only MDT, Logged MBSFN MDT, Trace or a combined Trace and MDT job. Applicable for Trace, MDT, RCEF and RLF reporting. See 3GPP TS 32.422 clause 5.9a for additional details.

enum:

- IMMEDIATE_MDT_ONLY

- LOGGED_MDT_ONLY

- TRACE_ONLY

- IMMEDIATE_MDT_AND_TRACE

- RLF_REPORT_ONLY

- RCEF_REPORT_ONLY

- LOGGED_MBSFN_MDT

- 5GC_UE_LEVEL_MEASUREMENTS_ONLY

- TRACE_AND_5GC_UE_LEVEL_MEASUREMENTS

- IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS

- TRACE_AND_IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS

default: TRACE_ONLY

listOfInterfaces-Type:

description: The interfaces to be recorded in the Network Element. See 3GPP TS 32.422 clause 5.5 for additional details.

type: object

properties:

MSCServerInterfaces:

type: array

items:

type: string

enum:

- A

- IU-CS

- MC

- MAP-G

- MAP-B

- MAP-E

- MAP-F

- MAP-D

- MAP-C

- CAP

MGWInterfaces:

type: array

items:

type: string

enum:

- MC

- NB-UP

- IU-UP

RNCInterfaces:

type: array

items:

type: string

enum:

- IU-CS

- IU-PS

- IUR

- IUB

- UU

SGSNInterfaces:

type: array

items:

type: string

enum:

- GB

- IU-PS

- GN

- MAP-GR

- MAP-GD

- MAP-GF

- GE

- GS

- S6D

- S4

- S3

- S13

GGSNInterfaces:

type: array

items:

type: string

enum:

- GN

- GI

- GMB

S-CSCFInterfaces:

type: array

items:

type: string

enum:

- MW

- MG

- MR

- MI

P-CSCFInterfaces:

type: array

items:

type: string

enum:

- GM

- MW

I-CSCFInterfaces:

type: array

items:

type: string

enum:

- CX

- DX

- MG

- MW

MRFCInterfaces:

type: array

items:

type: string

enum:

- Mp

- Mr

MGCFInterfaces:

type: array

items:

type: string

enum:

- MG

- MJ

- MN

IBCFInterfaces:

type: array

items:

type: string

enum:

- IX

- MX

E-CSCFInterfaces:

type: array

items:

type: string

enum:

- MW

- ML

- MM

- MI/MG

BGCFInterfaces:

type: array

items:

type: string

enum:

- MI

- MJ

- MK

ASInterfaces:

type: array

items:

type: string

enum:

- DH

- SH

- ISC

- UT

HSSInterfaces:

type: array

items:

type: string

enum:

- MAP-C

- MAP-D

- GC

- GR

- CX

- S6D

- S6A

- SH

- N70

- N71

- NU1

EIRInterfaces:

type: array

items:

type: string

enum:

- MAP-F

- S13

- MAP-GF

BM-SCInterfaces:

type: array

items:

type: string

enum:

- GMB

MMEInterfaces:

type: array

items:

type: string

enum:

- S1-MME

- S3

- S6A

- S10

- S11

- S13

SGWInterfaces:

type: array

items:

type: string

enum:

- S4

- S5

- S8

- S11

- GXC

PDN_GWInterfaces:

type: array

items:

type: string

enum:

- S2A

- S2B

- S2C

- S5

- S6B

- GX

- S8

- SGI

eNBInterfaces:

type: array

items:

type: string

enum:

- S1-MME

- X2

en-gNBInterfaces:

type: array

items:

type: string

enum:

- S1-MME

- X2

- UU

- F1-C

- E1

AMFInterfaces:

type: array

items:

type: string

enum:

- N1

- N2

- N8

- N11

- N12

- N14

- N15

- N20

- N22

- N26

- N41

- N42

AUSFInterfaces:

type: array

items:

type: string

enum:

- N12

- N13

NEFInterfaces:

type: array

items:

type: string

enum:

- N29

- N30

- N33

NRFInterfaces:

type: array

items:

type: string

enum:

- N27

NSSFInterfaces:

type: array

items:

type: string

enum:

- N22

- N31

PCFInterfaces:

type: array

items:

type: string

enum:

- N5

- N7

- N15

- N28

SMFInterfaces:

type: array

items:

type: string

enum:

- N4

- N7

- N10

- N11

- S5-C

- N16

- N16a

- N38

- N40

SMSFInterfaces:

type: array

items:

type: string

enum:

- N20

- N21

UDMInterfaces:

type: array

items:

type: string

enum:

- N8

- N10

- N13

- N21

- NU1

UPFInterfaces:

type: array

items:

type: string

enum:

- N4

ng-eNBInterfaces:

type: array

items:

type: string

enum:

- NG-C

- XN-C

- UU

gNB-CU-CPInterfaces:

type: array

items:

type: string

enum:

- NG-C

- XN-C

- UU

- F1-C

- E1

- X2-C

gNB-CU-UPInterfaces:

type: array

items:

type: string

enum:

- E1

gNB-DUInterfaces:

type: array

items:

type: string

enum:

- F1-C

listOfNeTypes-Type:

description: The Network Element types where Trace Session activation is needed. See 3GPP TS 32.422 clause 5.4 for additional details.

type: array

uniqueItems: true

items:

type: string

enum:

- MSC_SERVER

- SGSN

- MGW

- GGSN

- RNC

- BM_SC

- MME

- SGW

- PGW

- ENB

- EN_GNB

- GNB_CU_CP

- GNB_CU_UP

- GNB_DU

- AMF

- PCF

- SMF

- UPF

- AUSF

- SMSF

- HSS

- UDM

plmnTarget-Type:

type: object

description: The PLMN for which sessions shall be selected in the Trace Session in case of management based activation when several PLMNs are supported in the RAN (this means that shared cells and not shared cells are allowed for the specified PLMN. Note that the PLMN Target might differ from the PLMN specified in the Trace Reference, as that specifies the PLMN that is containing the management system requesting the Trace Session from the NE. See 3GPP TS 32.422 clause 5.9b for additional details.

properties:

mcc:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Mcc'

mnc:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Mnc'

listOfTraceMetrics:

description: The trace metrics to be reported. See 3GPP TS 32.422 clause 10 for additional details.

type: array

uniqueItems: true

items:

type: string

traceDepth-Type:

description: Specifies how detailed information should be recorded in the Network Element. The Trace Depth is a paremeter for Trace Session level, i.e., the Trace Depth is the same for all of the NEs to be traced in the same Trace Session. See 3GPP TS 32.422 clause 5.3 for additional details.

type: string

enum:

- MINIMUM

- MEDIUM

- MAXIMUM

- VENDORMINIMUM

- VENDORMEDIUM

- VENDORMAXIMUM

default: MAXIMUM

traceReference-Type:

type: object

description: The Trace Reference parameter shall be globally unique, therefore the Trace Reference shall compose as follows - MCC+MNC+Trace ID, where the MCC and MNC are coming with the Trace activation request from the management system to identify one PLMN containing the management system, and Trace ID is a 3 byte Octet String. See 3GPP TS 32.422 clause 5.6 for additional details.

properties:

mcc:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Mcc'

mnc:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Mnc'

traceId:

type: string

traceReportingFormat-Type:

type: string

description: Specifies whether file-based or streaming reporting shall be used for this Trace Session. See 3GPP TS 32.422 clause 5.11 for additional details.

enum:

- FILE_BASED

- STREAMING

default: FILE_BASED

triggeringEvents-Type:

type: object

description: Specifies when to start a Trace Recording Session and which message shall be recorded first, when to stop a Trace Recording Session and which message shall be recorded last respectively. See 3GPP TS 32.422 clause 5.1 for additional details.

properties:

MSC_SERVER:

type: array

items:

type: string

enum:

- MO_MT_CALLS

- MO_MT_SMS

- LU_IMSIATTACH_IMSIDETACH

- HANDOVER

- SS

SGSN:

type: array

items:

type: string

enum:

- PDPCONTEXT

- MO_MT_SMS

- RAU_GPRSATTACH_GPRSDETACH

- MBMSCONTEXT

MGW:

type: array

items:

type: string

enum:

- CONTEXT

GGSN:

type: array

items:

type: string

enum:

- PDPCONTEXT

- MBMSCONTEXT

IMS:

type: array

items:

type: string

enum:

- SIPSESSION_STANDALONETRANSACTION

BM_SC:

type: array

items:

type: string

enum:

- MBMSACTIVATION

MME:

type: array

items:

type: string

enum:

- UE_INITIATEDPDN_CONNECTIVITY_REQUEST

- SERVICE_REQUEST

- INITIALATTACH_TAU_DETACH

- UE_INITIATEDPDN_DISCONNECTION

- BEAR_ACTIVIATION_MODIFICATION_DELETION

- HANDOVER

SGW:

type: array

items:

type: string

enum:

- PDN_CONNECTION_CREATION

- PDN_CONNECTION_TERMINATION

- BEAR_ACTIVATION_MODIFICATION_DELETION

PGW:

type: array

items:

type: string

enum:

- PDN_CONNECTION_CREATION

- PDN_CONNECTION_TERMINATION

- BEAR_ACTIVATION_MODIFICATION_DELETION

AMF:

type: array

items:

type: string

enum:

- REGISTRATION

- SERVICE_REQUEST

- HANDOVER

- UE_DEREGISTRATION

- NETWORK_DEREGISTRATION

- UE_MOBILITY_FROM_EPC

- UE_MOBILITY_TO_EPC

- CONVERGED_CHARGING_SERVICE

SMF:

type: array

items:

type: string

enum:

- PDU_SESSION_ESTABLISHMENT

- PDU_SESSION_MODIFICATION

- PDU_SESSIOON_RELEASE

- PDU_SESSION_UP_ACTIVATION_DEACTIVATION

- MOBILITY_BTW_3GPP_AND_N3GPP_TO_5GC

- MOBILITY_FROM_EPC

- MOBILITY_BTW_ISMF_OR_VSMF

- CONVERGED_CHARGING_SERVICE

- OFFLINE_ONLY_CHARGING

PCF:

type: array

items:

type: string

enum:

- AM_POLICY

- SM_POLICY

- AUTHORIZATION

- BDT_POLICY

- SPENDING_LIMIT_CONTROL

UPF:

type: array

items:

type: string

enum:

- N4SESSION

AUSF:

type: array

items:

type: string

enum:

- UE_AUTHENTICATION

NEF:

type: array

items:

type: string

enum:

- EVENT_EXPOSURE

- PFD_MANAGEMENT

- PARAMETER_PROVISION

- TRIGGER

NRF:

type: array

items:

type: string

enum:

- NF_MANAGEMEENT

- NF_DISCOVERY

NSSF:

type: array

items:

type: string

enum:

- NS_SELECTION

- NSSAI

SMSF:

type: array

items:

type: string

enum:

- SMSERVICE

UDM:

type: array

items:

type: string

enum:

- UE_CONTEXT

- SUBSCRIBER_DATA

- UE_AUTHENTICATION

- EVENT_EXPOSURE

anonymizationOfMdtData-Type:

description: Specifies level of MDT anonymization. For additional details see 3GPP TS 32.422 clause 5.10.12.

type: string

enum:

- NO_IDENTITY

- TAC_OF_IMEI

default: NO_IDENTITY

beamLevelMeasurement-Type:

description: Determines whether beam level measurements shall be included in case of immediate MDT M1 measurement in NR. For additional details see 3GPP TS 32.422 clause 5.10.40.

type: boolean

collectionPeriodRRMLTE-Type:

description: See details in 3GPP TS 32.422 clause 5.10.20.

type: string

enum:

- 100ms

- 1000ms

- 1024ms

- 1280ms

- 2048ms

- 2560ms

- 5120ms

- 10000ms

- 10240ms

- 60000ms

collectionPeriodM6LTE-Type:

description: See details in 3GPP TS 32.422 clause 5.10.32.

type: string

enum:

- 1024ms

- 2048ms

- 5120ms

- 10240ms

collectionPeriodM7LTE-Type:

description: See details in 3GPP TS 32.422 clause 5.10.33.

type: integer

minimum: 1

maximum: 60

collectionPeriodRRMUMTS-Type:

description: See details in 3GPP TS 32.422 clause 5.10.21.

type: string

enum:

- 100ms

- 250ms

- 500ms

- 1000ms

- 2000ms

- 3000ms

- 4000ms

- 6000ms

collectionPeriodRRMNR-Type:

description: See details in 3GPP TS 32.422 clause 5.10.30.

type: string

enum:

- 1024ms

- 2048ms

- 5120ms

- 10240ms

- 60000ms

collectionPeriodM6NR-Type:

description: See details in 3GPP TS 32.422 clause 5.10.34.

type: string

enum:

- 120ms

- 240ms

- 480ms

- 640ms

- 1024ms

- 2048ms

- 5120ms

- 10240ms

- 20480ms

- 40960ms

- 1min

- 6min

- 12min

- 30min

collectionPeriodM7NR-Type:

description: See details in 3GPP TS 32.422 clause 5.10.35.

type: integer

minimum: 1

maximum: 60

eventListForEventTriggeredMeasurement-Type:

description: See details in 3GPP TS 32.422 clause 5.10.28.

type: string

enum:

- OUT_OF_COVERAGE

- A2_EVENT

eventThreshold-Type:

description: See details in 3GPP TS 32.422 clause 5.10.7 (LTE/NR Event A2 RSRP, RSRQ, SINR), 5.10.13 (UMTS Event 1f) and 5.10.14 (UMTS Event 1i).

type: object

properties:

EventThresholdRSRP:

oneOf:

- type: integer

minimum: 0

maximum: 97

- type: integer

minimum: 0

maximum: 127

EventThresholdRSRQ:

oneOf:

- type: integer

minimum: 0

maximum: 34

- type: integer

minimum: 0

maximum: 127

EventThresholdSINR:

type: integer

minimum: 0

maximum: 127

EventThreshold1F:

type: object

properties:

CPICH_RSCP:

type: integer

minimum: -120

maximum: 25

CPICH_EcNo:

type: integer

minimum: -24

maximum: 0

PathLoss:

type: integer

minimum: 30

maximum: 165

EventThreshold1I:

type: integer

minimum: -120

maximum: 25

listOfMeasurements-Type:

description: See details in 3GPP TS 32.422 clause 5.10.3 for details.

type: object

properties:

UMTS:

type: array

items:

type: string

enum:

- M1

- M2

- M3

- M4

- M5

- M6_DL

- M6_UL

- M7_DL

- M7_UL

LTE:

type: array

items:

type: string

enum:

- M1

- M2

- M3

- M4

- M5

- M1_EVENT_TRIGGERED

- M6

- M7

- M8

- M9

NR:

type: array

items:

type: string

enum:

- M1_DL

- M2_DL

- M4_DL

- M4_UL

- M5_DL

- M5_UL

- M6_DL

- M6_UL

- M7_DL

- M7_UL

- M1_EVENT_TRIGGERED

- M8

- M9

- M10

loggingDuration-Type:

description: See details in 3GPP TS 32.422 clause 5.10.9.

type: string

enum:

- 600s

- 1200s

- 2400s

- 3600s

- 5400s

- 7200s

loggingInterval-Type:

description: See details in 3GPP TS 32.422 clause 5.10.8.

type: object

properties:

UMTS:

type: array

items:

type: string

enum:

- 1.28s

- 2.56s

- 5.12s

- 10.24s

- 20.48s

- 30.72s

- 40.96s

- 61.44s

LTE:

type: array

items:

type: string

enum:

- 1.28s

- 2.56s

- 5.12s

- 10.24s

- 20.48s

- 30.72s

- 40.96s

- 61.44s

NR:

type: array

items:

type: string

enum:

- 0.32s

- 0.64s

- 1.28s

- 2.56s

- 5.12s

- 10.24s

- 20.48s

- 30.72s

- 40.96s

- 61.44s

- INFINITY

eventThresholdL1-Type:

description: See details in 3GPP TS 32.422 clause 5.10.36.

type: object

properties:

RSRP:

type: integer

minimum: 0

maximum: 127

RSRQ:

type: integer

minimum: 0

maximum: 127

hysteresisL1-Type:

description: See details in 3GPP TS 32.422 clause 5.10.37.

type: integer

minimum: 0

maximum: 30

timeToTriggerL1-Type:

description: See details in 3GPP TS 32.422 clause 5.10.38.

type: string

enum:

- 0ms

- 40ms

- 64ms

- 80ms

- 100ms

- 128ms

- 160ms

- 256ms

- 320ms

- 480ms

- 512ms

- 640ms

- 1024ms

- 1280ms

- 2560ms

- 5120ms

measurementPeriodLTE-Type:

description: See details in 3GPP TS 32.422 clause 5.10.23.

type: string

enum:

- 1024ms

- 2048ms

- 5120ms

- 10240ms

- 1min

measurementPeriodUMTS-Type:

description: See details in 3GPP TS 32.422 clause 5.10.22.

type: string

enum:

- 1000ms

- 2000ms

- 3000ms

- 4000ms

- 6000ms

- 8000ms

- 12000ms

- 16000ms

- 20000ms

- 24000ms

- 28000ms

- 32000ms

- 64000ms

measurementQuantity-Type:

description: See details in 3GPP TS 32.422 clause 5.10.15.

type: string

enum:

- CPICH_ECNO

- CPICH_RSCP

- PATHLOSS

eventThresholdUphUMTS-Type:

description: See details in 3GPP TS 32.422 clause 5.10.39.

type: integer

minimum: 0

maximum: 31

plmnList-Type:

description: See details in 3GPP TS 32.422 clause 5.10.24.

type: array

uniqueItems: true

items:

type: object

properties:

mcc:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Mcc'

mnc:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Mnc'

maxItems: 16

positioningMethod-Type:

description: See details in 3GPP TS 32.422 clause 5.10.19.

type: string

enum:

- GNSS

- E-CELL_ID

reportAmount-Type:

description: See details in 3GPP TS 32.422 clause 5.10.6.

type: string

enum:

- 1

- 2

- 4

- 8

- 16

- 32

- 64

- INFINITY

reportAmountM1LTE-Type:

description: See details in 3GPP TS 32.422 clause 5.10.6.

type: string

enum:

- 1

- 2

- 4

- 8

- 16

- 32

- 64

- INFINITY

reportAmountM4LTE-Type:

description: See details in 3GPP TS 32.422 clause 5.10.6.

type: string

enum:

- 1

- 2

- 4

- 8

- 16

- 32

- 64

- INFINITY

reportAmountM5LTE-Type:

description: See details in 3GPP TS 32.422 clause 5.10.6.

type: string

enum:

- 1

- 2

- 4

- 8

- 16

- 32

- 64

- INFINITY

reportAmountM6LTE-Type:

description: See details in 3GPP TS 32.422 clause 5.10.6.

type: string

enum:

- 1

- 2

- 4

- 8

- 16

- 32

- 64

- INFINITY

reportAmountM7LTE-Type:

description: See details in 3GPP TS 32.422 clause 5.10.6.

type: string

enum:

- 1

- 2

- 4

- 8

- 16

- 32

- 64

- INFINITY

reportAmountM1NR-Type:

description: See details in 3GPP TS 32.422 clause 5.10.6.

type: string

enum:

- 1

- 2

- 4

- 8

- 16

- 32

- 64

- INFINITY

reportAmountM4NR-Type:

description: See details in 3GPP TS 32.422 clause 5.10.6.

type: string

enum:

- 1

- 2

- 4

- 8

- 16

- 32

- 64

- INFINITY

reportAmountM5NR-Type:

description: See details in 3GPP TS 32.422 clause 5.10.6.

type: string

enum:

- 1

- 2

- 4

- 8

- 16

- 32

- 64

- INFINITY

reportAmountM6NR-Type:

description: See details in 3GPP TS 32.422 clause 5.10.6.

type: string

enum:

- 1

- 2

- 4

- 8

- 16

- 32

- 64

- INFINITY

reportAmountM7NR-Type:

description: See details in 3GPP TS 32.422 clause 5.10.6.

type: string

enum:

- 1

- 2

- 4

- 8

- 16

- 32

- 64

- INFINITY

reportingTrigger-Type:

description: See details in 3GPP TS 32.422 clause 5.10.4.

type: array

items:

type: string

enum:

- PERIODICAL

- A2_FOR_LTE_NR

- 1F_FOR_UMTS

- 1I_FOR_UMTS_MCPS_TDD

- A2_TRIGGERED_PERIODIC_FOR_LTE_NR

- ALL_CONFIGURED_RRM_FOR_LTE_NR

- ALL_CONFIGURED_RRM_FOR_UMTS

reportInterval-Type:

description: See details in 3GPP TS 32.422 clause 5.10.5.

type: object

properties:

UMTS:

type: array

items:

type: string

enum:

- 250ms

- 500ms

- 1000ms

- 2000ms

- 3000ms

- 4000ms

- 6000ms

- 8000ms

- 12000ms

- 16000ms

- 20000ms

- 24000ms

- 28000ms

- 32000ms

- 64000ms

LTE:

type: array

items:

type: string

enum:

- 120ms

- 240ms

- 480ms

- 640ms

- 1024ms

- 2048ms

- 5120ms

- 10240ms

- 60000ms

- 360000ms

- 720000ms

- 1800000ms

- 3600000ms

NR:

type: array

items:

type: string

enum:

- 120ms

- 240ms

- 480ms

- 640ms

- 1024ms

- 2048ms

- 5120ms

- 10240ms

- 20480ms

- 40960ms

- 60000ms

- 360000ms

- 720000ms

- 1800000ms

reportType-Type:

description: Report type for logged NR MDT. See details in 3GPP TS 32.422 clause 5.10.27.

type: string

enum:

- PERIODICAL

- EVENT_TRIGGERED

sensorInformation-Type:

description: See details in 3GPP TS 32.422 clause 5.10.29.

type: array

uniqueItems: true

items:

type: string

enum:

- BAROMETRIC_PRESSURE

- UE_SPEED

- UE_ORIENTATION

traceCollectionEntityId-Type:

description: See details in 3GPP TS 32.422 clause 5.10.11. Only TCE Id value may be sent over the air to the UE being configured for Logged MDT.

type: integer

excessPacketDelayThreshold-Type:

description: Excess Packet Delay Threshold for NR MDT. See details in 3GPP TS 32.422 clause 4.1.1 and 4.1.2.

type: object

properties:

fiveQIValue:

type: integer

excessPacketDelayThresholdValue:

type: string

enum:

- 0.25MS

- 0.5MS

- 1MS

- 2MS

- 4MS

- 5MS

- 10MS

- 20MS

- 30MS

- 40MS

- 50MS

- 60MS

- 70MS

- 80MS

- 90MS

- 100MS

- 150MS

- 300MS

- 500MS

excessPacketDelayThresholds-Type:

description: Array of type excessPacketDelayThreshold-Type.

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/excessPacketDelayThreshold-Type'

minItems: 0

maxItems: 255

layerOneRsrpPeriodicity-Type:

description: Periodicity that the UE shall use for the logging of the M10 measurements. See details in 3GPP TS 32.422 clause 5.10.X.

type: integer

enum:

- 2

- 3

- 4

- 5

traceConfig-Type:

description: Trace configuration parameters for NR. See details in 3GPP TS 28.622 clause 4.3.30.

type: object

properties:

listOfInterfaces:

$ref: '#/components/schemas/listOfInterfaces-Type'

listOfNeTypes:

$ref: '#/components/schemas/listOfNeTypes-Type'

traceDepth:

$ref: '#/components/schemas/traceDepth-Type'

triggeringEvents:

$ref: '#/components/schemas/triggeringEvents-Type'

immediateMDTConfig-Type:

description: Immediate MDT configuration parameters. See details in 3GPP TS 28.622 clause 4.3.30.

type: object

properties:

listOfMeasurements:

$ref: '#/components/schemas/listOfMeasurements-Type'

reportingTrigger:

$ref: '#/components/schemas/reportingTrigger-Type'

reportAmount:

$ref: '#/components/schemas/reportAmount-Type'

reportAmountM1LTE:

$ref: '#/components/schemas/reportAmountM1LTE-Type'

reportAmountM4LTE:

$ref: '#/components/schemas/reportAmountM4LTE-Type'

reportAmountM5LTE:

$ref: '#/components/schemas/reportAmountM5LTE-Type'

reportAmountM6LTE:

$ref: '#/components/schemas/reportAmountM6LTE-Type'

reportAmountM7LTE:

$ref: '#/components/schemas/reportAmountM7LTE-Type'

reportAmountM1NR:

$ref: '#/components/schemas/reportAmountM1NR-Type'

reportAmountM4NR:

$ref: '#/components/schemas/reportAmountM4NR-Type'

reportAmountM5NR:

$ref: '#/components/schemas/reportAmountM5NR-Type'

reportAmountM6NR:

$ref: '#/components/schemas/reportAmountM6NR-Type'

reportAmountM7NR:

$ref: '#/components/schemas/reportAmountM7NR-Type'

reportInterval:

$ref: '#/components/schemas/reportInterval-Type'

eventThreshold:

$ref: '#/components/schemas/eventThreshold-Type'

collectionPeriodRRMLTE:

$ref: '#/components/schemas/collectionPeriodRRMLTE-Type'

collectionPeriodM6LTE:

$ref: '#/components/schemas/collectionPeriodM6LTE-Type'

collectionPeriodM7LTE:

$ref: '#/components/schemas/collectionPeriodM7LTE-Type'

collectionPeriodRRMUMTS:

$ref: '#/components/schemas/collectionPeriodRRMUMTS-Type'

collectionPeriodRRMNR:

$ref: '#/components/schemas/collectionPeriodRRMNR-Type'

collectionPeriodM6NR:

$ref: '#/components/schemas/collectionPeriodM6NR-Type'

collectionPeriodM7NR:

$ref: '#/components/schemas/collectionPeriodM7NR-Type'

eventThresholdUphUMTS:

$ref: '#/components/schemas/eventThresholdUphUMTS-Type'

measurementPeriodUMTS:

$ref: '#/components/schemas/measurementPeriodUMTS-Type'

measurementPeriodLTE:

$ref: '#/components/schemas/measurementPeriodLTE-Type'

measurementQuantity:

$ref: '#/components/schemas/measurementQuantity-Type'

beamLevelMeasurement:

$ref: '#/components/schemas/beamLevelMeasurement-Type'

positioningMethod:

$ref: '#/components/schemas/positioningMethod-Type'

excessPacketDelayThresholds:

$ref: '#/components/schemas/excessPacketDelayThresholds-Type'

layerOneRsrpPeriodicity:

$ref: '#/components/schemas/layerOneRsrpPeriodicity-Type'

eventTriggerConfig:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/EventTriggerConfig'

loggedMDTConfig-Type:

description: Logged MDT configuration parameters. See details in 3GPP TS 28.622 clause 4.3.30.

type: object

properties:

traceCollectionEntityId:

$ref: '#/components/schemas/traceCollectionEntityId-Type'

loggingDuration:

$ref: '#/components/schemas/loggingDuration-Type'

loggingInterval:

$ref: '#/components/schemas/loggingInterval-Type'

eventThresholdL1:

$ref: '#/components/schemas/eventThresholdL1-Type'

hysteresisL1:

$ref: '#/components/schemas/hysteresisL1-Type'

timeToTriggerL1:

$ref: '#/components/schemas/timeToTriggerL1-Type'

mbsfnAreaList:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/MbsfnArea'

maxItems: 8

reportType:

$ref: '#/components/schemas/reportType-Type'

eventListForEventTriggeredMeasurement:

$ref: '#/components/schemas/eventListForEventTriggeredMeasurement-Type'

areaConfigurationForNeighCell:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/AreaConfig'

maxItems: 32

nPNIdentityList:

type: array

uniqueItems: true

items:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/NpnId-Type'

mdtConfig-Type:

description: MDT config parameters. See details in 3GPP TS 28.622 clause 4.3.30.

type: object

properties:

anonymizationOfMdtData:

$ref: '#/components/schemas/anonymizationOfMdtData-Type'

areaScope:

type: array

uniqueItems: true

items:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/AreaScope'

sensorInformation:

$ref: '#/components/schemas/sensorInformation-Type'

immediateMDTConfig:

$ref: '#/components/schemas/immediateMDTConfig-Type'

loggedMDTConfig:

$ref: '#/components/schemas/loggedMDTConfig-Type'

plmnList:

$ref: '#/components/schemas/plmnList-Type'

UECoreMeasConfig-Type:

description: UE level measurements configuration parameters for 5G system. See details in 3GPP TS 28.622 clause 4.3.x.

type: object

properties:

ueCoreMeasurements:

type: array

items:

type: string

minItems: 1

ueCoreMeasGranularityPeriod:

type: integer

nfTypeToMeasure:

type: string

objectInstances:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

rootObjectInstances:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

AreaScope:

oneOf:

- type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/EutraCellId'

- type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/NrCellId'

- type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Tac'

- type: array

uniqueItems: true

items:

$ref: '#/components/schemas/Tai'

- type: array

uniqueItems: true

items:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/NpnId-Type'

- type: array

items:

$ref: '#/components/schemas/PLMNInfo'

Tai:

type: object

properties:

mcc:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Mcc'

mnc:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Mnc'

tac:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Tac'

PLMNInfo:

type: object

properties:

plmnId:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/PlmnId'

sNSSAI:

type: array

items:

$ref: 'TS28541_NrNrm.yaml#/components/schemas/Snssai'

AreaConfig:

type: object

properties:

freqInfo:

$ref: '#/components/schemas/FreqInfo'

pciList:

type: array

uniqueItems: true

items:

type: integer

minimum: 0

maximum: 1007

minItems: 1

maxItems: 32

FreqInfo:

description: specifies the carrier frequency and bands used in a cell.

type: object

properties:

arfcn:

type: integer

freqBands:

type: array

uniqueItems: true

items:

type: integer

minItems: 1

MbsfnArea:

type: object

properties:

mbsfnAreaId:

type: integer

minimum: 1

earfcn:

type: integer

minimum: 1

TraceJob-Attr:

type: object

description: abstract class used as a container of all TraceJob attributes

properties:

jobType:

$ref: '#/components/schemas/jobType-Type'

plmnTarget:

$ref: '#/components/schemas/plmnTarget-Type'

nPNTarget:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/NpnId-Type'

listOfTraceMetrics:

type: array

uniqueItems: true

items:

type: string

traceReportingConsumerUri:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

traceCollectionEntityIpAddress:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/IpAddr'

traceReference:

$ref: '#/components/schemas/traceReference-Type'

jobId:

type: string

traceReportingFormat:

$ref: '#/components/schemas/traceReportingFormat-Type'

traceTarget:

$ref: '#/components/schemas/TraceTarget'

traceConfig:

$ref: '#/components/schemas/traceConfig-Type'

mdtConfig:

$ref: '#/components/schemas/mdtConfig-Type'

ueCoreMeasConfig:

$ref: '#/components/schemas/UECoreMeasConfig-Type'

#-------- end of Definition of types used in Trace control NRM fragment ----------

#-------- Definition of concrete IOCs --------------------------------------------

TraceJob-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

$ref: '#/components/schemas/TraceJob-Attr'

Files:

$ref: 'TS28623_FileManagementNrm.yaml#/components/schemas/Files-Multiple'

#-------- Definition of YAML arrays for name-contained IOCs ----------------------

TraceJob-Multiple:

type: array

items:

$ref: '#/components/schemas/TraceJob-Single'

#-------- Definitions in TS 28.623 for TS 28.532 ---------------------------------

resources-traceControlNrm:

oneOf:

- $ref: '#/components/schemas/TraceJob-Single'

<CODE ENDS>

*** END OF CHANGE 2 ***