3GPP TSG-SA WG6 Meeting #33	S6-191942

Sophia Antipolis, France, 2nd – 6th September 2019	(revision of S6-191859)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0174 | rev | 2 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Location infomation to SDS and Enhance Status |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | HOME OFFICE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCData2 |  |  |  |  |  |  | Date: |  | 2019-09-04 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Location information is important for Emergency Services in order to manage the resources in the field. To reduce the amount of voice traffic the Emergency Services are using more data services to manage the the users, for example Enhanced Status is used to send the status of an Ambulance Crew to their control room: “At Scene”, “Leaving Scn”, etc. The addition of the Location information allows the Control Room to manage the resources on the ground more efficiently. The increasing use of data, SDS, will also require Location Information. The Location Information is also used for auditing purposes and may be required by a court of law in the future. SA1 requirements: From 22.282 requirements [R-5.2.2-012] The MCData SDS shall provide the option to add a field indicating location of the sending user/UE. [R-5.2.2-013] The MCData SDS shall allow empty messages including only a field indicating location of the sending user/UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of Location Information as an optional parameter as part of the SDS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reduces the capabilities of a Control Room to manage it’s resources efficiently, also the Location information is important for audit trails. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.2.1.1, 7.4.2.1.3, 7.4.2.1.5, 7.4.2.1.7, 7.4.2.1.8, 7.4.2.1.10, 7.4.2.1.11, 7.4.2.1.13, 7.4.2.1.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

##### 7.4.2.1.1	MCData standalone data request

Table 7.4.2.1.1-1 describes the information flow for the MCData standalone data request sent from the MCData client to the MCData server and from the MCData server to another MCData client.

Table 7.4.2.1.1-1: MCData standalone data request

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData ID | M | The identity of the MCData user towards which the data is sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the payload is for application consumption or MCData client consumption |
| Location | O | Location of the Originating MCData user sending the SDS |
| Application identifier (see NOTE) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| Payload | M | SDS content |
| NOTE: The application identifier shall be included only if the payload destination type indicates that the payload is for application consumption. |  |  |

##### 7.4.2.1.3	MCData standalone session data request

Table 7.4.2.1.3-1 describes the information flow for the MCData standalone session data request sent from the MCData client to the MCData server and from the MCData server to another MCData client.

Table 7.4.2.1.3-1: MCData standalone session data request

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData ID | M | The identity of the MCData user towards which the data is sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Transaction type | M | Standalone transaction |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the SDS payload is for application consumption or MCData user consumption |
| Location | O | Location of the Originating MCData user sending the SDS message |
| Application identifier (see NOTE) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| SDP offer | M | Media parameters offered |
| NOTE: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption. |  |  |

##### 7.4.2.1.5	MCData session data request

Table 7.4.2.1.5-1 describes the information flow for the MCData session data request sent from the MCData client to the MCData server and from the MCData server to another MCData client.

Table 7.4.2.1.5-1: MCData session data request

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData ID | M | The identity of the MCData user towards which the data is sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Transaction type | M | Session based transactions |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the SDS payload is for application consumption or MCData user consumption |
| Location | O | Location of the Originating MCData user sending the SDS message |
| Application identifier (see NOTE) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| SDP offer | M | Media parameters offered |
| Requested priority | O | Application priority level requested for this communication session |
| NOTE: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption. |  |  |

##### 7.4.2.1.7	MCData group standalone data request (MCData client – MCData server)

Table 7.4.2.1.7-1 describes the information flow for the MCData group standalone data request (in subclause 7.4.2.5.2) sent from the MCData client to the MCData server.

Table 7.4.2.1.7-1: MCData group standalone data request (MCData client – MCData server)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData group ID | M | The MCData group ID to which the data is to be sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the payload is for application consumption or MCData client consumption |
| Location | O | Location of the Originating MCData user sending the SDS |
| Application identifier (see NOTE) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| Payload | M | SDS content |
| NOTE: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption. |  |  |

##### 7.4.2.1.8	MCData group standalone data request (MCData server – MCData client)

Table 7.4.2.1.8-1 describes the information flow for the MCData group standalone data request (in subclause 7.4.2.5.2) sent from the MCData server to the MCData client.

Table 7.4.2.1.8-1: MCData group standalone data request (MCData server – MCData client)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData group ID | M | The MCData group ID to which the data is to be sent |
| MCData ID | M | The identity of the MCData user towards which the data is sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the payload is for application consumption or MCData client consumption |
| Location | O | Location of the Originating MCData user sending the SDS |
| Application identifier (see NOTE) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| Payload | M | SDS content |
| NOTE: The application identifier shall be included only if the payload destination type indicates that the payload is for application consumption. |  |  |

##### 7.4.2.1.10	MCData group session standalone data request (MCData client – MCData server)

Table 7.4.2.1.10-1 describes the information flow for the MCData group session standalone data request (in subclause 7.4.2.6.2) sent from the MCData client to the MCData server.

Table 7.4.2.1.10-1: MCData group session standalone data request (MCData client – MCData server)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData group ID | M | The MCData group ID to which the data is to be sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Transaction type | M | Standalone transaction |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the SDS payload is for application consumption or MCData user consumption |
| Location | O | Location of the Originating MCData user sending the SDS message |
| Application identifier (see NOTE) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| SDP offer | M | Media parameters offered |
| Requested priority | O | Application priority level requested for this communication session |
| NOTE: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption. |  |  |

##### 7.4.2.1.11	MCData group session standalone data request (MCData server – MCData client)

Table 7.4.2.1.11-1 describes the information flow for the MCData group session standalone data request (in subclause 7.4.2.6.2) sent from the MCData server to the MCData client.

Table 7.4.2.1.11-1: MCData group session standalone data request (MCData server – MCData client)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData group ID | M | The MCData group ID to which the data is to be sent |
| MCData ID | M | The identity of the MCData user towards which the data is sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Transaction type | M | Standalone transaction |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the SDS payload is for application consumption or MCData user consumption |
| Location | O | Location of the Originating MCData user sending the SDS message |
| Application identifier (see NOTE) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| SDP offer | M | Media parameters offered |
| NOTE: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption. |  |  |

##### 7.4.2.1.13	MCData group data request (MCData client – MCData server)

Table 7.4.2.1.13-1 describes the information flow for the MCData group data request sent from the MCData client to the MCData server.

Table 7.4.2.1.13-1: MCData group data request (MCData client – MCData server)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData group ID | M | The MCData group ID to which the data is to be sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Transaction type | M | Session based transactions |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the SDS payload is for application consumption or MCData user consumption |
| Location | O | Location of the Originating MCData user sending the SDS message |
| Application identifier (see NOTE) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| SDP offer | M | Media parameters offered |
| Requested priority | O | Application priority level requested for this communication session |
| NOTE: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption. |  |  |

##### 7.4.2.1.14	MCData group data request (MCData server – MCData client)

Table 7.4.2.1.14-1 describes the information flow for the MCData group data request sent from the MCData server to the MCData client.

Table 7.4.2.1.14-1: MCData group data request (MCData server – MCData client)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData group ID | M | The MCData group ID to which the data is to be sent |
| MCData ID | M | The identity of the recipient MCData user |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Transaction type | M | Session based transactions |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the SDS payload is for application consumption or MCData user consumption |
| Location | O | Location of the Originating MCData user sending the SDS message |
| Application identifier (see NOTE) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| SDP offer | M | Media parameters offered |
| NOTE: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption. |  |  |