SA WG2 Meeting #137E (e-meeting)	S2-2001822

# Feb 24 - 27, 2020, Elbonia		                                          			  (revision of S2-2001822)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 1834 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification related with the (non) support of PWS and LADN on Wireline access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2020-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clause 4.4 does not mention that LADN is not supported for FN RG |  |  |  |  |  |  |  |  |
| Summary of change: |  | The LADN service defined in clause 5.6.5 in TS 23.501 does not apply for RG connected to 5GC via wireline access. PWS functionality as described in TS 23.041 [xx] is not supported for Wireline access but may be supported by RG(s) connected over 3GPP access. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong understanding from BBF about LADN and PWS support |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4 ; 4.9.0 ; 2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 4.4.0	General

This clause specifies the delta related to session management defined in TS 23.501 [2] clause 5.6.

The LADN service defined in clause 5.6.5 in TS 23.501 [2] does not apply for RG connected to 5GC via wireline access.

### 4.4.1	Session management for 5G-RG

Session management of 5G-RG connected to 5GC via wireline access follows the principle defined in TS 23.501 [2] clause 5.6 with the following difference:

-	UE is replaced by 5G-RG.

-	5G-RG is connected to 5GC via wireline RAT type instead of 3GPP access.

-

### 4.4.2	Session management for FN-RG

Session management of FN-RG follows the principle defined in TS 23.501 [2] clause 5.6 with the follow difference:

-	UE is replaced by W-AGF

-	FN-RG is connected to 5GC via wireline access instead of 3GPP access.

-	Secondary authentication/authorization by a DN-AAA server during the establishment of a PDU Session of FN-RG is not applicable.

NEXT CHANGE (2)

## 4.9	Support of specific services

### 4.9.0	General

This clause specifies high level definition of services specific for WWC scenario.

PWS functionality as described in TS 23.041 [xx] is not supported for Wireline access but may be supported by RG(s) connected over 3GPP access.

NEXT CHANGE (3)

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	BBF TR-124 issue 5: "Functional Requirements for Broadband Residential Gateway Devices".

[6]	BBF TR-101 issue 2: "Migration to Ethernet-Based Broadband Aggregation".

[7]	BBF TR-178 issue 1: "Multi-service Broadband Network Architecture and Nodal Requirements".

[8]	CableLabs DOCSIS MULPI: "Data-Over-Cable Service Interface Specifications DOCSIS 3.1, MAC and Upper Layer Protocols Interface Specification".

[9]	BBF WT-456: "AGF Functional Requirements".

[10]	BBF WT-457: "FMIF Functional Requirements".

Editor's note:	The references to BBF WT-456 and WT-457 will be revised when finalized by BBF.

[11]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[12]	BBF TR-177 Issue 1 Corrigendum 1: "IPv6 in the context of TR-101".

[13]	IETF RFC 6788: "The Line-Identification Option".

[14]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[15]	IETF RFC 3315: "Dynamic Host Configuration Protocol for IPv6 (DHCPv6)".

[16]	IETF RFC 6603: "Prefix Exclude Option for DHCPv6-based Prefix Delegation".

[17]	IETF RFC 3633: "IPv6 Prefix Options for Dynamic Host Configuration Protocol (DHCP) version 6".

[18]	BBF TR-069: "CPE WAN Management Protocol".

[19]	BBF TR-369: "User Services Platform (USP)".

[20]	IETF RFC 3046: "DHCP Relay Agent Information Option".

[21]	IETF RFC 4604: "Using Internet Group Management Protocol Version 3 (IGMPv3) and Multicast Listener Discovery Protocol Version 2 (MLDv2) for Source-Specific Multicast".

[22]	3GPP TR 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[23]	3GPP TS 38.413: "NG RAN; NG Application Protocol (NGAP)".

[24]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[25]	3GPP TS 22.011: "Service accessibility".

[26]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[27]	CableLabs WR-TR-5WWC-ARCH: "5G Wireless Wireline Converged Core Architecture".

[28]	IETF RFC 3376: "Internet Group Management Protocol, Version 3".

[29]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS)".

[30]	BBF TR-198: "DQS:DQM systems functional architecture and requirements".

[31]	3GPP TS 23.203: "Policy and charging control architecture".

[32]	3GPP TS 33.126: "Lawful Interception Requirements".

[33]	IETF RFC 2236: "Internet Group Management Protocol, Version 2".

[34]	IETF RFC 4861: "Neighbor Discovery for IP version 6 (IPv6)".

[35]	IETF RFC 1112: "Internet Group Management Protocol".

[36]	IETF RFC 2710: "Multicast Listener Discovery Version for IPv6".

[37]	IETF RFC 2010: "Operational Criteria for Root Name Servers".

[xx]	3GPP TS 23.041: "Public Warning System".

NEXT CHANGE (4)

NEXT CHANGE (5)

END OF CHANGES