# 3GPP TSG SA Meeting #87E	SP-200086

# 17-20 March 2020, Electronic meeting

Source:	SA WG2

Title:	New WID: Dynamically Changing AM Policies in the 5GC (TEI17)

Document for:	Approval

Agenda Item:	6.4

3GPP TSG-WG SA2 Meeting #137E e-meeting	S2-2002599

Elbonia, February 24 – 27, 2020	(revision of xx-yyxxxx)

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title:	Dynamically Changing AM Policies in the 5GC

# Acronym:	TEI17_DCAMP

# Unique identifier:	870005

# Potential target Release:	Rel-17

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan.

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

Dependency on non-3GPP (draft) specification:

## 3	Justification

The RFSP is a useful parameter that allows networks to steer certain services to preferred carriers in the RAN. This can be for reasons of capacity management, improving coverage, to access features that are only available on certain carriers, to avoid inter-modulation issues and more. In some circumstances, the appropriate RFSP index for a certain service will change dynamically.

Release 16 supports AM policies in the PCF, but the options for dynamically triggering the establishment/modification/termination of AM policies by third parties or based on application detection have been overlooked.

The use cases detailed in S2- 2002195 call for allowing to dynamically change AM policies.

## 4	Objective

The objective is to specify an NF service for the authorization of AF requests to create/modify/terminate AM policies.

This service should have create/modify/terminate operations like those defined for the Npcf_PolicyAuthorization Service. However, as the AF may not know the UE address, required input for these operations should be any one of UE address, SUPI or GPSI. For the use cases we identified, the optional inputs and the outputs could be the same as for Npcf_PolicyAuthorization Service operations. However, the exact list of inputs and outputs is expected to be defined by SA2.

NOTE: Based on architectural criteria, SA2 is expected to decide whether to create a new NF service, or extend an existing one.

PCF shall be considered among the example consumers of this NF service as well; this is to recognize that an SM policy decision may trigger the need for a new AM policy decision.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 TS 23.502 TS 23.503 |  | TSG#88 | This will be handled as “TEI17”. |

## 6	Work item Rapporteur(s)

Ropolyi, Robert 	Robert dot Ropolyi at team dot telstra dot com

## 7	Work item leadership

SA2.

## 8	Aspects that involve other WGs

None.

## 9	Supporting Individual Members

Supporting IM name

Telstra

Orange

Charter Communications

AT&T

Verizon UK Ltd

Vodafone