# 3GPP TSG SA Meeting #88e	SP-200599_Rev2

# e-meeting, 30 June -3 July 2020	revision of SP-20xxxx

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 0856 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on SUPI definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-Sec |  |  |  |  |  | Date: |  |  | 2020-06-25 |
| Category: |  |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SUPI is well defined by 2.2A of TS23.003 as following, where SUPI contains SUPI type. It is important that we avoid the introduction of any misunderstanding or redefining of SUPI in this specification. 2.2A Subscription Permanent Identifier (SUPI) The SUPI is a globally unique 5G Subscription Permanent Identifier allocated to each subscriber in the 5G System. It is defined in clause 5.9.2 of 3GPP TS 23.501 [119]. The SUPI is defined as: - a SUPI type: in this release of the specification, it may indicate an IMSI, a network specific identifier, a Global Line Identifier (GLI) or a Global Cable Identifier (GCI); and - dependent on the value of the SUPI type: - an IMSI as defined in clause 2.1; - a network specific identifier, taking the form of a Network Access Identifier (NAI) as defined in clause 28.7.2; - a Global Cable Identifier (GCI) taking the form of a NAI as defined in clause 28.15.2; - a Global Line Identifier (GLI) taking the form of an NAI as defined in clause 28.16.2. NOTE: Depending on the protocol used to convey the SUPI, the SUPI type can take different formats. See clauses 4.7.2, 4.7.3 and 4.7.4 of 3GPP TS 23.316 [131] for details on which types of SUPI are supported by 5G-BRG, FN-BRG, 5G-CRG and FN-CRG. - in A.7.0. of TS 33.501 the original text actually means P0 is set to IMSI or NAI based on the SUPI type. However the current wording might give the impression that another definition SUPI is provided that can exist without a SUPI type. TS 33.501 shall not redefine SUPI in conflict with TS 23.003, if SUPI type is not used then it is not SUPI anymore, it is just a subset of string of SUPI. Note: for R16, SUPI type is extended to GLI and GCI, thus the corresponding description is added. - In F.3 of TS 33.501, It is clearly mentioned UE shall use SUPI as the Identity for key derivation. There is a proposal in SP-200550 to further clarify what is the identity, however that proposal conflicts with the original text and introduces a non-backward compatibility change. TS 23.003 has a clear definition of SUPI which contains the SUPI type mandatorily as part of its definition. So as a reminder for implementation, a reference to TS 23.003 can be added here. Encoding of SUPI refer to TS 25.971 and this is the only CT protocol define encoding of SUPI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the wording in Annex A.7.0 of TS 33.501, to clarify for P0= IMSI or NAI or GLI or GCI for Rel16，which depends on the SUPI type. - Add a reference in F.3 of TS 33.501 that SUPI definition refers to TS 23.003. The encoding of SUPI is specified in TS 29.571. - Add TS29.571 in Reference |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It might lead to the misunderstanding that there are two different definitions of SUPI in 3GPP specifications which might cause different interpretations by ME and the core network implementations and can lead to interoperability problems and failure of authentication |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.3, A.7.0, 2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** START OF 1st CHANGES ****

F.3	Subscriber identity and key derivation

EAP-AKA' uses the subscriber identity (Identity) as an input to the key derivation when the key derivation function has value 1 ( i.e. MK = PRF'(IK'|CK',"EAP-AKA'"|Identity)). RFC 4187 [21] clause 7 describes that the Identity is taken from the EAP-Response/Identity or EAP-Response/AKA-Identity AT_IDENTITY attribute sent by the peer. This principle is not applied to the 5GS.

If the AT_KDF_INPUT parameter contains the prefix "5G:", the AT_KDF parameter has the value 1 and the authentication is not related to fast re-authentication, then the UE shall use SUPI (as defined in clause 2.2A of TS 23.003 [19]) as the Identity for key derivation. The encoding of SUPI is specified in TS 29.571[x], i.e. the encoding of SUPI will take the value of imsi-<IMSI> or nai-<NAI> or gci-<GCI>or gli-<GLI> depending on SUPI Type. This principle applies to all full EAP-AKA' authentications, even if the UE sent SUCI in NAS protocol or if the UE sent SUCI in the respose to the EAP identity requests as described in Table F.2-1 or if no identity was sent because the network performed re-authentication. The only exception is fast re-authentication when the UE follows the key derivation as described in RFC 5448 [12] for fast re-authentication.

NOTE 1:	The fast re-authentication is not supported in 5GS.

NOTE 2: 	The prefix "5G:" is part of serving network name as specified in clause 6.1.1.4.

**** END OF 1st CHANGES ****

**** START OF 2nd CHANGES ****

A.7	KAMF derivation function

A.7.0	Parameters for the input S to the KDF

When deriving a KAMF from KSEAF the following parameters shall be used to form the input S to the KDF.

-	FC = 0x6D

-	P0 = IMSI or NAI or GLI or GCI (dependent on the value of the SUPI type)

-	L0 = P0 length - number of octets in P0

-	P1 = ABBA parameter

-	L1 = P1 length - number of octets in P1

The input key KEY shall be the 256-bit KSEAF

When the SUPI type is IMSI, P0 shall be set to IMSI as defined in clause 2.2 of TS 23.003 [19]. When the SUPI type is network specific identifier, P0 shall be set to Network Access Identifier (NAI) as defined in clause 28.7.2 of TS 23.003 [19]. When the SUPI type is GLI, P0 shall be set to GLI taking format of NAI as defined in clause 28.15.2 of TS 23.003 [19]. When the SUPI type is GCI, P0 shall be set to GCI taking format of NAI as defined in clause 28.16.2 of TS 23.003 [19]. P0 shall be represented as a character string as specified in B.2.1.2 of TS 33.220 [28].

For ABBA parameter values please refer to clause A.7.1.

**** END OF 2nd CHANGES ****

**** START OF 3rd CHANGES ****

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[4]	IETF RFC 4303: "IP Encapsulating Security Payload (ESP)".

[5]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[6]	IETF RFC 4301: "Security Architecture for the Internet Protocol".

[7]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[8]	3GPP TS 23.502: "Procedures for the 5G System".

[9]	3GPP TS 33.102: "3G security; Security architecture".

[10]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[11]	3GPP TS 33.402: "3GPP System Architecture Evolution (SAE); Security aspects of non-3GPP accesses".

[12]	IETF RFC 5448: " Improved Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA')".

Editor’s note:	This reference will be removed and references to it updated when the IETF updates the RFC and publishes a new RFC that supercedes this RFC.

[13]	3GPP TS 24.301: " Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[14]	3GPP TS 35.215: " Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2; Document 1: UEA2 and UIA2 specifications".

[15]	NIST: "Advanced Encryption Standard (AES) (FIPS PUB 197)".

[16]	NIST Special Publication 800-38A (2001): "Recommendation for Block Cipher Modes of Operation".

[17]	NIST Special Publication 800-38B (2001): "Recommendation for Block Cipher Modes of Operation: The CMAC Mode for Authentication".

[18]	3GPP TS 35.221: " Specification of the 3GPP Confidentiality and Integrity Algorithms EEA3 & EIA3; Document 1: EEA3 and EIA3 specifications".

[19]	3GPP TS 23.003: "Numbering, addressing and identification".

[20]	3GPP TS 22.101: "Service aspects; Service principles".

[21]	IETF RFC 4187: "Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA)".

[22]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[23]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[24]	3GPP TS 33.117: "Catalogue of general security assurance requirements".

[25]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2)"

[26]	Void

[27]	IETF RFC 3748: "Extensible Authentication Protocol (EAP)".

[28]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[29]	SECG SEC 1: Recommended Elliptic Curve Cryptography, Version 2.0, 2009. Available http://www.secg.org/sec1-v2.pdf

[30]	SECG SEC 2: Recommended Elliptic Curve Domain Parameters, Version 2.0, 2010. Available at http://www.secg.org/sec2-v2.pdf

[31]	3GPP TS 38.470: "NG-RAN; F1 General aspects and principles".

[32]	3GPP TS 38.472: "NG-RAN; F1 signalling transport".

[33] 	3GPP TS 38.474: "NG-RAN; F1 data transport".

[34]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)"

[35]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[36] 	3GPP TS 35.217: "Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2; Document 3: Implementors' test data".

[37] 	3GPP TS 35.223: "Specification of the 3GPP Confidentiality and Integrity Algorithms EEA3 & EIA3; Document 3: Implementors' test data".

[38]	IETF RFC 5216: "The EAP-TLS Authentication Protocol".

[39]	IETF RFC 4346: "The Transport Layer Security (TLS) Protocol Version 1.1".

[40]	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2".

[41]	3GPP TS 38.460: "NG-RAN; E1 general aspects and principles".

[42]	Void.

[43]	IETF RFC 6749: "OAuth2.0 Authorization Framework".

[44]	IETF RFC 7519: "JSON Web Token (JWT)".

[45]	IETF RFC 7515: "JSON Web Signature (JWS)".

[46]	IETF RFC 7748: "Elliptic Curves for Security".

[47]	IETF RFC 7540: " Hypertext Transfer Protocol Version 2 (HTTP/2)".

[48]	IETF RFC 5280: "Internet X.509 Public Key Infrastructure Certificate and Certificate Revocation List (CRL) Profile".

[49]	IETF RFC 6960: "X.509 Internet Public Key Infrastructure Online Certificate Status Protocol - OCSP".

[50]	IETF RFC 6066: "Transport Layer Security (TLS) Extensions: Extension Definitions".

[51]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[52]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[53]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[54]	3GPP TS28.533: " Management and orchestration; Architecture framework".

[55]	3GPP TS28.531: "Management and orchestration of networks and network slicing; Provisioning".

[56]	IETF RFC 4279 "Pre-Shared Key Ciphersuites for Transport Layer Security (TLS)".

[57]	IETF RFC 7542: "The Network Access Identifier".

[58]	IETF RFC 6083: " Datagram Transport Layer Security (DTLS) for Stream Control Transmission Protocol (SCTP)".

[59]	IETF RFC 7516: "JSON Web Encryption (JWE)".

[60]	IETF RFC 8446: "The Transport Layer Security (TLS) Protocol Version 1.3".

[61]	IETF RFC 5705,"Keying Material Exporters for Transport Layer Security (TLS)".

[62]	IETF RFC 5869 "HMAC-based Extract-and-Expand Key Derivation Function (HKDF)".

[63]	NIST Special Publication 800-38D: "Recommendation for Block Cipher Modes of Operation: Galois Counter Mode (GCM) and GMAC".

[64]	IETF RFC 6902: "JavaScript Object Notation (JSON) Patch".

[65]	3GPP TS 31.115: "Secured packet structure for (Universal) Subscriber Identity Module (U)SIM Toolkit applications.

[66]	3GPP TS 31.111: "Universal Subscriber Identity Module (USIM), Application Toolkit (USAT)".

[67]	Internet draft draft-ietf-emu-rfc5448bis: "Improved Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA')".

[68]	3GPP TS 29.510: "5G System; Network function repository services".

[69]	3GPP TS 36.331: "Radio Resource Control (RRC); Protocol specification".

[70]	3GPP TS 29.505: "5G System; Usage of the Unified Data Repository services for Subscription Data; Stage 3".

[71]	3GPP TS 24.302: "Access to the 3GPP Evolved Packet Core (EPC) via non-3GPP access networks; Stage 3".

[72]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC)".

[73]	3GPP TS 29.500: "Technical Realization of Service Based Architecture".

[74]	3GP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[75]	IEEE TSN network aspects: see 3GPP TS 23.501 [2] references [95], [96], [97], [98], [104], and [107].

[76]	Internet draft draft-ietf-emu-eap-tls13: "Using EAP-TLS with TLS 1.3"

[77]	IETF RFC 8446: "The Transport Layer Security (TLS) Protocol Version 1.3".

[78]	3GPP TS 38.401: "NG-RAN; Architecture description".

[79]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)"

[80]	IEEE Std 802.11-2016 (Revision of IEEE Std 802.11-2012) - IEEE Standard for Information technology—Telecommunications and information exchange between systems Local and metropolitan area networks—Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications.

[81]	IETF RFC 2410 "The NULL Encryption Algorithm and Its Use With IPsec".

[x]                        3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

**** END OF 3rd CHANGES ****