3GPP TSG-SA5 Meeting #139e	S5-215383

# 11 - 20 October 2021, E-meeting

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0330 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add measurements related to SM policy authorization for PCF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ePM_KPI_5G |  |  |  |  |  | Date: |  |  | 2021-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | To ensure the 5GS has proper AM and SM polices supporting an external application, the AF may need to query, create, or change the AM and SM policies for a UE in the 5GS. The PCF may authorize the AF to, directly or indirectly via NEF, query, create or change the AM and SM policies. The performance of SM policy management authorization to AF may directly impact the user’s experience when using the subject applications, therefore the policy authorization needs to be monitored. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the measurements related to SM policy authorization for PCF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance of SM policy authorization for PCF cannot be monitored. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st modified section

### 5.5.x	SM policy authorization related measurements

#### 5.5.x.1	Creation of SM policy authorization

##### 5.5.x.1.1	Number of SM policy authorization creation requests

a)	This measurement provides the number of SM policy authorization creation requests received by the PCF.

b)	CC

c)	Receipt of an Npcf_PolicyAuthorization_Create request by the PCF from an NF consumer (e.g., AF) (see 3GPP TS 23.502 [7]).

d)	An integer value

e)	PAU.SmCreateReq

f)	PCFFunction

g)	Valid for packet switched traffic

h)	5GS

##### 5.5.x.1.2	Number of successful SM policy authorization creations

a)	This measurement provides the number of successful SM policy authorization creations at the PCF.

b)	CC

c)	Transmission of an Npcf_PolicyAuthorization_Create response by the PCF to an NF consumer indicating a successful SM policy authorization creation (see TS 29.507 [39]).

d)	An integer value

e)	PAU.SmCreateSucc

f)	PCFFunction

g)	Valid for packet switched traffic

h)	5GS

##### 5.5.x.1.3	Number of failed SM policy authorization creations

a)	This measurement provides the number of failed SM policy authorization creations at the PCF.

b)	CC

c)	Transmission of an Npcf_PolicyAuthorization_Create response by the PCF to an NF consumer indicating a failed SM policy authorization creation (see TS 29.507 [39]), each message increments the relevant subcounter per failure cause by 1.

d)	An integer value

e)	PAU.SmCreateFail.cause
Where cause indicates the failure cause of the SM policy authorization creation.

f)	PCFFunction

g)	Valid for packet switched traffic

h)	5GS

#### 5.5.x.2	Update of SM policy authorization

##### 5.5.x.2.1	Number of SM policy authorization update requests

a)	This measurement provides the number of SM policy authorization update requests received by the PCF.

b)	CC

c)	Receipt of an Npcf_PolicyAuthorization_Update request by the PCF from an NF consumer (e.g., AF) (see 3GPP TS 23.502 [7]).

d)	An integer value

e)	PAU.SmUpdateReq

f)	PCFFunction

g)	Valid for packet switched traffic

h)	5GS

##### 5.5.x.2.2	Number of successful SM policy authorization updates

a)	This measurement provides the number of successful SM policy authorization updates at the PCF.

b)	CC

c)	Transmission of an Npcf_PolicyAuthorization_Update response by the PCF to an NF consumer indicating a successful SM policy authorization update (see TS 29.507 [39]).

d)	An integer value

e)	PAU.SmUpdateSucc

f)	PCFFunction

g)	Valid for packet switched traffic

h)	5GS

##### 5.5.x.2.3	Number of failed SM policy authorization updates

a)	This measurement provides the number of failed SM policy authorization updates at the PCF.

b)	CC

c)	Transmission of an Npcf_PolicyAuthorization_Update response by the PCF to an NF consumer indicating a failed SM policy authorization update (see TS 29.507 [39]), each message increments the relevant subcounter per failure cause by 1.

d)	An integer value

e)	PAU.SmUpdateFail.cause
Where cause indicates the failure cause of the SM policy authorization update.

f)	PCFFunction

g)	Valid for packet switched traffic

h)	5GS

#### 5.5.x.3	Deletion of SM policy authorization

##### 5.5.x.3.1	Number of SM policy authorization deletion requests

a)	This measurement provides the number of SM policy authorization deletion requests received by the PCF.

b)	CC

c)	Receipt of an Npcf_PolicyAuthorization_Delete request by the PCF from an NF consumer (e.g., AF) (see 3GPP TS 23.502 [7]).

d)	An integer value

e)	PAU.SmDeleteReq

f)	PCFFunction

g)	Valid for packet switched traffic

h)	5GS

##### 5.5.x.3.2	Number of successful SM policy authorization deletions

a)	This measurement provides the number of successful SM policy authorization deletions at the PCF.

b)	CC

c)	Transmission of an Npcf_PolicyAuthorization_Delete response by the PCF to an NF consumer indicating a successful SM policy authorization deletion (see TS 29.507 [39]).

d)	An integer value

e)	PAU.SmDeleteSucc

f)	PCFFunction

g)	Valid for packet switched traffic

h)	5GS

##### 5.5.x.3.3	Number of failed SM policy authorization deletions

a)	This measurement provides the number of failed SM policy authorization deletions at the PCF.

b)	CC

c)	Transmission of an Npcf_PolicyAuthorization_Delete response by the PCF to an NF consumer indicating a failed SM policy authorization deletion (see TS 29.507 [39]), each message increments the relevant subcounter per failure cause by 1.

d)	An integer value

e)	PAU.SmDeleteFail.cause
Where cause indicates the failure cause of the SM policy authorization deletion.

f)	PCFFunction

g)	Valid for packet switched traffic

h)	5GS

End of modified sections