3GPP TSG-SA5 Meeting #139e	S5-215384

# 11 - 20 October 2021, E-meeting

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0331 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add measurements related to event exposure for PCF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ePM_KPI_5G |  |  |  |  |  | Date: |  |  | 2021-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | An NF (e.g., NEF or NWDAF) may need to subscribe and get notified about PCF events for a group of UE(s) or any UE accessing a combination of (DNN, S-NSSAI). The events can be subscribed by a NF consumer from PCF are described in clause 6.1.3.18 of TS 23.503 [46]. The NF consumer may use the events exposed by PCF for controlling the UE, hence the performance of event exposure needs to be monitored. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the measurements related to event exposure for PCF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance of event exposure for PCF cannot be monitored. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 5.5.x (new), A.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st modified section

## 3.3	Measurement family

The measurement names defined in the present document are all beginning with a prefix containing the measurement family name. This family name identifies all measurements which relate to a given functionality and it may be used for measurement administration.

The list of families currently used in the present document is as follows:

-	DRB (measurements related to Data Radio Bearer).

-	RRC (measurements related to Radio Resource Control).

-	UECNTX (measurements related to UE Context).

-	RRU (measurements related to Radio Resource Utilization).

-	RM (measurements related to Registration Management).

-	SM (measurements related to Session Management).

-	GTP (measurements related to GTP Management).

-	IP (measurements related to IP Management).

-	PA (measurements related to Policy Association).

-	MM (measurements related to Mobility Management).

-	VR (measurements related to Virtualized Resource).

-	CARR (measurements related to Carrier).

-	QF (measurements related to QoS Flow).

-	AT (measurements related to Application Triggering).

-	SMS (measurements related to Short Message Service).

-	PEE (measurements related to Power, Energy and Environment).

-	NFS (measurements related to NF service).

-	PFD (measurements related to Packet Flow Description).

-	RACH (measurements related to Random Access Channel).

-	MR (measurements related to Measurement Report).

-	L1M (measurements related to Layer 1 Measurement).

-	NSS (measurements related to Network Slice Selection).

-	PAG (measurements related to Paging).

-	NIDD (measurements related to Non-IP Data Delivery).

-	EPP (measurements related to external parameter provisioning).

-	TI (measurements related to traffic influence).

-	CE (measurements related to Connection Establishment).

-	SPP (measurements related to Service Parameter Provisioning).

-	BDTP (measurements related to Background Data Transfer Policy).

-	DM (measurements related to Data Management).

-	BDTP (measurements related to Background Data Transfer Policy).

-	AFQ (measurements related to AF session with QoS).

-	UCM (measurements related to UE radio Capability Management).

-	EEX (measurements related to Event Exposure).

Next modified section

### 5.5.x	Event exposure related measurements

#### 5.5.x.1	Event exposure subscribe

##### 5.5.x.1.1	Number of event exposure subscribe requests

a)	This measurement provides the number of event exposure subscribe requests received by the PCF.

b)	CC

c)	Receipt of an Npcf_EventExposure_Subscribe request by the PCF from an NF consumer (e.g., NEF) (see 3GPP TS 23.502 [7]).

d)	An integer value

e)	EEX.SubscribeReq

f)	PCFFunction

g)	Valid for packet switched traffic

h)	5GS

##### 5.5.x.1.2	Number of successful event exposure subscribe

a)	This measurement provides the number of successful event exposure subscribe at the PCF.

b)	CC

c)	Transmission of an Npcf_EventExposure_Subscribe response by the PCF to an NF consumer (e.g., NEF) indicating a successful event exposure subscribe (see TS 29.507 [39]).

d)	An integer value

e)	EEX.SubscribeSucc

f)	PCFFunction

g)	Valid for packet switched traffic

h)	5GS

##### 5.5.x.1.3	Number of failed event exposure subscribe

a)	This measurement provides the number of failed event exposure subscribe at the PCF.

b)	CC

c)	Transmission of an Npcf_EventExposure_Subscribe response by the PCF to an NF consumer (e.g., NEF) indicating a failed event exposure subscribe (see TS 29.507 [39]), each message increments the relevant subcounter per failure cause by 1.

d)	An integer value

e)	EEX.SubscribeFail.cause
Where cause indicates the failure cause of the event exposure subscribe.

f)	PCFFunction

g)	Valid for packet switched traffic

h)	5GS

#### 5.5.x.2	Event exposure unsubscription

##### 5.5.x.2.1	Number of event exposure unsubscribe requests

a)	This measurement provides the number of event exposure unsubscribe requests received by the PCF.

b)	CC

c)	Receipt of an Npcf_EventExposure_Unsubscribe request by the PCF from an NF consumer (e.g., NEF) (see 3GPP TS 23.502 [7]).

d)	An integer value

e)	EEX.UnsubscribeReq

f)	PCFFunction

g)	Valid for packet switched traffic

h)	5GS

##### 5.5.x.2.2	Number of successful event exposure unsubscribe

a)	This measurement provides the number of successful event exposure unsubscribe at the PCF.

b)	CC

c)	Transmission of an Npcf_EventExposure_Unsubscribe response by the PCF to an NF consumer (e.g., NEF) indicating a successful event exposure unsubscribe (see TS 29.507 [39]).

d)	An integer value

e)	EEX.UnsubscribeSucc

f)	PCFFunction

g)	Valid for packet switched traffic

h)	5GS

##### 5.5.x.2.3	Number of failed event exposure unsubscribe

a)	This measurement provides the number of failed event exposure unsubscribe at the PCF.

b)	CC

c)	Transmission of an Npcf_EventExposure_Unsubscribe response by the PCF to an NF consumer (e.g., NEF) indicating a failed event exposure unsubscribe (see TS 29.507 [39]), each message increments the relevant subcounter per failure cause by 1.

d)	An integer value

e)	EEX.UnsubscribeFail.cause
Where cause indicates the failure cause of the event exposure unsubscribe.

f)	PCFFunction

g)	Valid for packet switched traffic

h)	5GS

#### 5.5.x.3	Event exposure notification

##### 5.5.x.3.1	Number of event exposure notifications

a)	This measurement provides the number of event exposure notifications sent by the PCF.

b)	CC

c)	Transmission of an Npcf_EventExposure_Notify message by the PCF to an NF consumer (e.g., NEF) (see TS 29.502 [7]).

d)	An integer value

e)	EEX.NotifyNbr

f)	PCFFunction

g)	Valid for packet switched traffic

h)	5GS

Next modified section

# A.x	Monitoring of event exposure

An NF (e.g., NEF or NWDAF) may need to subscribe and get notified about PCF events for a group of UE(s) or any UE accessing a combination of (DNN, S-NSSAI).

The events can be subscribed by a NF consumer from PCF are described in clause 6.1.3.18 of TS 23.503 [46].

The NF consumer may use the events exposed by PCF for controlling the UE, hence the performance of event exposure needs to be monitored.

End of modified sections