3GPP TSG-SA WG6 Meeting #48-e	S6-220760

e-meeting, 5th – 14th April 2022	(revision of S6-220612)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0049 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications related to multi carrier support for MBS session update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2022-04-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In alignment with 3GPP TS 23.247, in case multi carrier deplyment is supported, and based on the business agreement, the MC service server is capable of updating the MBS Frequency Selection Area ID (MBS FSA ID) once the need has emerged. The MBS FSA ID update is applicable to broadcast MBS sessions, and is done by initiating an MBS session update procedure towards the 5GC (either directly towards MB-SMF or indirectly via NEF). However, the MBS FSA ID can be included within the service area related parameters, and the text under clause 7.3.3.2.1 is updated to be generalized and to reflect the different parameters which be updated by the MC service server. Reference to 3GPP TS 23.247 is added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Among the aspects that can be updated during an MBS session update procedure (with or without dynamic PCC rule), the MBS FSA ID is included as service area related parameters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MC service server is unable to update the MBS FSA ID. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 7.3.3.2.1	General

The MC service server can create one or several MBS sessions based on certain service requirements (e.g., QoS profile), a certain service area, or the activity status of multicast MBS sessions. However, during the life cycle of the MBS sessions, the MC service server may need to update the sessions to meet emerging needs, including  the service requirements, service area related parameters, etc…, as defined in 3GPP TS 23.247 [17]. .

In case of dynamic PCC rule, the MC service server needs to determine what aspects are needed to be updated (either service area related aspects/ multicast activation status, service requirements aspects , or both as described in 3GPP TS 23.247 [17]) in order to interact with the required entity in the 5GC (either directly in case the MC service server is in trusted domain) or indirectly via NEF/MBSF.

# * * * * End of changes * * * *