3GPP TSG-SA WG6 Meeting #42-e	S6-210531

e-meeting, 1st – 9th March 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.286 | CR | 0043 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to VAE server APIs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XAPP |  |  |  |  |  | Date: |  |  | 2021-02-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The information elements for the procedures specified in clause 9.16.3 and clause 9.16.6 are to be specified. Also the API definition for these procedures is required to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the information flows and the API definition |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 3 may not be able to progress. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.16.2, 9.16.2.1 (new), 9.16.2.2 (new), 9.16.2.3 (new), 10.2.1, 10.2.x (new), 10.2.x.1 (new), 10.2.x.2 (new), 10.2.x.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 9.16.2	Information flows

#### 9.16.2.1	Subscribe HD map dynamic information request

Table 9.16.2.1-1 describes the information flow for a V2X application specific server to subscribe to HD map dynamic information at the VAE server.

Table 9.16.2.1-1: Subscribe HD map dynamic information request

| Information element | Status | Description |
| --- | --- | --- |
| V2X UE ID | M | Identifier of the V2X UE |
| Application defined proximity range information |  | M |  | Description of the range information over which the HD map dynamic information is required. |  |

#### 9.16.2.2	Subscribe HD map dynamic information response

Table 9.16.2.2-1 describes the information flow for VAE server to respond for HD map dynamic subscription request from the V2X application specific server.

Table 9.16.2.2-1: Subscribe HD map dynamic information response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result from the VAE server in response to subscription request indicating success or failure |
| Subscription ID (NOTE) | O | Identifier of a successful subscription. |
| NOTE: This IE is included when the Result indicates success |  |  |

#### 9.16.2.3	Notify HD map dynamic information

Table 9.16.2.3-1 describes the information flow for a V2X application specific server to receive notification about HD map dynamic information from the VAE server.

Table 9.16.2.3-1: Notify HD map dynamic information

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID |  | M |  | Identifier of the subscription for this notification. |  |
| List of V2X UEs information | M | The information of the V2X UEs which were detected in the application defined proximity range. |
| >Nearby V2X UE ID | M | The identifier of nearby V2X UE |
| >Location information | M | Location information of the nearby V2X UE within the application defined proximity range |
| >Distance information | M | Distance information of the nearby V2X UE |

* * * Next Change * * * *

### 10.2.1	General

Table 10.2.1-1 illustrates the VAE server APIs.

Table 10.2.1-1: List of VAE server APIs

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| VAE_MessageDelivery API | Deliver_DL_Message | V2X application specific server | Request/ Response |
|  | Deliver_UL_Message | V2X application specific server | Subscribe/notify |
| VAE_FileDistribution API | Distribute_File | V2X application specific server | Request/ Response |
| VAE_ApplicationRequirement API | Reserve_NetworkResource | V2X application specific server | Subscribe/notify |
|  | Notify_NetworkResource |  |  |
| VAE_DynamicGroup API | Configure_DynamicGroup | V2X application specific server | Request/Response |
|  | Notify_DynamicGroup |  |  |
| VAE_ServiceContinuity API | Query_ServiceContinuity | VAE server | Request/Response |
| VAE_HDMapDynamicInfo API | Subscribe_HDMapDynamicInfo | V2X application specific server | Subscribe/notify |
|  | Notify_HDMapDynamicInfo |  |  |

The SEAL server(s) APIs available via SEAL-S reference point as specified in 3GPP TS 23.434 [6] are consumed by the V2X application specific server via the VAE server (acting as a VAL server) over the Vs reference point. When V2X application specific server invokes a SEAL server API via the VAE server over the Vs reference point, the VAE server shall interact with the corresponding SEAL server over the SEAL-S reference point for the API invocation request and response.

NOTE:	When V2X application specific server invokes a SEAL server API directly, the V2X application specific server acting as VAL server shall interact with the corresponding SEAL server over the SEAL-S reference point for the API invocation request and response as specified in 3GPP TS 23.434 [6].

Editor's note:	Whether the VAE_DynamicGroup API is to be moved to SEAL is FFS.

* * * Next Change * * * *

### 10.2.x	VAE_HDMapDynamicInfo API

#### 10.2.x.1	General

API description: This API enables the V2X application specific server to communicate with the VAE server to subscribe and receive notifications for HD map dynamic information.

#### 10.2.x.2	Subscribe_HDMapDynamicInfo operation

API operation name: Subscribe_HDMapDynamicInfo

Description: Subscribes for the HD map dynamic information at the VAE server.

Known Consumers: V2X application specific server.

Inputs: Refer subclause 9.16.2.1.

Outputs: Refer subclause 9.16.2.2.

See subclause 9.16.3 for the details of usage of this API operation.

#### 10.2.x.3	Notify_HDMapDynamicInfo operation

API operation name: Notify_HDMapDynamicInfo

Description: Notify the HD map dynamic information at the VAE server.

Known Consumers: V2X application specific server.

Inputs: Refer subclause 9.16.2.3.

Outputs: None.

See subclause 9.16.6 for the details of usage of this API operation.