TSG-SA Working Group 1 (Services) meeting #4           TSGS1#4(99)425

Quebec; Canada 7th-9th July 1999                            Agenda Item: 6.3.1.3

Agenda Item:	22.105 (3.5.0)

Source: 	T-Mobil

Title:	Proposed changes to Chapter 5 “Application Groups”

Document for:	Discussion and decision

___________________________________________________________________________________

- Introduction

The main message in this chapter is that UMTS has to support 4 application groups:

- Conversational,

- Interactive,

- Streaming,

- Background.

Therefore the description of the 4 application groups should be moved to the beginning of Chapter 5. From this starting point all further QoS requirements for UMTS are identified in terms of bearer service requirements, information transfer attributes, information quality attributes. This has to be reflected in the subsequent structure of this chapter.

- Proposal

Current chapter 5. 4 Range of QoS requirements.

The table is moved to the Annex as the figures in it do not give too much differentiated information. The important figures are already mentioned in Chapter 5.3. The same is applied for the three tables in chapter 5.5.

Instead a short new table which summarises the values from the old tables is added. The main purpose of this table is to provide the necessary values (from End user point of view) that are required by the system designers for choosing bearers and error coding interleaving etc.

- The purpose of this table is to provide all the necessary values (from End user

point of view) that are required by the system designers for choosing bearers

and error coding interleaving etc.

| Application Class | Data rate | BER | Delay | Delay Variation. |
| --- | --- | --- | --- | --- |
| Conversational | 4- 768 kb/s | 10-3 - 10-5 | 200- 300 ms | No |
| Interactive | 32- 384 Kb/s | 10-3 - 10-7 | 1 s | High |
| Streaming | 32 KB/s - 2Mb/s | 10-3 - 10-7 | < 10 s | Low |
| Background | Best effort |  |  |  |

Current chapter 5.2.1 we propose to delete the the clause on real time and non - real time applications under traffic type. The requirements of real time applications are already stated with the traffic type “guaranteed/contstant bit.