TSG-SA Working Group 1 CAMEL rel.00 kick off meeting	TSG S1 (00) 250

Sophia Antipolis, France

5th – 6th of April 2000

Source: 		Siemens

Agenda Item:

Work Item Description

# Title: Scope of CAMEL 4 in Release 2000

1		3GPP Work Area

|  | Radio Access |
| --- | --- |
| X | Core Network |
| X | Services |

2		Linked work items

none known

3		Justification

It was agreed to use CAMEL as a proper toolkit to provide services. For Release 2000 additions are needed. This Work Item Description summarise the work to be done.

4		Objective

- CAMEL 4 shall be a superset of CAMEL 3 together with R00 CAMEL additions.

- Any R00 CAMEL addition, shall be valuable  and applicable for GSM and UMTS networks, except otherwise stated.

- The same set of CAMEL 4 capabilities shall be available when roaming in- or outside the home network.

- CAMEL 4 capabilities shall apply to any service where appropriate, regardless the transport plan. That means, CAMEL procedures shall  be useable for CS-, PS- and IM- based services.

- CAMEL procedures shall be enhanced for the manipulation of media streams where appropriate

- New CAMEL procedures shall be identified for the handling of user messages, i.e. administration, distribution, forwarding etc. of voice messages.

- New CAMEL procedures shall be identified to handle the user profile, i.e. access to the user profile from other equipment than the users terminal. To guarantee the users confidentially, a security policy has to addressed.

- Concepts to execute simple services at the visited network shall be investigated, i.e. simple screening, translation or forwarding services might be done at the visited network when sufficient logic is provided by the home network.

- Capabilities left over from CAMEL 3 shall be examined case by case and taken to CAMEL 4 if needed. As an initial idea, Siemens propose to cope with the triggering for USSD, to support the MidCall trigger for active Follow On calls and to get a flexible approach to provide User Interactions.

- Service Aspects

In contrast to 2G systems new services in 3GPP will not be standardised. New services will be enabled by Service Capabilities, which are based on toolkits (e.g. CAMEL). If in addition to existing Service Capabilities new SCs particular to IM services are required or existing SCs need to be modified this needs to be identified.

6		MMI-Aspects

none identified.

7		Charging Aspects

Sufficient Charging capabilities has to be foreseen to allow flexible charging scenarios from the service provider and the network operator (visited and home) point of view. From the subscribers point of view, online charging methods

8		Security Aspects

A special attention has to be given to a Security Policy. Secure access to service and user confidential information has to be provided.

9	Impacts

| Affects: | USIM | ME | AN | CN | Others |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No |  |  | X |  |  |
| Don't know | X | X |  |  | X |

- Expected Output and Time scale (to be updated at each plenary)

The results of this Workitem shall be provided in a Technical Standard.

In order to clearly state the TSG-S1 Service Requirements to other TSG’s and WG’s in a timely fashion the following Work Plan is proposed.

| S1 | Dates | Actions |
| --- | --- | --- |
| S1 R2000 adhoc | April 5-6, 2000 | - Start the work - Produce the Work Item Description - Draft the scope for CAMEL R00 |
| S1#8 | April 10-14, 2000 | - Scope to be submitted to and approve at S1#8 |
| S1 CAMEL adhoc | May 16-18, 2000 | - Work on of any new Stage 1’s required so they are suitable for v1.0.0 at SA#9. - Produce initial CR’s to the existing 22-series |
| S1#9 | July 17-21, 2000 | - Prepare any new Stage 1’s for submission to SA#10. - Complete CR’s to the 22-series. |
| S1 CAMEL adhoc | August 16-17, 2000 | - Progress and finalise work on the TS |
| S1#10 | November 13-17, 2000 | - Revise Stage 1’s in line with feedback from other TSG’s and WG’s. - Begin TR on R2001. |

| New specifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Spec No. | Title |  | Prime rsp. WG | 2ndary rsp. WG(s) | Presented for information at plenary# |  | Approved at plenary# |  | Comments |
| 22.078 | CAMEL 4 |  | S1 |  |  |  |  |  |  |
| Ed comment: there will be potentially other new specs, yet to be identified |  |  |  |  |  |  |  |  |  |
| Affected existing specifications |  |  |  |  |  |  |  |  |  |
| Spec No. | CR | Subject |  |  |  | Approved at plenary# |  | Comments |  |
| Ed comment: there will be probably various R99 specifications impacted, yet to be identified |  |  |  |  |  |  |  |  |  |

11		Work item raporteurs

[ ]

12		Work item leadership

TSG S1

13		Supporting Companies

[Siemens, Alcatel ]

14		Classification of the WI (if known)

|  | Feature (go to 14a) |
| --- | --- |
|  | Building Block (go to 14b) |
|  | Work Task (go to 14c) |

- Service Control in the home network

complete control of the service will lay in the home network

standardised functionality (protocols) will be used to execute the services in the home network

mandatory functions (SSF) has to be implemented in 
service network

- Remote Service Control from the home network

similar to the above scenario but remote control of the
serving network from the home network (master and
slave Call Control agents)