3GPP TSG-SA WG1 Meeting #102	S1-231228

Berlin, Germany, 22-26 May 2023	(revision of S1-23xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.104 | CR | 0095 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification about LPHAP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi, Inspur |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LPHAP |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA2 sends a LS(S2-2303414) to SA1 to ask for the requirement about LPHAP: SA2 kindly ask SA1: whether SA1 specifies the requirement on “low power or high accuracy” positioning in release 18. In TS22.104: Table 5.7.1-1: Positioning performance requirements does not include Service Level 1 (Horizontal accuracy is 10m), since Service Level 1 is not regarded as high accuracy positioning. Table A.7.2-1: Low power high accuracy positioning use cases has 9 use cases. The Horizontal accuracy of Use Case 1 and 9 are 10m (Service Level 1), which is not for high accuracy positioning, but can be regarded as Low power positioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that LPHAP includes ‘Low Power and High Accuracy Positioning’ and ‘Low Power Positioning’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec is not clear and may confuse the downstream groups. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============================  Start of Changes =======================================

## A.7.2	Low Power High Accuracy Positioning

Low power high accuracy positioning is an integral part of a considerable number of industrial applications. The total energy needed for a specific operation time for such a low power high accuracy positioning optimized IoT-device is a combination of energy for positioning (varies depending on the used positioning method), energy for communication/‌synchronization and a difficult to predict factor to take additional losses through e.g. security, power management, microcontroller, and self-discharge of batteries into account.

Examples of target applications for low power high accuracy positioning are asset tracking in process automation, tracking of vehicles, and tool tracking.

Table A.7.2-1 gives an indication of the required operation time of the 5G enabled IoT device and duty cycle of the updated position information for different use cases.

Table A.7.2-1: Low power high accuracy positioning use cases

| Use Case # | Horizontal accuracy | Corresponding service level (22.261) | Positioning interval/ duty cycle | battery life time/ minimum operation time |
| --- | --- | --- | --- | --- |
| 1 | 10 m | Service Level 1 | on request | 24 months |
| 2 | 2 m to 3 m | Service Level 2 | < 4 seconds | > 6 months |
| 3 | < 1 m | Service Level 3 | no indication | 1 work shift - 8 hours (up to 3 days, 1 month for inventory purposes) |
| 4 | < 1 m | Service Level 3 | 1 second | 6 - 8 years |
| 5 | < 1 m | Service Level 3 | 5 seconds - 15 minutes | 18 months |
| 6 | < 1 m | Service Level 3 | 15 s to 30 s | 6 - 12 months |
| 7 | 30 cm | Service Level 5 | 250 ms | 18 months |
| 8 | 30 cm | Service Level 5 | 1 second | 6 - 8 years (no strong limitation in battery size) |
| 9 | 10 m | Service Level 1 | 20 minutes | 12 years (@20mJ/position fix) |

NOTE: Use Case 1 and 9 are for Low power positioning, while other use cases are for Low power and high accuracy positioning.

Use case one

Process automation: Dolly tracking (outdoor).

Use case two

Process automation: Asset tracking.

Use case three

Flexible modulare assembly area: Tool tracking in flexible, modular assembly areas in smart factories.

Use case four

Process automation: Sequence container (Intralogistics).

Use case five

Process automation: Palette tracking (e.g. in turbine construction).

Use case six

Flexible modulare assembly area: Tracking of workpiece (in- and outdoor) in assembly area and warehouse.

Use case seven

Flexible modulare assembly area: Tool assignment (assign tool to vehicles in a production line, left/right) in flexible, modular assembly area in smart factories.

Use case eight

Flexible modulare assembly area: Positioning of autonomous vehicles for monitoring purposes (vehicles in line, distance 1.5 meter).

Use case nine

(Intra-)logistics: Asset tracking

==============================  End of Changes =======================================