3GPP TSG SA WG 1 Meeting #102 	S1-231485

Berlin, Germany,  22 - 26 May 2023	(revision of S1-231406)

Source:	Qualcomm, Futurewei, Wiliot, KPN

pCR Title:	Update to Use Case on Finding Lost Item

Draft Spec:	3GPP  TR 22.840 v1.1.0

Agenda item:	7.2

Document for:	Approval

Contact:	Lola Awoniyi-Oteri <oawoniyi@qti.qualcomm.com>

Abstract: Updating use case in clause 5.8 to include multiple MNOs

1. Introduction

Updating use case to introduce multiple MNO support and update KPIs.

2. Reason for Change

- Updating use case pre-conditions and service flows to introduce multiple MNO scenario

- Adding two PRs ([PR 5.8.6-001] and [PR 5.8.6-007]) for multiple MNO communication and security support

- Adding values for postioning service availabillity and position accuracy KPIs

- Removed the comparators on the KPI values

- Revised PR 5.8.6-001 and removed Note 4

- Added NOTE 2 to express the fact that the 3rd party entity/application can be provided by the operator.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.840 v1.1.0

* * * First Change * * * *

### 5.8.2	Pre-conditions

Alice subscribed to a “lost tag finding” service with the EasyTracking Inc, a company that provides this service using Ambient IoT tags.

She attaches the tag to her baggage and associates the tag with her mobile phone in this service.

EasyTracking Inc, has agreements with multiple operators (e.g., MNO A in New York (JFK) and MNO B in New Jersey(Newark Airport) to provide lost tag finding service throughout airports across the United States.

* * * Next Change * * * *

### 5.8.3	Service Flows

The case of remote lost item finding by crowdsourcing

- Alice is traveling from San Diego (SAN) to New York (JFK) using airplane with her tag (tag-A) attached to her baggage. When she arrives in JFK airport, she does not find her baggage in baggage claim area. Unfortunately, her bag was mistakenly taken to Newark airport.

- At the Newark airport, the tag-A attached to Alices’ baggage identifies that it is lost and notify any nearby UEs/RAN entities most of which are belong to  to MNO B that support the lost tag finding service that the tag-A is currently lost.

- As shown in Figure 1, the nearby UEs/RAN entities belonging to MNO B which is notified from the tag-A connects to server providing the lost tag finding service and reports the tag-A’s current location and time. (For privacy, the tag-A’s identity should not be known to other UEs/RAN entities other than Alice’s UE or service provider.)

- Alice, after finding that her baggage is missing in airport, opens an app in her mobile phone to find her tag-A’s whereabout. The app communicates with the server and gets the recent information about , location/time/etc, related to the tag-A (e.g. information via MNO A determining the tag was not in JFK after Alice searches unsuccessfully for her tag and information via MNO B reporting the tag was found at the Newark airport). This information via MNO A and B may not be sent to the application simultaneously.

- Alice can track the location of her baggage with the help of lost item finding service.

* * * Next Change * * * *

### 5.8.6	Potential New Requirements needed to support the use case

[PR 5.8.6-001] The 5G System shall be able to support means for  communication between Ambient IoT device(s) and a trusted 3rd  party entity/application via one or more network(s) that are managed by the same or different MNOs.

NOTE 1:	It is assumed that certain business relationship and agreement between the trusted 3rd party and the network operator(s) has been established.

NOTE 2:  The trusted 3rd party entity/application may be hosted by an operator.

[PR.5.8.6-002] The 5G system shall be able to assist an Ambient IoT device with discovery and communication with 5GS entities that can provide location related information.

[PR 5.8.6-003] Based on operator policy, the 5G system shall be able to support authorization of UEs communicating with an Ambient IoT device.

[PR.5.8.6-004] The 5G system shall be able to support means to support RAN entities and authorized UEs to communicate with Ambient IoT devices and transfer related information to other 5G system entities (e.g., core network) / servers.

[PR.5.8.6-005] The 5G system shall be able to provide a mechanism to protect the privacy of information (e.g., location and identity) exchanged during communication with an Ambient IoT device.

NOTE 3:	This requirement refers to communication between Ambient IoT devices and 5G System entities (e.g., core network, RAN entities), application servers or authorized UEs.

[PR 5.8.6-006] The 5G system shall be able to support a UE to authenticate an Ambient IoT device.

[PR 5.8.6-007] The 5G System shall be able to support mechanisms to authenticate and authorize communication with Ambient IoT devices via a trusted 3rd party service provider.

NOTE 4:	It is assumed that certain business relationship and agreement between the trusted 3rd party and 5G network operator(s) has been established.

[PR 5.8.6-008] The 5G system shall be able to support Ambient IoT devices with the following KPIs:

Table 5.8.6-1:  KPI Requirements for “Finding Remote Lost Item” Service

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Remote lost item finding (Indoor) | 5s | 99% (Note 1 ) | NA | NA | 256 bits (Note 2 ) | 750 devices/100m2 (Note 3) | 10m | NA | NA | NA | NA | 90% | ~3m |
| Remote lost item finding (Outdoor) | 5s | 99% (Note 1) | NA | NA | 256 bits (Note 2) | 750 devices/100m2 (Note 3) | 100m | NA | NA | NA | NA | 90% | ~10m |
| Note 1: Service can be potentially provided by both multiple UEs and RAN entities. Note 2: 64bits corresponds to 20 digits in decimal number. 20 digits is assumed for the length of tag ID. Additional 192bits were assumed for control and other data (e.g., location information, IP address of server). Note 3: 100m2 space can be packed with about 250 large size baggage (with H=0.7m, W=0.5m). Assuming 3 layers of baggage stacked gives = 250x3 = 750 |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Change * * * *