3GPP TSG-SA WG1 Meeting #102 	S1-231593

Berlin, Germany, 22 - 26 May 2023	(revision of S1-231109)

Title:	pCR to TR 22.865: Requirement Consolidation for KPIs

Agenda Item:	7.3

Document for:	Approval

Source:	Huawei, Samsung, Nokia, Nokia Shanghai Bell, Tencent

Contact:	Alice Li (alice.li1@huawei.com), Erik Guttman (erik.guttman@samsung.com)

Abstract: This pCR proposes an initial consolidation of KPI requirements for the FS_Metaverse study.

Revision to resolve the comments received online, including

- removing the column “Message Data Volume”, and converting them into notes

- removing the row of “Mobile Metaverse for immersive gaming and live shows” with the understanding that this will be addressed in a separate pCR

- removing the comments on the side

Start of changes

# 8	Consolidated potential requirements and KPIs

…

## 8.2	Consolidated potential KPIs

Editor’s Note: update is needed when the following KPIs are finalised in SA1#102 meeting:

Table 5.7.6-1

Table 5.7.6-2

Table-5.10.6-1

Editor’s Note: PRs 4, 5 and 6 are FFS

| Use Cases | Characteristic parameter (KPI) |  |  |  | Influence quantity |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max allowed end-to-end latency | Service bit rate: user-experienced data rate | Reliability | Area Traffic capacity | Message size (byte) | Transfer Interval | Positioning accuracy | UE Speed | Service Area |  |
| 5G-enabled Traffic Flow Simulation and Situational Awareness (NOTE 2) | [5-20] ms (NOTE 1) | [10~100 Mbit/s] [25] (NOTE 6) | > 99.9% | [~39.6 Tbit/s/km2 ] (NOTE 5) | - | 20~100 ms (NOTE 3) | - | < 250 km/h | City or Country wide (NOTE 4) | UL |
| Collaborative and concurrent engineering | [≤10] ms [14] (NOTE 7) | [1-100] Mbit/s [14] | [> 99.9%] [14] | [1.55] Tbit/s/km2 (NOTE 8) | Video: 1500 Audio: 100 [14] | - | - | Stationary or Pedestrian | typically < 100 km2 (NOTE 9) | UL and DL audio/video |
|  | [5] ms UL [1-50] ms DL [14] (NOTE 7) | [<1] Mbit/s [14] | [> 99.9%] (without compression) [> 99.999%] (with compression (NOTE 10)) [26] | [2.25] Tbit/s/km2 (NOTE 8) | 1 DoF: 2-8 3 DoFs: 6-24 6 DoFs: 12-48 [14] | 0.25-10 ms [14] |  |  |  | UL and DL haptic feedback |
| Metaverse-based Tele-Operated Driving (NOTE 16) | [100] ms [25] (NOTE 11) | [10~50 Mbit/s] [25] | 99% [25] | [~360 Mbit/s/km2 ] (NOTE 14) | - | 20~100 ms [25] (NOTE 12) | [10] cm [25] | [10-50] km/h (vehicle) [25] Stationary/Pedestrian (user) | Up to 10km radius [25] (NOTE 13) | UL real-time vehicle data (video streaming and/or sensor data) [25] |
|  | [20] ms [25] | [0.1~0.4 Mbit/s] [25] | 99,999% [25] | [~4 Mbit/s/km2 ] (NOTE 14) | Up to 8Kb [25] | 20 ms [25] (NOTE 12) | [10] cm [25] | [10-50] km/h (vehicle) [25] Stationary/Pedestrian (user) | Up to 10km radius [25] (NOTE 13) | DL control traffic (commands from the remote driver) [25]. |
|  | 1-20ms (NOTE 15) | 16 kbit/s -2 Mbit/s (without haptic compression encoding); 0.8 - 200 kbit/s (with haptic compression encoding) (NOTE 15) | 99.999% (NOTE 15) | [~20 Mbit/s/km2 ] (NOTE 14) | 2-8 (1 DoF) (NOTE 15) |  |  | Stationary/Pedestrian (user) | Up to 10km radius [25] (NOTE 13) | Haptic feedback |
| NOTE 1: The mobile metaverse server receives the data from various sensors, performs data processing, rendering and provide feedback to the vehicles and users. The end-to-end latency refers to the transmission delay between a UE and the mobile metaverse server. NOTE 2: Examples of typical data volume including 1) camera: 10 Mbit/s per sensor (unstructured), 2) LiDAR: 90 Mbit/s per sensor (unstructured), 3) radar: 10 Mbit/s per sensor (unstructured), and 4) real-time Status information including Telemetry data: [< 50 kbit/s] per sensor/vehicle/VRU (structured). This is to support at least 80 vehicles and 1600 users present at the same location (e.g. in an area of 40m*250m) to actively enjoy immersive metaverse services for traffic simulation and traffic awareness, the area traffic capacity is calculated considering 2 cameras, 2 Radars, 2 LiDARs on road side, 1600 user’s smart phones and 80 vehicles with 7 cameras, 4 radar and 2 LiDAR for each vehicle. NOTE 3: The frequency considers different sensor types such as Radar/LiDAR (10Hz) and camera (10~50Hz). NOTE 4: The service area for traffic flow simulation and situational awareness depends on the actual deployment, for example, it can be deployed for a city or a district within a city or even countrywide. In some cases a local approach (e.g. the application servers are hosted at the network edge) is preferred in order to satisfy the requirements of low latency and high reliability. NOTE 5: The calculation is this table is done per one 5G network, in case of N 5G networks to be involved for such use case in the same area, this value can be divided by N. NOTE 6: User experienced data rate refers to the data rate needed for the vehicle or human, the value is observed from industrial practice. NOTE 7: The network based conference focus is assumed, which receives data from all the participants, performs rendering (image synthesis), and then distributes the results to all participants. The latency refers to the transmission delay between a UE and the application server. As rendering and hardware introduce some delay, the communication delay for haptic feedback is typically less than 5ms. NOTE 8: To support at least 15 users present at the same location (e.g. in an area of 20m*20m) to actively enjoy immersive Metaverse service concurrently, the area traffic capacity is calculated considering per user consuming non-haptic XR media (e.g. for video per stream up to 40000 kbit/s) and concurrently 60 haptic sensors (per haptic sensor generates data up to 1024 kbit/s). NOTE 9: In practice, the service area depends on the actual deployment. In some cases a local approach (e.g. the application servers are hosted at the network edge) is preferred in order to satisfy the requirements of low latency and high reliability. NOTE 10: The arrival interval of compressed haptic data usually follow some statistical distributions, such as generalized Pareto distribution, and Exponential distribution [26]. NOTE 11: The end-to-end latency refers to the transmission delay between a UE and the mobile metaverse server or vice-versa, not including sensor acquisition or actuator control on the vehicle side, processing, and rendering on the user side (estimated additional 100ms total). Target e2e user experienced max delay depends on reaction time of the remote driver (e.g. at 50km/h, 20ms means 27cm of remote vehicle movement). NOTE 12: UL data transfer interval around 20ms (video) to 100ms (sensor), DL data transfer interval (commands) around 20ms. NOTE 13: The service area for teleoperation depends on the actual deployment; for example, it can be deployed for a warehouse, a factory, a transportation hub (seaport, airport etc.), or even a city district or city. In some cases, a local approach (e.g., the application servers are hosted at the network edge) is preferred to satisfy low latency and high-reliability requirements. NOTE 14: The area traffic capacity is calculated for one 5G network, considering 4 cameras + sensors on each vehicle. Density is estimated to 10 vehicles/km2, each of the vehicles with one user controlling them. [25] NOTE 15: KPI comes from [5] clause 7.11 “remote control robot” use case. NOTE 16: Examples of typical data volume including 1) ~8Mbps video stream. Four cameras per vehicle (one for each side): 4*8=32Mbps. 2) sensor data (interpreted objects), assuming 1 kB/object/100 ms and 50 objects: 4 Mbps [25]. |  |  |  |  |  |  |  |  |  |  |