3GPP TSG SA WG 1 Meeting #104		 S1-233014

Chicago, USA, 13 - 17 November 2023

Title:	Reply LS on requirements for slice-based PLMN selection

Response to:	LS S1-233012 on requirements for slice-based PLMN selection from CT WG1

Release:	Rel-18

Work Item:	PLMNsel NS

Source:	SA1

To:	CT1

Cc:	SA2

Contact person:	Yvette Koza

Yvette (dot) koza (at) zte (dot) com (dot) cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

SA WG1 thank CT WG1 for their LS S1-233012 asking SA1 clarification and guidance with regards to the requirement related to slice-based PLMN selection issue as defined in 3GPP TS 22.261as follows:

For a roaming UE activating a service/application requiring a network slice not offered by the serving network but available in the area from other network(s), the HPLMN shall be able to provide the UE with prioritization information of the VPLMNs with which the UE may register for the network slice.

After discussion SA1 would like to provide the following answers to the questions raised by CT1:

Question 1:

Would SA1 be able to provide any additional guidance regarding Scenario 1? In particular:

- Whether the "prioritization information" defines priorities between the PLMNs or between the slices?

- If the answer to a) is “between the slices”, should the prioritization of slices be determined by the network or by the UE?

Answer 1:

The SA1 requirement above addresses roaming. So, the prioritization information defines priorities between the PLMNs based on home operator policies. This requirement doesn't require any information on slices to be provided by HPLMN.

Question 2:

Should a slice determined according to Scenario 2 be considered as being offered by the serving network or not? If yes, then is the slice determined from the non-default URSP rule to be considered as higher priority than the slice determined from the default URSP rule?

Answer 2:

To the mentioned Scenario 2 describing a service/application mapped to a slice using default URSP rule with the “match-all” traffic descriptor, a slice determined according to this scenario should be considered as being offered by the serving network.

To the question whether a non-default URSP rule is to be considered as higher priority than the slice determined from the default URSP rule, it is specified that each URSP rule contains a Rule Precedence value that determines the priority of the rule within the policy. A URSP rule with the "match all" Traffic descriptor should have the lowest priority.

Question 3:

Would SA1 be able to provide any additional guidance regarding the interaction between legacy PLMN selection prioritization and new prioritization information for slice based PLMN selection? In particular:

- which list has a higher priority while performing PLMN selection?

- does the trigger to perform SOR apply when the UE uses slice-based PLMN selection?

Answer 3:

As answered in Q1 the network only provides the prioritization information of VPLMNs under control of the home operator in roaming case. It may be unnecessary to introduce a new list or enhance the operator-controlled list with slice information since the UE will not be aware of which slices are supported during registration and which one is required. If a prioritization considering Slice information and VPLMNs is required, SA1 would need to clarify and update their requirements.

# 2	Actions

To CT1

ACTION: 	SA1 kindly ask CT1 to take the provided responses above into account.

# 3	Dates of next TSG SA WG 1 meetings

SA1#105	26 Feb -01 Mar 2024		Athens, GR

SA1#106	27-31 May 2024 			Korea, Jeyju