3GPP TSG SA WG 1 Meeting #104 	S1-233101

Chicago, USA, 13 - 17 November 2023	(revision of S1-23xxxx)

Source:	Qualcomm

pCR Title:	Resolving editor`s note in KPI Table

Draft Spec:	3GPP TS 22.137 v.1.0.0

Agenda item:	7.1.2

Document for:	Approval

Contact:	Lola Awoniyi-Oteri , oawoniyi@qti.qualcomm.com

Abstract: This pCR proposes resolving editor`s note in the KPI Table

1. Introduction

The KPIs for  Sensing Service Category 8 KPIs can be supported by 5G system even those can could be deemed stringent like the posistioning accuracy and sensing resolution.

- The vertical and horizontal positioning accuracy KPIs for Sensing Service Category 8  are based on the minimum supported positioning accuracy in 5G systems specified in [TS 22.261]

- The sensing range resolution KPI of 0.375m is estimated assuming bandwidth of 400 MHz (typically large bandwidth for 5G systems), the range resolution can be determined using the formular C/2*B where C is the speed of light, B is the bandwidth.

Based on the fact that the sensing service category 8 KPIs can be supported by 5G systems. Similarly for the use cases with KPIs in category 5, therefore, we see no reason why the KPIs should be not included in the KPI table and therefore, propose that the EN be removed.

2. Reason for Change

Remove the FFS in the KPI Table

4. Proposal

It is proposed to agree the following changes to 3GPP TS 22.137 v.1.0.0.

* * * First Change * * * *

Table 6.2-1:	Performance requirements for 5G Wireless sensing

| Scenario | Sensing service category | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] | Sensing service description in a target sensing service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| Object detection and tracking | 1 | 95 | 10 | 10 | N/A | N/A | 10 | 10 | 1000 | 1 | 5 | 2 | Indoor/outdoor (e.g., detection of human, UAV) |
|  | 2 | 95 | 5 | 1 | 1 | 1 | 1 | 1 | 1000 | 1 | 5 | 5 | Outdoor (e.g., detection of human, UAV) requiring higher performance than category 1 |
|  | 3 | 95 | 1 | N/A | 1 | N/A | 1 | 1 x 1 | 100 (NOTE 2), or 1000 (NOTE 3) | 0.1 | 2 | 2 | Indoor/outdoor (e.g., detection tracking of human, animal, UAV) |
|  | 4 | 99 for public safety, otherwise, 95 | 0.5 | 0.5 | 1.5 for pedestrian, 15 for vehicle, otherwise, 0.1 | 1.5 for pedestrian, otherwise, N/A | 0.5 | 5 x 5 for factories 0.5 may be needed | 250 | 0.25 | 1 | 5 | Indoor/outdoor (e.g., detection tracking of human, animal, UAV, AGV, vehicle) requiring higher performance than category 3 |
|  | 5 | 95 | short range radar; 0.02, otherwise; 0.1 | 0.5 | 0.03 | N/A | 0.4 | 0.1 x 0.6 | 50 | 0.05 | 1 | 1 | Indoor/outdoor (e.g., detection tracking of human, animal, UAV, AGV, vehicle) requiring higher performance than category 4 |
| Environment monitoring | 6 | 95 | 10 | 0.2 | N/A | N/A | N/A | N/A | 6000 | 60 | 10 | 3 | Nature of environments monitored by sensing. |
| Motion monitoring | 7 | 95 | N/A | N/A | N/A | N/A | N/A | N/A | 60000 | 60 | 5 | 5 | Human motions and activities obtained by sensing (NOTE 4). |
|  | 8 | 95 | 0.2 | 0.2 | 0.1 | 0.1 | 0.375 | 0.3 | 5 | 0.1 | 5 | 5 | Human hand gestures obtained by sensing |
| NOTE 1: The additional information on some of the performance requirements can be found in [2]. NOTE 2: The value 100 ms is sourced from [3] and is valid for sensing at crossroads. NOTE 3: To realize 1m granularity tracking, when the velocity resolution is 1 m/s, the maximum corresponding sensing service latency is 1 s. NOTE 4: Additional KPI on human motion rate accuracy is defined for contactless sleep/sports monitoring [2]. |  |  |  |  |  |  |  |  |  |  |  |  |  |

.

* * * End of Change * * * *